3GPP TSG-SA5 Meeting #104 	S5-156116

# 16-20 November 2015, Anaheim (USA)												revision of S5-15xabc

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.658 | CR | 0013 | rev | - | Current version: | 12.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Align id attribute definitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM11 |  |  |  |  |  |  | Date: |  | 2015-11-04 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - The id attribute is missing in IOCs, but needs to be defined explicitly as inherits from Top (which doesn’t have any id attribute). - Editorial mistake in earlier CR implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the id attribute to relevant IOCs definition. - Editorial correction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TS would have faults which potentially could lead to implementation errors and interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Introduction, 2, 4.3.9.2, 4.3.11,2, 4.3.13.2, 4.3.19.2, 4.3.21.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  | TS 28.659 CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; as identified below:

TS 28.657		Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements

TS 28.658		Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

TS 28.659		Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions

Next Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 23.003: "Numbering, addressing and identification".

[4]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[5]	3GPP TS 28.628: "Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[6]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[7]	3GPP TS 32.602: "Telecommunication management; Configuration Management (CM); Basic CM Integration Reference Point (IRP) Information Service (IS)".

[8]	 3GPP TS 36.321: "Universal Terrestrial Access Network (UTRAN); Medium Access Control (MAC) protocol specification".

[9]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[10]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol specification".

[11]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[12]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation"

[13]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception"

[14]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E_UTRA); Base Station (BS) radio transmission and reception"

[15]	3GPP TS 32.500: "Telecommunication Management; Self-Organizing Networks (SON); Concepts and requirements"

[16]	3GPP TS 32.150: "Telecommunication management; Integration Reference Point (IRP) Concept and definitions"

[17]	3GPP TS 21.905: "Vocabulary for 3GPP Specifications"

[18]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)"

[19]	3GPP TS 23.002: "Network Architecture"

[20]	3GPP TS 32.652: "Telecommunication management; Configuration Management (CM); GERAN network resources Integration Reference Point (IRP); Network Resource Model (NRM)"

[21]	3GPP TS 28.652: "Telecommunication management; Universal Terrestrial Radio Access Network (UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

[22]	3GPP2 S.S0028-D "OAM&P for cdma2000 (Overview, 3GPP R7 Delta Specification, 3GPP2 Network Resource Model IRP)"

[23]	3GPP TS 28.708: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)"

[24]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol (X2AP)".

[25]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[26]	3GPP TS 28.625: "Telecommunication management; State Management Data Definition Integration Reference Point (IRP); Information Service (IS)".

[27]	3GPP TS 36.413: "Evolved Universal Terrestrial Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[28]	3GPP TS 36.443: "Evolved Universal Terrestrial Access Network (E-UTRAN); M2 Application Protocol (M2AP)".

[29]	3GPP TS 22.011: "Service accessibility".

[30]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[31]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[32]	3GPP TS 32.662: "Telecommunication management; Configuration Management (CM); Kernel CM; Information service (IS)".

[33]	3GPP TS 23.203: "Policy and charging control architecture".

[34]	3GPP TS 23.207: "End-to-end Quality of Service (QoS) concept and architecture".

[35]	RFC 2474: "Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers".

[36]	3GPP TS 45.008: "Technical Specification Group GSM/EDGE Radio Access Network; Radio subsystem link control".

[37]	3GPP TS 32.302: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Information Service (IS)".

[38]	3GPP TS 36.133: "Universal Terrestrial Access Network (UTRAN); Requirements for support of radio resource management".

Next Modified Section

### 4.3.9	EUtranRelation

#### 4.3.9.1	Definition

This IOC represents a NR from a source cell to a target cell, where the target cell is a EUtranGenericCell or ExternalEUtranGenericCell instance.

The source cell can be an EUtranGenericCell instance. This is the case for an Intra-E-UTRAN NR.

The source cell can be a UtranGenericCell instance. This is the case for Inter-RAT NR from UTRAN to E-UTRAN. See 3GPP TS 28.652 [21].

The source cell can be a GsmCell instance. This is the case for Inter-RAT NR from GERAN to E-UTRAN. See 3GPP TS 32.652 [20].

NRs are unidirectional.

#### 4.3.9.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | M | - | M | - |
| tCI | O | M | M | - | M |
| isRemoveAllowed | CM | M | M | - | M |
| isHOAllowed | CM | M | M | - | M |
| isICICInformationSendAllowed | CM | M | M | - | M |
| isLBAllowed | CM | M | M | - | M |
| isESCoveredBy | CM | M | M | - | M |
| qOffset | CM | M | M | - | M |
| cellIndividualOffset | CM | M | - | - | M |
| Attribute related to role |  |  |  |  |  |
| adjacentCell | M | M | M | - | M |

#### 4.3.9.3	Attribute constraints

| Name | Definition |
| --- | --- |
| isRemoveAllowed Support Qualifier | The condition is "ANR function is supported in the source cell, and the source cell is an EUtranGenericCell or a UtranGenericCell". |
| isHOAllowed Support Qualifier | The condition is "ANR function is supported in the source cell, and the source cell is an EUtranGenericCell or a UtranGenericCell". |
| isICICInformationSendAllowed Support Qualifier | The condition is "ICIC function is supported". |
| isLBAllowed Support Qualifier | The condition is "LB function is supported". |
| isESCoveredBy Support Qualifier | The condition is "Energy Saving function is supported". |
| qOffset Support Qualifier | The condition is "Neither an EM-centralized nor a distributed SON function support the SON use cases for which this attribute is relevant (see §6.5.1)". |
| cellIndividualOffset Support Qualifier | The condition is "HOO function is supported" or "Load Balancing Optimization function is supported". |

#### 4.3.9.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

Next Modified Section

### 4.3.11 	Cdma2000Relation

#### 4.3.11.1	Definition

This IOC represents a NR from one EUtranGenericCell to a CDMA2000 sector. NRs are directional.

See 3GPP2 TS S.S0028 [22]

#### 4.3.11.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | M | - | M | - |
| Attribute related to role |  |  |  |  |  |
| adjacentSector | M | M | - | - | M |

#### 4.3.11.3	Attribute constraints

None.

#### 4.3.11.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

Next Modified Section

### 4.3.13	 MBSFNArea

#### 4.3.13.1	Definition

This IOC represents MBSFN Area. For more information about MBSFN Area, see 3GPP TS 36.300 [11].

#### 4.3.13.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | M | - | M | - |
| mbsfnAreaId | M | M | M | - | M |
| Attribute related to role |  |  |  |  |  |
| cellIdList | M | M | M | - | M |

#### 4.3.13.3	Attribute constraints

None.

#### 4.3.13.4	Notifications

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAttributeValueChange | See Kernel CM IRP (3GPP TS 32.662 [32]) |  |
| notifyObjectCreation | See Kernel CM IRP (3GPP TS 32.662 [32]) |  |
| notifyObjectDeletion | See Kernel CM IRP (3GPP TS 32.662 [32]) |  |

Next Modified Section

### 4.3.19	CellOutageCompensationInformation

#### 4.3.19.1	Definition

This IOC represents information relevant in case of a Cell Outage Compensation taking place.

#### 4.3.19.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | M | - | M | - |
| cOCStatus | M | M | - | - | M |
| isCOCAllowed | M | M | M | - | M |

#### 4.3.19.3	Attribute constraints

None.

#### 4.3.19.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, with the addition that notifyAttributeValueChange shall be supported (Support Qualifier M).

Next Modified Section

### 4.3.21 	EUtranCellNMCentralizedSON

#### 4.3.21.1	Definition

This abstract IOC represents the properties of an E-UTRAN generic cell which relate to SON functions. Its purpose is to enable configuration and tuning of the cell behaviour by the operator for SON functions which are not (yet) implemented in the eNodeB. NMS level SON should consider when configuring and tuning the cell the correlation of different attributes to optimise the eNodeB and network performance. For more information about cells, see 3GPP TS 23.401 [9].

#### 4.3.21.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | M | - | M | - |
| a1ThresholdRsrp | CM | M | M | - | M |
| a1ThresholdRsrq | CM | M | M | - | M |
| a2ThresholdRsrp | CM | M | M | - | M |
| a2ThresholdRsrq | CM | M | M | - | M |
| a3Offset | CM | M | M | - | M |
| a4ThresholdRsrp | CM | M | M | - | M |
| a4ThresholdRsrq | CM | M | M | - | M |
| a5Threshold1Rsrp | CM | M | M | - | M |
| a5Threshold1Rsrq | CM | M | M | - | M |
| b1ThresholdUtraRscp | CM | M | M | - | M |
| b1ThresholdUtraEcN0 | CM | M | M | - | M |
| b1ThresholdGeran | CM | M | M | - | M |
| b1ThresholdCdma2000 | CM | M | M | - | M |
| b2Threshold1Rsrp | CM | M | M | - | M |
| b2Threshold1Rsrq | CM | M | M | - | M |
| b2Threshold2UtraRscp | CM | M | M | - | M |
| b2Threshold2UtraEcN0 | CM | M | M | - | M |
| b2Threshold2Geran | CM | M | M | - | M |
| b2Threshold2Cdma2000 | CM | M | M | - | M |
| commonChannelPowerOffset | CM | M | M | - | M |
| configurationIndex | CM | M | M | - | M |
| contentionResolutionTimer | CM | M | M | - | M |
| hysteresisEutraA1 | CM | M | M | - | M |
| hysteresisEutraA2 | CM | M | M | - | M |
| hysteresisEutraA3 | CM | M | M | - | M |
| hysteresisEutraA4 | CM | M | M | - | M |
| hysteresisEutraA5 | CM | M | M | - | M |
| hysteresisIratB1 | CM | M | M | - | M |
| hysteresisIratB2 | CM | M | M | - | M |
| numberOfRaPreambles | CM | M | M | - | M |
| preambleInitialReceivedTargetPower | CM | M | M | - | M |
| preambleTransMax | CM | M | M | - | M |
| pMax | CM | M | M | - | M |
| powerRampingStep | CM | M | M | - | M |
| qHyst | CM | M | M | - | M |
| qOffsetUtra | CM | M | M | - | M |
| qOffsetGeran | CM | M | M | - | M |
| qOffsetCdma2000 | CM | M | M | - | M |
| qQualMinUtra | CM | M | M | - | M |
| qRxLevMinEUtraSib1 | CM | M | M | - | M |
| qRxLevMinEUtraSib3 | CM | M | M | - | M |
| qRxLevMinGeran | CM | M | M | - | M |
| qRxLevMinUtra | CM | M | M | - | M |
| responseWindowSize | CM | M | M | - | M |
| rootSequenceIndex | CM | M | M | - | M |
| sIntraSearch | CM | M | M | - | M |
| sizeOfRAPreamblesGroupA | CM | M | M | - | M |
| timeToTriggerEutraA1 | CM | M | M | - | M |
| timeToTriggerEutraA2 | CM | M | M | - | M |
| timeToTriggerEutraA3 | CM | M | M | - | M |
| timeToTriggerEutraA4 | CM | M | M | - | M |
| timeToTriggerEutraA5 | CM | M | M | - | M |
| timeToTriggerIratB1 | CM | M | M | - | M |
| timeToTriggerIratB2 | CM | M | M | - | M |
| tReselectionCdma2000 | CM | M | M | - | M |
| tReselectionEUtra | CM | M | M | - | M |
| tReselectionGeran | CM | M | M | - | M |
| tReselectionUtra | CM | M | M | - | M |
| tStoreUeContext | CM | M | M | - | M |

#### 4.3.21.3	Attribute constraints

| Name | Definition |
| --- | --- |
| All Support Qualifiers | The condition is "Neither an EM-centralized nor a distributed SON function support the SON use cases for which this attribute is relevant (see §6.5.1)". |

#### 4.3.21.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

End of modifications