# 3GPP TSG SA WG5 (Telecom Management) Meeting #108	S5-164209

# 11-15 July 2016, Harbin (China)	revision of S5-164078

# Source:	Huawei

# Title:	Modification of procedures of virtualization-specific aspect failure detection and notification

# Document for:	Approval

Agenda Item:	6.4.1.3

1	Decision/action requested

Discuss and approve the text proposal.

2	References

[1]	Draft TS 28.516 Fault Management (FM) for mobile networks that include virtualized network functions Procedures v0.2.0.

3	Rationale

In draft TS 28.516 [1], an Editor’s note in the virtualization-specific aspect failure detection and notification procedures is described as follows:

Editor’s note: The figure needs to be revisited to illustrate using operations and remove the unused function entities.

This contribution proposes to solve the issue described in the Editor’s note.

Furthermore, it also proposes to change the wording of title in Sec 4.3, Sec 4.3.1 and Sec 4.3.2 to align with Sec 4.4

4	Detailed proposal

It is proposed to make the following changes to TS 28.516 [1].

1st proposed change

4.3	Virtualization-specific aspect failure detection and notification

4.3.1	Virtualization-specific aspect failure detection and notification by VNFM

The following figure illustrates the procedure of notifying the EM about a virtualization-specific failure of a virtualized network function when virtualization-specific aspect failure detection and notification by VNFM.

Figure 4.3.1-1 Procedure of virtualization-specific aspect failure detection and notification by VNFM

- EM subscribes to VNF-related virtualization fault reports.

- A new virtualization-specific failure occurs and is detected by VNF virtualization-specific component.

- VNF sends virtualization-specific fault notification to VNFM.

- VNFM creates VNF virtualization-related fault report as described in ETSI NFV ISG.

- VNFM sends VNF instance alarms generated due to changes in the state of virtualized resources used by the VNFs and their constituent VNFC instances to EM. The VNF and/or VNFC alarms related to virtualized resources include AlarmId, affected VNF identifier, affected VNFC identifiers, and FaultDetails which provides additional information about the fault, for example the resource identifier which causes the fault (e.g. VM identifier, vNIC identifier or vPORT identifier). The Alarm information element see clause 9.3.4 of [3].

4.3.2	Virtualization-specific aspect failure detection and notification by EM

The following figure illustrates the procedure of notifying the EM about a virtualization-specific failure of a virtualized network function.

Figure 4.3.2-1 Procedure of virtualization-specific aspect failure detection and notification by EM

- A new virtualization-specific failure occurs and is detected by VNF virtualization-specific component.

- VNF sends virtualization-specific fault notification to EM through proprietary interfaces, the corresponding VNF instance identifier and/or VNFC identifier should be included.

End of changes