3GPP TSG-SA5 Meeting #112 	S5-171799

# Guilin,China, 27-31 March 2017														Revision of S5-171603

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.274 | CR | 0038 | rev | 1 | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Improve Message flows offline charging for SMS delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Alcatel-Lucent Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH14-SMST4 |  |  |  |  |  |  | Date: |  | 2017-03-29 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The trigger indication is missing for SMS Delivery from SMS-SC message flow. In Online the clause 5.3.2.10 is not needed: the Delivery scenario for the first attempt is already covered in 5.3.2.1 (simple submission) and 5.3.2.2 (enhanced submission), and for the subsequent re-attempt it is covered in 5.3.2.4 (Origination retry). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a trigger in figure 5.2.2.2.1 Add SMS Deliver to reflect the Delivery in alignment with 5.2.2.2 Void 5.3.2.10 Delivery scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading description for SMS delivery |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2, 5.3.2.1, 5.3.2.4, 5.3.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.2.2.2	SMS Delivery from SMS-SC

Figure 5.2.2.2.1 describes the scenario where SMS-SC originates SM transfer towards the receiving party.

# Figure 5.2.2.2.1: Offline charging SMS Transfer from SMS-SC

1)	The SMS-SC decides to forward "SMS Deliver" message towards the receiving party, as a first attempt or due to internal trigger for a retry delivery of a previously failed and stored SM.

2)	The SMS-SC forwards the "SMS Deliver" message towards the receiving party.

3)	The SMS-SC receives "SMS Deliver Answer" message as the delivery success or failure of the SM transfer attempt.

4)	The SMS-SC triggers a Charging Data Request[Event] to record successful or unsuccessful result of SM delivery.

5)	The CDF creates a SC-SMT CDR and acknowledges the reception of the data.

Next change

#### 5.3.2.1	Simple submission

This clause contains message flows for the different operation models IEC (figure 5.3.2.1.1) and ECUR (figure 5.3.2.1.2).

# Figure 5.3.2.1.1: Online charging in simple submission for IEC

1)	Depending on which SMS mechanism (i.e. SMS or SMSIP) is in operation, the SMS node receives an incoming SM-Submit or a MAP-Forward-SM.

2)	The SMS node triggers a Debit Units Request message to the OCS.

3)	The OCS performs the appropriate credit processing based on the received request.

4)	The OCS responds with a Debit Units Response message to the SMS node.

5)	If authorized, the SMS node continues the SM processing as appropriate for the origination procedures.

# Figure 5.3.2.1.2: Online charging in simple submission for ECUR

1)	Depending on which SMS mechanism (i.e. SMS or SMSIP) is in operation, the SMS node receives an incoming SM-Submit or a MAP-Forward-SM.

2)	The SMS node triggers a Reserve Units Request [Initial] message to the OCS.

3)	The OCS performs the appropriate credit processing based on the received request.

4)	The OCS responds with a Reserve Units Response message to the SMS node.

5)	If authorized, the SMS node continues the SM processing as appropriate for the origination procedures.

6)	The SM transaction is successfully acknowledged.

7)	The SMS node triggers a Reserve Units Request [Terminate] message to the OCS reporting the successful event transaction.

8)	The OCS performs the appropriate credit processing based on the received request.

9)	The OCS responds with a Reserve Units Response message to the SMS node.

Next change

#### 5.3.2.4	Origination retry

This clause contains message flows for the different operation models IEC (figure 5.3.2.4.1) and ECUR (figure 5.3.2.4.2) for redelivery attempts in the origination direction.

# Figure 5.3.2.4.1: Online charging in origination redelivery attempt for IEC

1)	An SMS node internal trigger occurs to attempt a redelivery of a previously failed and stored SM.

2)	The SMS node triggers a Debit Units Request message to the OCS.

3)	The OCS performs the appropriate credit processing based on the received request.

4)	The OCS responds with a Debit Units Response message to the SMS node.

5)	If authorized, the SMS node continues the SM processing as appropriate for the origination procedures.

# Figure 5.3.2.4.2: Online charging in origination redelivery attempt for ECUR

1)	An SMS node internal trigger occurs to attempt a redelivery of a previously failed and stored SM.

2)	The SMS node triggers a Reserve Units Request[Initial] message to the OCS.

3)	The OCS performs the appropriate credit processing based on the received request.

4)	The OCS responds with a Reserve Units Response message to the SMS node.

5)	If authorized, the SMS node continues the SM processing as appropriate for the origination or termination procedures.

6)	The SM transaction is successfully acknowledged.

7)	The SMS node triggers a Reserve Units Request[Terminate] message to the OCS reporting the successful event transaction.

8)	The OCS performs the appropriate credit processing based on the received request.

9)	The OCS responds with a Reserve Units Response message to the SMS node.

Next change

#### 5.3.2.10	Void

End of changes