# 3GPP TSG SA WG5 (Telecom Management) Meeting #114	S5-174283

# 21-25 August 2017, Sophia Antipolis, France	revision of S5A-17xabc

# Source:	Ericsson

# Title:	Triggers and reporting in the separate SMF and UPF - Registration procedure

# Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the changes in the TR.

# 2	References

[1]	3GPP T3 32.899 Skeleton update

[2]	3GPP TS 22.115: " Charging and billing".

[3]	3GPP TS 22.261: "Service requirements for the 5G system;".

[4]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

# 3	Rationale

Additional solution for the key issue “Triggers and reporting in the separate SMF and UPF”.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TR 32.899 [1]

First change

#### Solution#3.4: Charging at Registration procedure

##### 5.3.4.x.1	Architecture description

The same architecture as per clause 5.3.4.2.1 applies.

##### 5.3.4.x.2	Procedure description

A UE needs to register with the network to get authorised to receive services, to enable mobility tracking and to enable reachability. Even if there are an authorization of services there won’t be any accounting or charging since this doesn’t provide any specific service only the availability.

The Registration procedure is used when the UE needs to perform initial registration to the 5G system, mobility registration update upon changing to a new Tracking area (TA) outside the UE's registration area in idle mode, when the UE performs a periodic registration update (due to a predefined time period of inactivity), and additionally when the UE needs to update its capabilities or protocol parameters that are negotiated in Registration procedure. For more information on the registration procedure see TS 23.502 [x] clause 4.2.2.

End of changes