# 3GPP TSG SA WG5 (Telecom Management) Meeting #117	S5-181378

# 29 January - 2 February 2018, Rome, Italy	revision of S5-181132

# Source:	ETRI, KT

# Title:	Update on Management aspects of a slice instance

# Document for:	Approval

Agenda Item:	6.4.1

1	Decision/action requested

The group is asked to discuss and approve the contribution.

2	References

[a]	TS 28.530 v0.3.0: “Management of network slicing in mobile networks; Concepts, use cases and requirements”.

3	Rationale

This pCR proposes to modify preparation, commissioning, and operation phase of “Management aspects of a slice instance” to reflect network resource optimal usage.

4	Detailed proposal

It is proposed to make the following changes to TR 28.530.

1st modified section

4	Background and concepts

4.1	Management aspects of a network slice instance

4.1.1	Introduction

This clause describes management aspects of a Network Slice Instance, which can be described by the four phases shown in Figure 4.1.1.1, the phases are:

- Preparation

- Commissioning

- Operation

- Decommissioning

Figure 4.1.1.1: Management aspects of network slice instance

Each phase, described in subsequent clauses, defines high level tasks and should include appropriate verification of the output of each task.

4.1.2	Preparation

In the preparation phase the NSI does not exist. The preparation phase includes network slice template design, network slice capacity planning, on-boarding and, evaluation of the network slice requirements, preparing the network environment and other necessary preparations required to be done before the creation of an NSI.

4.1.3	Commissioning

Network slice provisioning in the commissioning phase involves the creation of an NSI. During NSI creation all resources to the NSI have been created and configured to satisfy the network slice requirements. The creation of an NSI can include the creation or modification of NSI constituents.

4.1.4	Operation

The Operation phase includes the activation, supervision, performance reporting (e.g. for KPI monitoring), resource capacity planning, modification, and de-activation of an NSI.

Activation includes any actions that makes the NSI active and ready to support communication services.

NSI modification could map to several workflows, e.g. changes capacity, changes topology. The modification can include creation or modification of NSI constituents. NSI modification can be triggered by receiving new network slice requirements or as the result of supervision/reporting

The deactivation includes any actions that make the NSI inactive and stops providing communication services.

Network slice provisioning in the operation phase involves activation, modification and de-activation of a NSI.

4.1.5	Decommissioning

Network slice provisioning in the decommissioning phase includes the following: decommissioning the non-shared constituents if required and removing the NSI specific configuration from the shared constituents. After the decommissioning phase, the NSI is terminated and does not exist anymore.

End of 1st modified section

2nd modified section

The Operation phase includes the activation, supervision, performance reporting (e.g. for KPI monitoring), resource capacity planning, modification, and de-activation of an NSI.

Activation includes any actions that makes the NSI active and ready to support communication services.

Resource capacity planning includes any actions that calculates resource usage based on an NSI provisioning, and performance monitoring and generates modification polices as a result of the calculation.

Note: automation of resource capacity planning is out of scope of this technical specification.

NSI modification could map to several workflows, e.g. changes capacity, changes topology. The modification can include creation or modification of NSI constituents. NSI modification can be triggered by receiving new network slice requirements or as the result of supervision/reporting

The deactivation includes any actions that make the NSI inactive and stops providing communication services.

Network slice provisioning in the operation phase involves activation, modification and de-activation of a NSI.

End of modified sections