# 3GPP TSG SA WG5 (Telecom Management) Meeting #118	S5-182550

# 9-13 April 2018, Beijing, China	revision of S5-182387

# Source:	Huawei

# Title:	Add Requirements for Feasibility Check

# Document for:	Approval

Agenda Item:	6.4.2

# 1	Decision/action requested

Discuss and approve the text proposal

# 2	References

[1]	Draft 3GPP TS 28.531: "Management of network slicing in mobile networks Concepts, use cases and requirements v0.3.1".

[2]	3GPP TS 23.501: "System Architecture for the 5G System v15.0.0".

[3]	3GPP TR 23.791: “Study of Enablers for Network Automation for 5G v0.1.0”

[4]	[4]	Draft 3GPP TS 28.532: “Management and orchestration of networks and network slicing; Provisioning; Stage 2 and stage 3”

[5] 	3GPP RS 23.503: “Policy and Charging Control Framework for the 5G System v15.0.0; stage2 (Release 15)”

# 3	Rationale

--------------------------Start Extract from TS23.503 [5] Section 6.2.9 -----------------------------------------------

### 6.2.9	Network Data Analytics Function (NWDAF)

NWDAF represents operator managed network analytics logical function. NWDAF provides slice specific network data analytics to the PCF and NSSF. NWDAF provides network data analytics (i.e., load level information) to PCF and NSSF on a network slice level and the NWDAF is not required to be aware of the current subscribers using the slice. NWDAF notifies / publishes slice specific network status analytic information to the PCF(s) and NSSF that are subscribed to it. PCF(s) and NSSF can collect directly slice specific network status analytic information from NWDAF. This information is not subscriber specific. PCF uses that data in its policy decisions. NSSF may use the load level information provided by NWDAF for slice selection.

--------------------------End Extract from TS23.503 [5] Section 6.2.9 -----------------------------------------------

# 4	Detailed proposal

It is proposed to make the following changes to TS 28.531 [1].

Start of changes

### 5.1.6 	Use case for network slice feasibility check

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To check the feasibility of provisioning a network slice instance to determine whether the network slice instance (NSI) requirements can be satisfied (e.g., in terms of resources) |  |
| Actors and Roles | Network slice management service consumer |  |
| Telecom resources | Network slice instance |  |
| Assumptions | Network slice management service consumer has decided to check the feasibility of provisioning a network slice instance based on, for example, internal decision or to facilitate an external service request. |  |
| Pre-conditions | Network slice requirements have been derived or received by network slice management service consumer. |  |
| Begins when | Network slice management service provider receives the request to provision a network slice instance according to the network slice requirements. |  |
| Step 1 (M) | Network slice management service provider identifies the network slice subnets according to the requirements. |  |
| Step 2 (M) | Network slice management service provider fulfils feasibility of provisioning a network slice by requesting the network slice subnets service provider(s) regarding the availability of resources. |  |
| Step 3 (M) | Network slice subnet management service provider(s) checks the feasibility of provisioning a slice subnet(s) by analysing network constituents to ensure that their capabilities, e.g., resources, management services, etc. are adequate to provision network slice instance, satisfying all requirements without impacting existing services. For the purspose of checking the feasibility of provisioning a network slice subnet(s) of the network slice instance, network slice subnet management service provider(s) may obtain information from the network (e.g., load level information from the NWDAF). |  |
| Ends when | Feasibility check results have been provided to Network slice management service consumer. |  |
| Exceptions | One of the mandatory steps fails. |  |
| Post-conditions | N/A |  |
| Traceability | REQ-NSM_NSSM-FUN –X, REQ-NSM_NSSMF-FUN-Z, REQ-NSM_NSMSP-FUN-X |  |

2nd change

### 5.2.3 Requirements for interface provided by NSSMF

REQ-NSM_NSSMF-FUN-1	The NSSMF shall have the capability allowing its consumer to request a network slice subnet instance.

REQ-NSM_NSSMF-FUN-2	The NSSMF shall have the capability to send NSSI information to its consumer.

REQ-NSM_NSSMF-FUN-3	The NSSMF shall have the capability to send NS lifecycle management request to NFVO.

REQ-NSM_NSSMF-FUN-4	The NSSMF shall have the capability to receive network slice subnet related requirements from NSMF.

REQ-NSM_NSSMF-FUN-5	The NSSMF shall have the capability to decide to create a new NSSI or reuse an existing NSSI based on the network slice subnet related requirements.

REQ-NSM_NSSMF-FUN-6	The NSSMF shall have the capability to associate the NS instance with corresponding NSSI.

REQ-NSM_NSSMF-FUN-7	The NSSMF shall have the capability to provide the NSSI information.

REQ-NSM_NSSMF-FUN-8	The NSSMF shall have the capability to configure the RAN NSSI constituents with the RRM policy information for simultaneous support of multiple NSIs.

REQ-NSM_NSSMSP-FUN-9	The NSS management service provider shall have the capability allowing its consumer to obtain network slice subnet instance information.

REQ-NSM_NSSMF-FUN-Z The NSS management service provider shall have the capability allowing its consumer to obtain information regarding available NSS resources.

### 5.2.4 Requirements for network slice subnet management service

Editor’s Note: The name of the service in this clause needs to be revisited to align with the 5G network management architecture.

REQ-NSM_NSSM-FUN-1	The NetworkSliceSubnetAllocation service shall have the capability of allowing its authorized consumer to request the deallocation of a network slice subnet instance.

REQ-NSM_NSSM-FUN-2	The NetworkSliceSubnetActivation service shall have the capability allowing its authorized consumer to request activation of a network slice subnet instance.

REQ-NSM_NSSM-FUN-3	The NetworkSliceSubnetActivation service shall have the capability allowing its authorized consumer to request deactivation of a network slice subnet instance.

Editor’s Note: The activation and deactivation of a shared network slice subnet instance is FFS.

REQ-NSM_NSSM-FUN-4	The NetworkSliceSubnetModification service shall have the capability allowing its authorized consumer to request modification of a network slice subnet instance.

REQ-NSM_NSSM-FUN –X The NetworkSliceSubnetFeasibilityCheck service shall have the capability allowing its authorized consumer to obtain the feasibility of provisioning a network slice subnet instance.

3rd change

## 5.2 Requirements

### 5.2.1 Requirements for interface provided by NSMF

REQ-NSM_NSMSP-FUN-X	The NS management service provider shall have the capability allowing its authorized consumer to obtain the feasibility of provisioning the requested NSI.

End of changes