# 3GPP TSG SA WG5 (Telecom Management) Meeting #119	S5-183129

# 14-18 May 2018, La Jolla (US)	revision of S5-18xabc

# Source:	Huawei

# Title:	Obtaining exposure for slice deliverable types

# Document for:	Approval

Agenda Item:	6.4.2

1	Decision/action requested

Discuss and approve the text proposal

2	References

[1]	3GPP TS 28.530: "Telecommunication management; Management and orchestration of networks and network slicing; Provisioning; Concepts, use cases and requirements. v0.6.0".

[2]	3GPP TS 28.531: "Telecommunication management; Management and orchestration of networks and network slicing; Provisioning; Stage 1. v0.4.0".

[3]	3GPP TS 28.533: "Telecommunication management; Management and orchestration of networks and network slicing; Management and orchestration architecture v0.2.0".

3	Rationale

In TS 28.530 [1], Network slice delivery concepts have been proposed to illustrate that the different management capabilities can be accessed by customers shown as follows:

------------------------------------------ Extract from 3GPP TS 28.530 [1] ----------------------------------------------------------

4.1.8	Network slice delivery concepts

Network slice instances are provided in different compositions to the customer which may include access to different management capabilities and network slice instance provisioning procedures for the customer.

For example, a network slice may be delivered

a) 	to meet customer's communication service requirements without any exposure of internal NSI structures (applicable to both individual subscribers and NSaaS); or

b)	to meet the network slice instance requirements, with some exposure of the internal NSI structures (e.g. NFs, topology, etc.) and with some network monitoring capability as enabled by the provider; or

c) 	to meet the network slice instance requirements with some exposure of the internal NSI structures (e.g. NFs) with some management capabilities as enabled by the provider.

--------------------------------------------  End of the extract ---------------------------------------------------------------------------

To realize different deliverable types of network slice instances, management and data exposure of the network slice compositions are required. In [1], two use cases are used briefly to explain the procedures of how to expose management capability and data, shown as follows

------------------------------------------ Extract from 3GPP TS 28.530 [1] ----------------------------------------------------------

5.4.11		Exposure of network slice management data for network slice as a service case

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To expose network slice management data to a Communication Service Provider (CSP) consuming Network Slice as a Service (NSaaS) based on mutual agreement. |  |
| Actors and Roles | CSP consuming NSaaS |  |
| Telecom resources | 3GPP management system NSI |  |
| Assumptions | Network slice management data of NSI can be exposed to the CSP consuming NSaaS according to the pre-defined agreements. |  |
| Pre-conditions | 1. NSaaS level exposure has been agreed upon and the CSP offering the NSaaS is aware of it. 2. An NSI used for NSaaS is created. |  |
| Begins when | The CSP consuming NSaaS wants to get the management data of the network slice instance. |  |
| Step 1 (M) | The CSP consuming NSaaS sends requests to the 3GPP management system for the exposure management data of network slice instance. |  |
| Step 2 (M) | The 3GPP management system provides the CSP consuming NSaaS of exposed management data for the NSaaS scenario. |  |
| Ends when | The network slice management data is provided. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The CSP consuming NSaaS is aware of the management data of the network slice instance. |  |
| Traceability | REQ-3GPPMS-CON-27 |  |

5.4.12		Exposure of network slice management capability

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To expose limited network slice management capability to a Communication Service Customer (CSC) consuming Network Slice as a Service (NSaaS) based on mutual agreement. |  |
| Actors and Roles | CSC consuming NSaaS CSP offering NSaaS service |  |
| Telecom resources | 3GPP management system NSI |  |
| Assumptions | Network slice management capability of 3GPP management system can be partially exposed to the CSC consuming NSaaS according to the pre-defined agreements. |  |
| Pre-conditions | Level of management exposure has been agreed upon and the CSP offering the NSaaS service is aware of it. |  |
| Begins when | The CSC consuming NSaaS wants to get certain management capability to manage the network slice instance, e.g., PM, FM, CM, based on the mutual agreement between CSC and CSP. |  |
| Step 1 (M) | The CSC consuming NSaaS sends requests to the 3GPP management system for the exposure of management capability of network slice instance. |  |
| Step 2 (M) | The 3GPP management system provides the CSC consuming NSaaS with the requested capability via appropriate methods, e.g., exposing network slice management service to the CSC. |  |
| Ends when | The network slice management capability is provided. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The limited network slice management capability has been exposed to the CSC consuming NSaaS. |  |
| Traceability | FFS |  |

--------------------------------------------  End of the extract ---------------------------------------------------------------------------

Moreover, two specification level use cases are also given in TS 28.531 [2] for exposing management capabilities and data, shown as follows.

------------------------------------------ Extract from 3GPP TS 28.531 [2] ----------------------------------------------------------

5.1.14	Exposure of network slice management data

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Enable network slice management service consumer to obtain network slice management data (e.g. PM data, FM data). |  |
| Actors and Roles | Network slice management service consumer. For example, CSMF plays the role of network slice management service consumer. Network slice management service provider. For example, NSMF plays the role of network slice management service provider. |  |
| Telecom resources | Network slice instance |  |
| Assumptions | Network slice management service consumer is authorized to obtain the network slice management data from network slice management service provider. |  |
| Pre-conditions | NSI is created. |  |
| Begins when | Network slice management service consumer wants to obtain the network slice management data. |  |
| Step 1 (M) | Network slice management service consumer sends a request to network slice management service provider to obtain the network slice management data. |  |
| Step 2 (M) | Network slice management service provider provides the network slice management service consumer with the network slice management data . |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | Network slice management service consumer obtained the network slice management data. |  |
| Traceability | REQ-NSM_NSM-FUN-7 |  |

5.1.15	Network slice subnet instance management capability exposure

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Enable authorized NSS management service consumer to obtain network slice subnet management capability (e.g. obtaining measurement, updating resource allocations). |  |
| Actors and Roles | NSS management service consumer (e.g., the operator or NSMF or NSSMF) |  |
| Telecom resources | NSS management service provider (e.g., NSSMF) Network slice subnet instance NF(s) |  |
| Assumptions | NSS management service consumer is authorized to obtain the allowed management capability from NSS management service provider. |  |
| Pre-conditions | NSSI is created. |  |
| Begins when | NSS management service consumer wants to obtain the network slice subnet management capability. |  |
| Step 1 (M) | NSS management service consumer sends a request to NSS management service provider to obtain the network slice subnet instance management capability. The information indicating which specific management capability need to be obtained may be included in the request. |  |
| Step 2 (M) | NSS management service provider processes this request. |  |
| Step 3 (M) | NSS management service provider provides the required exposure interfaces to NSS management service consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | NSS MS consumer obtained the allowed network slice subnet instance management capability. |  |
| Traceability | REQ-NSM_NSSM-FUN-15 |  |

--------------------------------------------  End of the extract ---------------------------------------------------------------------------

To expose management capabilities and data normally require corresponding exposure on network function level. An example of how to expose them is introduced in TS 28.533 [3], shown as follows.

------------------------------------------ Extract from 3GPP TS 28.533 [3] ----------------------------------------------------------

NF management function for multiple managed network functions as shown in Figure B.2.1 provides:

- Management services exposure; and,

- Data governance for management service exposure.

NOTE 1: One example of a management object with multiple management services is NF pooling design. Another example of a management object with this multiple NF management services is 5G Core control plane.

NOTE 2: NF management function can behave as consumer to ETSI ISG NFV MANO interfaces (e.g. Ve-vnfm-em interface exposed by VNFM).

Figure B.2.1: Example NF management function layer structure

As shown in Figure B.2.1, an example of NF management function model structure is given. Exposure governance management function (EGMF) shown in Figure B.2.1 is management function in network function model with the role of management service exposure governance (i.e. abstraction, simplification, filtering, etc.). When multiple NF management services are exposed to network management, the particular group of multiple NF management services can be represented by a set of NF management services. This set of NF management services are exposed as a bulk NF management service, by NF management function. Additional management service abstraction may be needed based on NF management services because of lack of trust relationship between management service provider/consumer cannot address management services to build a global view of subnet or meaningful management purposes.

--------------------------------------------  End of the extract ---------------------------------------------------------------------------

Therefore, to link the management and data exposure of different levels (e.g., NSI, NSSI, NFs) to deliverable types, we propose to include a use case of obtaining different network slice delivery in TS 28.531 [2].

4	Detailed proposal

It is proposed to make the following changes to TS 28.531 [2].

Start of changes

5.1.X	Obtaining exposure for slice deliverable types

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Enable authorized NS management service consumer to obtain network slice management exposure (e.g. obtaining measurement, updating resource allocations). |  |
| Actors and Roles | NS management service consumer (e.g., the operator or NSMF) |  |
| Telecom resources | NS management service provider (e.g., NSMF) Network slice instance Network slice subnet instance NF(s) |  |
| Assumptions | NS management service consumer is authorized to obtain the allowed management capability and/or information from NS management service provider. |  |
| Pre-conditions | NSI is created. |  |
| Begins when | NS management service consumer wants a network slice deliverable that requires obtaining network slice management capability and/or information. |  |
| Step 1 (M) | NS management service consumer sends a request to NS management service provider for the desired network slice deliverable type. The information indicating which specific management capability and/or information need to be obtained may be included in the request. |  |
| Step 2 (M) | NS management service provider processes this request. NS management service provider decomposes the request into network slice constituents to be exposed. |  |
| Step 3 (M) | NS management service provider obtains management capability and/or information exposure from network slice constituents. For example, the NS management service provider requests network slice subnet instance management capability exposure from NSS management service consumer. NS management service provider can request management capability and/or information exposure from NF management services. | 5.1.15 Network slice subnet instance management capability exposure, 5.1.14 Exposure of network slice management data |
| Step 4 (M) | NS management service provider provides the required exposure interfaces to NS management service consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | NS MS consumer obtained the requested slice deliverable. |  |
| Traceability | REQ-NSM_NSSM-FUN-15 |  |

End of changes