# 3GPP TSG SA WG5 (Telecom Management) Meeting #119	S5-183131

# 14-18 May 2018, La Jolla (US)	revision of S5-18xabc

# Source:	Huawei

# Title:	Add procedures for NSSI feasibility check

# Document for:	Approval

Agenda Item:	6.4.2

1	Decision/action requested

Discuss and approve the text proposal

2	References

[1]	3GPP TS 28.530: "Telecommunication management; Management of 5G networks and network slicing; Concepts, use cases and requirements v0.5.1".

[2]	Draft 3GPP TS 28.531: "Management of network slicing in mobile networks Concepts, use cases and requirements v0.4.0".

[2]	3GPP TS 28.532:  Management and orchestration of networks and network slicing; Provisioning; Stage 2 and Stage 3 v0.2.0”.

3	Rationale

3GPP TS 28.530 [1], TS 28.531 [2] and TS 28.532 [3] mention exposure and feasibility check in UCs and procedures. For instance, Clause 5.4.11 in 3GPP TS 28.530 [1], and Clause 5.1.5 3GPP TS 28.531 [2] provide use cases for information exposure regarding MNSI and MNSSIs. Therefore, first aim of this proposal is to add a management service for exposure.

In addition, feasibility check UC is presented in Clause 5.1.6 3GPP TS 28.531 [2].  Clause 5.1.6 mentions a service request for checking the feasibility of network slice subnet instances. However, a management service for feasibility check of network slice subnet instances is not yet defined in Stage-2/3. Therefore, second aim of this proposal is to add feasibility check as a management service.

Finally, 3GPP TS 28.531 [2] includes a UC to check the feasibility of provisioning a network slice instance to determine whether the network slice instance (NSI) requirements can be satisfied (e.g., in terms of resources) in Clause 5.1.6. In step-3 of Clause 5.1.6, network slice subnet management service providers checks the feasibility of provisioning slice subnet(s) by analysing network constituents to ensure that their capabilities, e.g., resources, management services, etc. are adequate to provision network slice instance, satisfying all requirements without impacting existing services. Also, 3GPP TS 28.532 [3] contains feasibility check of network slice subnet during network slice subnet allocation This proposals aims to add procedures for checking feasibility of a network slice subnet.

4	Detailed proposal

It is proposed to make the following changes to TS 28.532 [3].

First change

5	Interfaces for provisioning of networks and network slicing

Table 5.1.1: Management services for network slice provisioning

| Service Name | Service interfaces |
| --- | --- |
| NetworkSliceAllocation | allocateMNSI deallocateMNSI |
| NetworkSliceConfiguration | readMNSIconfig modifyMNSIconfig |
| NetworkSliceActivation | activateMNSI deactivateMNSI |

Editor’s Note: The list of service interfaces may need to be revisited when the details of the service interfaces have been agreed.

Table 5.1.2: Management services for network slice subnet provisioning

| Service Name | Service interfaces |
| --- | --- |
| NetworkSliceSubnetAllocation | allocateMNSSI deallocateMNSSI |
| NetworkSliceSubnetConfiguration | readMNSSIconfig modifyMNSSIconfig |
| NetworkSliceSubnetActivation | activateMNSSI deactivateMNSSI |
| NetworkSliceSubnetExposure | exposeMNSSIinfo exposeMNSSImgmt |
| NetworkSliceSubnetFeasibilityCheck | enquiryMNSSIconstituent reserveMNSSIconstituent checkMNSSIfeasibility |

Editor’s Note: The list of service interfaces may need to be revisited when the details of the service interfaces have been agreed.

Editor's NOTE 1: The interface here means the service based interface which represents how a set of services is provided/exposed by a management service provider. The name may change depending on the discussion result of service based management architecture.

Editor's NOTE 2: This clause will contain individual operation descriptions with appropriate parameters and results following the pattern of:

    - XXX operation

      - Description

      - Input parameters

      - Output parameters

      - Results

Second change

7.X	Procedure of Checking Feasibility of Network Slice Subnet

Figure 7.X-1 Network slice subnet feasibility check procedure

1)	Network Slice Subnet Management Service Provider (NSSMS_Provider) receives a provisioining NSSI request (e.g., AllocateNssiRequest, ModifyNssiRequest) from Network Slice Subnet Management Service Consumer (NSSMS_Consumer) with network slice subnet related requirements (e.g. Area information, User Number, traffic demand, Qos Quality, whether the requested network slice instance could be shared). The request is evaluated and initial resources to be allocated are identified.

2) [Optional] NSSMS_Provider may request information and updates from NSSMS_Provider and Other_MS_Provider regarding the resources.

3)	NSSMS_Provider sends reservation requests to Other Management Service Providers (Other_MS_Provider), e.g., MANO, TN manager. NSSMS_Provider receives responses with information regarding reserved resources, e.g., their availability, identification information of reserved resources and so on.

4)	NSSMS_Provider evaluates the responses to determine if the network slice subnet requirements can be satisfied.

5)	If feasible,

5.a)	NSSMS_Provider is ready for provisioining.

5.b)	[Optional] Acknowledgement regarding fesibility check results can be sent to NSSMS_Customer.

6)	If not feasible,

6.a)	NSSMS_Provider cancels reservations, optionally may receive acknowledgement.

6.b)	NSSMS_Provider is not ready for provisioining.

6.c) 	NSSMS_Provider may send negative acknowledgement regarding results of feasibility check to NSSMS_Customer.

End of changes