# 3GPP TSG SA WG5 (Telecom Management) Meeting #119	S5-183132

# 14-18 May 2018, La Jolla (US)	revision of S5-18xabc

# Source:	Huawei

# Title:	Add Procedures of Querying MANO capability

# Document for:	Approval

Agenda Item:	6.4.2

1	Decision/action requested

Discuss and approve the text proposal

2	References

[1]	Draft 3GPP TS 28.531: "Management of network slicing in mobile networks Concepts, use cases and requirements v0.4.0".

[2]	3GPP TS 28.532:  Management and orchestration of networks and network slicing; Provisioning; Stage 2 and Stage 3 v0.2.0”.

3	Rationale

3GPP TS 28.531 includes a UC for creating a 3GPP sub-network in Clause 5.1.18. The Pre-conditions of the UC has a number of assumptions regarding resources, e.g., NSD(s) of the NS realizing the 3GPP sub-network have been on-boarded to ETSI NFV MANO system. The 3GPP management system may need to query capabilities of network service providers to make appropriate requests, e.g., while provisioning a network slice (subnet) instance or creating sub-network. Therefore, this proposal suggests adding procedures for MANO querying, so that information about capabilities of MANO can be obtained.

4	Detailed proposal

It is proposed to make the following changes to TS 28.532 [2].

Start of changes

7.X	Procedure of Querying MANO Capability

Figure 7.X-1 Network slice subnet feasibility check procedure

1)	Network Service Consumer (NS_Consumer) sends a query request to obtain NS capability of MANO, i.e., which NSDs are on-boarded, which NSs can be provided. For instance, this information can be used for creation of a 3GPP sub-network, or network slice (subnet) instance.

2) MANO processes the query request with consideration to openness and exposure aggreements between the NS_Consumer and MANO.

3)	MANO sends appropriate NS capability information.

End of changes