# 3GPP TSG SA WG5 (Telecom Management) Meeting #119 Ad-hoc	S5-184120

# 26 - 28 June 2018, Stockholm, Sweden	S5-184250

# Source:	ETRI, KT

# Title:	Add use case and requirements for capacity planning of network slice management capability

# Document for:	Approval

Agenda Item:	4.2.2

1	Decision/action requested

Discuss and approve the text proposal.

2	References

[1]	3GPP TS 28.530 Management of 5G networks and network slicing; Concepts, use cases and requirements v0.6.0

[2]	3GPP TS 28.531 Management and orchestration of networks and network slicing; Provisioning; Stage 1 v0.4.0

3	Rationale

In TS 28.530 [1] Clause 4.3.4 indicates the requirements of capacity planning as an operation phase of the management aspect of a network slice instance.

***********************************Extract from TS 28.530 start*************************************

4.3	Management aspects of a network slice instance

4.3.4	Operation

The Operation phase includes the activation, supervision, performance reporting (e.g. for KPI monitoring), resource capacity planning, modification, and de-activation of an NSI.

Activation makes the NSI ready to support communication services.

Resource capacity planning includes any actions that calculates resource usage based on an NSI provisioning, and performance monitoring and generates modification polices as a result of the calculation.

NOTE: 	Automation of resource capacity planning is out of scope of the present document.

NSI modification could be including e.g. capacity or topology changes. The modification can include creation or modification of NSI constituents. NSI modification can be triggered by receiving new network slice requirements or as the result of supervision/reporting

The deactivation includes actions that make the NSI inactive and stops the communication services.

Network slice provisioning actions in the operation phase involves activation, modification and de-activation of an NSI.

***********************************Extract from TS 28.530 end**************************************

TS 28.531 [2] does not provide capacity planning operation yet. This proposal aims to propose requirement and use case of the capacity planning operation for the management of a network slice instance.

4	Detailed proposal

It is proposed to make the following changes to draft TS 28.531 [2].

Start of changes

5.1.	Network slice resource capacity planning

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To calculate capacity of network slice instances and network slice subnet instances. |  |
| Actors and Roles | Network slice management service consumer. For example, CSMF or CSP providing NSaaS plays the role of network slice management service consumer when a resource capacity planning is triggered by a provisioning process. And NSMF or NSSMF plays the role of network slice management service consumer when a capacity planning operation is a scheduled one. |  |
| Telecom resources | Network slice instance Network slice subnet instance Network slice management service provider. For example, NSMF or NSSMF plays the role of network slice management service provider. |  |
| Assumptions | Network slice management service consumer has decided to calculate optimal network slice resource capacity. |  |
| Pre-conditions | Network slice provisioning requirements have been received or periodical schedules of capacity planning by network slice management service consumer. |  |
| Begins when | Network slice management service consumer requests capacity planning of the NSIs and/or NSSIs. The request can be initiated by network slice management consumer receiving CSP’s network slice provisioning request or a periodical capacity planning schedule. | Network slice instance creation use case includes fesibility check which may invoke network slice capacity planning operation |
| Step 1 (M) | Network slice management service provider fetches slice provisioning requirements, existing active or non-active NSI and/or NSSI resource information, performance measurement data. |  |
| Step 2 (M) | Network slice management service provider calculates resource availability based on the information fetched in Step 1. The objective of calculation is to find an optimal capacity against the given CSP request, any CSP requirement changes, any resource availability status changes, and the current resource availability. The optimization algorithm is out of scope of the standardization. |  |
| Step 3 (M) | Network slice management service provider generates provisioning or modification policies based on the calculation results and proceeds with network slice (NSI and/or NSSI) provisioning or modification appropriately. | Network slice instance creation or modification/network slice subnet instance creation or modification use cases |
| Step 4 (M) | Network slice management service provider updates capacity availability information after provisioning or modification processes. |  |
| Ends when | The capacity calculation and associated policy generation end.. |  |
| Exceptions | One of the mandatory steps fails. |  |
| Post-conditions | Capacity planning policy for either provisioning or modification is generated. |  |
| Traceability | REQ-NSM_NSSM-FUN-3, REQ-NSM_NSM-FUN-9 |  |

2nd change

5.2	Requirements

5.2.1	Requirements for network slice management service

Editor’s Note: The name of the service in this clause needs to be revisited to align with the 5G network management architecture.

REQ-NSM_NSM-FUN-1	The network slice management service provider shall have the capability allowing its authorized consumer to request a network slice instance.

REQ-NSM_NSM-FUN-2	The network slice management service provider shall have the capability allowing its authorized consumer to send the network slice related requirements.

NOTE: The network slice related requirements include requirements such as area traffic capacity, charging, coverage area, isolation, end-to-end latency, mobility, overall user density, priority, service availability, service reliability, UE speed; see TS 22.261 [5] where these parameters are defined for end user services.

REQ-NSM_NSM-FUN-3	The network slice management service provider shall have the capability allowing its authorized consumer to request the deallocation of a network slice instance.

REQ-NSM_NSM–FUN-4	The network slice management service provider shall have the capability allowing its authorized consumer to request activation of a network slice instance.

REQ-NSM_NSM–FUN-5	The network slice management service provider shall have the capability allowing its authorized consumer to request deactivation of a network slice instance.

REQ-NSM_NSM-FUN-6	The network slice management service provider shall have the capability allowing its authorized consumer to request the modification of a network slice instance.

REQ-NSM_NSM-FUN-7	The network slice management service provider shall have the capability allowing its consumer to obtain the network slice management data.

REQ-NSM_NSM-FUN-8	The network slice management service provider shall have the capability allowing its authorized consumer to obtain the feasibility of provisioning the requested network slice instance.

REQ-NSM_NSM-FUN-9	The network slice management service provider shall have the capability allowing its authorized consumer to request the capacity planning of a network slice instance.

End of changes