3GPP TSG-SA5 Meeting #122	S5-187050

# Spokane,US, 12-16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0008 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use case and definitions of flow setup related measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2018/11/2 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The purpose of the PDU Session Resource Setup procedure is to assign resources on Uu and NG-U for one or several PDU session resources and the corresponding QoS flows, and to setup corresponding Data Radio Bearers for a given UE. The QoS flow setup related measurements are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following measurements: - number of QoS flows attempted to setup - number of QoS flows successfully established - number of QoS flows failed to setup |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The QoS flows setup measurement is missing, the UE setup the flow for traffic failure can’t be monitored . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.X (new), A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

5.1.1.X  QoS Measurement

5.1.1.X.1 Number of QoS flows attempted to setup

- This measurement provides the number of QoS flows attempted to setup. The measurement is split into subcounters per QoS level (5QI).

- CC.

- On receipt by the gNB of an INITIAL CONTEXT SETUP REQUEST or PDU SESSION RESOURCE SETUP REQUEST or PDU SESSION RESOURCE MODIFY REQUEST message, each requested QoS Flow Setup Request Item in the message is added to the relevant measurement per QoS level, the possible QoS levels are included in TS 38.413. The sum of all supported per QoS level measurements shall equal the total number of QoS flows attempted to setup. In case only a subset of per QoS level measurements is supported, a sum subcounter will be provided first.

- A single integer value. The number of measurements is equal to the number of 5QI plus a possible sum value identified by the .sum suffix.

- The measurement name has the form Fow.EstabAttNbr.5QI
where 5QI identifies the flow level quality of service class.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

5.1.1.X.2 Number of QoS  flows successfully established

- This measurement provides the number of QoS flows successfully established. The measurement is split into subcounters per QoS level (QCI).

- CC

- On transmission by the gNodeB INITIAL CONTEXT SETUP RESPONSE or PDU SESSION RESOURCE SETUP RESPONSE or PDU SESSION RESOURCE MODIFY RESPONSE message, each QoS flows successfully established is added to the relevant measurement per 5QI, the possible 5QIs are included in TS 38.413. The sum of all supported per 5QI measurements shall equal the total number of QoS flows successfully setup. In case only a subset of per 5QI measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of 5QIs plus a possible sum value identified by the .sum suffix.

- The measurement name has the form Flow.EstabSuccNbr.5QI
where 5QI identifies the flow level quality of service class.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

5.1.1.X.3 Number of QoS  flows failed to setup

- This measurement provides the number of QoS lows failed to setup. The measurement is split into subcounters per QoS level (QCI).

- CC

- On transmission by the gNodeB INITIAL CONTEXT SETUP RESPONSE or PDU SESSION RESOURCE SETUP RESPONSE or PDU SESSION RESOURCE MODIFY RESPONSE message, each QoS flows failed to setup is added to the relevant measurement per 5QI, the possible 5QIs are included in TS 38.413. The sum of all supported per 5QI measurements shall equal the total number of QoS flows failed to setup. In case only a subset of per 5QI measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of 5QIs plus a possible sum value identified by the .sum suffix.

- The measurement name has the form Flow.EstabFailNbr.5QI
where 5QI identifies the flow level quality of service class.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

Next modified section

A.x	 Monitor of call (/session) setup performance

Call (/session) setup is one of most important step to start delivering services by the networks to users. The QoS flow is the finest grained unit for monitoring.

The success or failure of a call (/session) setup directly impacts the quality level for delivering the service by the networks, and also the feeling of the end user. So the success or failure of call (/session) setup needs be monitored, this can be achieved by the calculation of call setup success rate which gives a direct view to evaluate the call setup performance, and the analysis of the specific reason causing the failure to find out the problem and ascertain the solutions.

End of modifications