3GPP TSG-SA5 Meeting #122	S5-187053

# Spokane,US, 12-16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0011 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use case and definitions of TB related measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2018/11/2 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The TB related Error Rate in UL/DL can reflect the BLER, initial BLER and residual BLER, and has an influence on MCS selection and user throughput. It can be helpful to estimate the performance of radio resource management like radio resource scheduling transport layer and be helpful in trouble shooting |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use case and definitions of TB related is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TB related Error Rate is missing, it is not ease to estimate the performance of radio resource management. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.X (new), 5.1.1.X.1 (new), 5.1.1.X.2 (new), 5.1.1.X.3 (new), 5.1.1.X.4 (new), 5.1.1.X.5 (new), 5.1.1.X.6 (new), 5.1.1.X.7 (new), 5.1.1.X.8 (new), 5.1.1.X.9 (new), 5.1.1.X.10 (new), A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

5.1.1.X TB related Measurements

5.1.1.X.1  Total Number of DL  initial TBs

a)  This measurement provides the total number of initial TBs transmitted on the downlink in a cell. HARQ retransmissions are excluded from this measurement.

b)  CC

c)  On transmission by the gNB of TB to UE during the period of measurement.

d)  A single integer value

e)  TB.TotNbrDlInitial

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

5.1.1.X.2  Intial Error Number of DL TBs

- This measurement provides the number of initial faulty TBs transmitted on the downlink in a cell.

- CC

- On receipt by the gNB of a NACK or DTX from UE which indicates a faulty reception of TB by UE at first HARQ feedback during the period of measurement.

- A single integer value.

- TB.IntialErrNbrDl

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

5.1.1.X.3  Total Number of DL TBs

- This measurement provides the total number of TBs transmitted on the downlink in a cell.The measurement is split into subcounters per layer at MU-MIMO case.

- CC

- On transmission by the gNB of TB to UE during the period of measurement.The measurement is split into subcounters per Layer at MU-MIMO case.A single integer value. The sum value identified by the .sum suffix.

- A single integer value

- TB.TotNbrDl.X

Where X identified by MU-MIMO maximum layer.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

5.1.1.X.4  Total Error Number of DL TBs

- This measurement provides the number of total faulty TBs transmitted on the downlink in a cell .The measurement is split into subcounters per layer at MU-MIMO case.

- CC

- On receipt by the gNB of a NACK or DTX from UE which indicates a faulty reception of TB by UE during the period of measurement. The measurement is split into subcounters per Layer at MU-MIMO case.A single integer value. The sum value identified by the .sum suffix.

- A single integer value.

- TB.ErrToltalNbrDl.X

Where X identified by MU-MIMO maximum  Layer .

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

5.1.1.X.5  Residual Error Number of DL TBs

- This measurement provides the number of final faulty TBs transmitted on the downlink in a cell at last HARQ retransmissions.

- CC

- On receipt by the gNB of a NACK or DTX from UE which indicates a faulty reception of TB by UE at the last HARQ feedback during the period of measurement.

- A single integer value.

- TB.ResidualErrNbrDl

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

5.1.1.X.6  Total Number of UL Initial TBs

- This measurement provides the total number of initial TBs on the uplink in a cell.

- CC

- On receipt by the gNB of TB from UE during the period of measurement.

- A single integer value.

- TB.TotNbrUlInit

- NRCellDU

g)	Valid for packet switched traffic

h)	5GS

5.1.1.X.7  Error Number of UL Initial TBs

- This measurement provides the number of initial faulty TBs on the uplink in a cell.

- CC

- On receipt by the gNB of a initial TB on which CRC fails or DTX from UE during the period of measurement.

- A single integer value.

- TB.ErrNbrUlInitial

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

5.1.1.X.8  Total Number of UL TBs

- This measurement provides the total number of TBs on the uplink in a cell.The measurement is split into subcounters per layer at MU-MIMO case.

- CC

- On receipt by the gNB of TB from UE during the period of measurement.The measurement is split into subcounters per Layer at MU-MIMO case.A single integer value. The sum value identified by the .sum suffix.

- A single integer value.

- TB.TotNbrUl.X

Where X identified by MU-MIMO maximum layer.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

5.1.1.X.9 Total Error Number of UL TBs

- This measurement provides the number of total faulty TBs on the uplink in a cell.  The measurement is split into subcounters per layer at MU-MIMO case.

- CC

- On receipt by the gNB of a TB on which CRC fails or DTX from UE during the period of measurement. The measurement is split into subcounters per Layer at MU-MIMO case.A single integer value. The sum value identified by the .sum suffix.

- A single integer value.

- TB.ErrToltalNbrUl.X

Where X identified by MU-MIMO maximum layer.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

5.1.1.X.10 Residual Error Number of UL TBs

- This measurement provides the number of final faulty TBs on the uplink in a cell.

- CC

- On receipt by the gNB of a TB on which CRC fails or DTX at last HARQ retransmissions from UE during the period of measurement.

- A single integer value.

- TB.ResidualErrNbrUl

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

Next modified section

A.x Monitor of BLER performance

The TB Error Rate in UL/DL can directly reflect the BLER, and has an influence on MCS selection and user throughput. It can be helpful to estimate the performance of radio resource management like radio resource schedulein transport layer and be helpful in trouble shooting. Furthermore, they should be taken into account to optimize the system performance. To obtain TB Error Rate by calculating, the number of total and error TBs transmitted in a cell should be monitored.

Next modified section

A.y	Monitor of ARQ and HARQ performance

Reliable Packet Delivery is one of the important Performance factor for a better User experience. HARQ retransmissions at the MAC layer ensure reliable packet delivery

In addition, RLC can be configured to operate in acknowledged mode for those applications that need very low packet drops and can tolerate a slightly higher delay from RLC retransmissions.

If a MAC PDU is not delivered, HARQ takes care of retransmitting (upto a maximum configurable number). If all the retransmissions fail at MAC layer, and if RLC is configured to operate in acknowledged mode, RLC’s ARQ mechanism will take care of any residual packet errors.

It is important to:

a) Maintain the block error rate or packet error rate within tolerable limits

b) Ensure that HARQ retransmissions take care of most packet errors, instead of relying on RLC layer retransmissions (which would increase the delay).

So, it is important to monitor the performance of these schemes.

HARQ Performance if viewed at MCS (Modulation Coded Scheme) can help in monitoring the MCS Performance also.

End of modifications