3GPP TSG-SA5 Meeting #122	S5-187192

# Spokane, US, 12-16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0010 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add network slice management interactions with severity type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2018-10-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The network slice priority as a feature has not been addressed. The priority related management interaction with NFV MANO is missing. Use case on how 3GPP management interaction to NFV MANO is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A use case added to illustrate management interaction with NFV MANO is about NS and VNF management operation severity sent to NFVO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.yy (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS 28.541. |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

5.1.yy	Management interaction with NFV MANO with severity types

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request creation of a 3GPP sub-network, management interactions with NFV MANO is needed. This management interaction will set NS and VNF management operation (e.g. LCM) severity types in relation with network slice subnet creation with higher network slice instance priority. |  |
| Actors and Roles | An authorized consumer of sub-network creation or modification service. |  |
| Telecom resources | VNF package(s) of the virtualized part of 3GPP NF(s); NSD(s) of the NS(s); ETSI NFV MANO system; Network creation service producer; NF configuration service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The non-virtualized part of the NFs (including completely non-virtualized NFs) constituting the 3GPP sub-network have been deployed; The VNF package(s) of the virtualized part of 3GPP NF(s) have been on-boarded to ETSI NFV MANO system; The NSD(s) of the NS realizing the 3GPP sub-network have been on-boarded to ETSI NFV MANO system. |  |
| Begins when | The authorized consumer needs to create a 3GPP sub-network. This creation operation is set as high priority due to support of high priority communication services in related network slice instance. |  |
| Step 1 (M) | The authorized consumer requests the sub-network creation service producer to create a 3GPP sub-network. The request needs to indicate the network capacity (e.g., the number of instances of each kind of NFs, and the capacity of each NF instance, for example, number of flows with certain QoS attributes to be supported), network topology information (e.g., the connections between NF instances), and the network QoS requirements (e.g., bandwidth and latency requirements of the interface between two NF instances). |  |
| Step 2 (M) | The network creation service producer interacts, or requests another network creation service producer to interact, with ETSI NFV MANO system to instantiate the NS(s) realizing the sub-network. Due to network slice priority and the management interaction identified as critical, the management interaction with NFV MANO will be identified by severity type (e.g. Critical). |  |
| Step 3 (M) | ETSI NFV MANO informs the NF configuration service producer about the instantiation of the NSs and the new VNFs. The management operation servirty type as informed to NFV MANO and it is used as input attribute for prioritisation of management operations when virtual resource, managed by NFV MANO, is in shortage or in disaster situation. |  |
| Step 4 (M) | The NF configuration service producer creates the MOI(s) of the 3GPP NFs that are realized by the newly instantiated VNF(s); there may be MOI(s) that specify the topology of the instantiated NSs. |  |
| Step 5 (M) | The sub-network creation service producer is using the NF configuration service to configure the 3GPP NF instance(s) that are constituting the subject 3GPP sub-network. | NF configuration service |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The 3GPP sub-network has been created. |  |
| Traceability | REQ-PRO_NW-FUN-1, REQ-PRO_NW-FUN-2 |  |

End of changes