3GPP TSG-SA5 Meeting #122	S5-187208

# Spokane,US, 12-16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0031 | rev | <Rev#> | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the performance measurement of MCS distribution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | MCS represents the modulation and coding schemes scheduled for the physical resources by NG-RAN which have an impacts on the transmission efficieny of the scheduled physical resources. Scheduled spectral efficiency comprehensively considers the MCS of physical resources scheduled by NG-RAN and the spatial multiplexing technologies used in the cell. Therefore, the measurement of MCS distribution is a useful metric reflecting the efficiency for PDSCH and PUSCH RBs. It is helpful for operator to optimize the scheduling of physical resources to improve the network efficiency and overall service quality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the performance measurement of MCS distribution in TS 28.552. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Scheduled MCS for PDSCH and PUSCH RBs can’t be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1.1.X(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[3]	3GPP TS 32.404: "Performance Management (PM); Performance measurements - Definitions and template".

[4]	3GPP TS 23.501: "System Architecture for the 5G System".

[5]	IETF RFC 5136: "Defining Network Capacity".

[6]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

[7]	3GPP TS 23.502: "Procedures for the 5G System".

[8]	3GPP TS 28.554: “Management and orchestration; 5G end to end Key Performance Indicators (KPI)”.

[9]	3GPP TS 32.425: “Performance Management (PM); Performance measurements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN)”

[10]	3GPP TS 32.451: "Key Performance Indicators (KPI) for Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Requirements”

[x]	3GPP TS 38.214: "NR; Physical layer procedures for data”

Second change

5.1.1.X	RF Measurements

5.1.1.X.1  	MCS Distribution in PDSCH

a)	This measurement provides the distribution of the MCS in different ranges of spectral efficiency. The spectral efficiency is obtained according to the MCS scheduled for PDSCH RB by NG-RAN, the RI (rank indicator) of the cell and Table 5.1.3.1-1, Table 5.1.3.1-2, Table 5.1.3.1-3 in clause 5.1.3.1, TS 38.214 [x]. Which table should be used depends on the configuration of PDSCH-Config described in TS 38.214 [x]. The ranges of spectral efficiency depends on the operator’s decisions.

b)	CC.

c)	This measurement is obtained by incrementing the appropriate measurement bin with the number of the PDSCH RBs according to the MCS scheduled for the PDSCH RBs and the mapped spectral efficiency.

d)	A set of integers, each representing the (integer) number of PDSCH RBs in the range represented by that bin.

e)	CARR.PdschMcsDist.BinX
where X represents the index of the spectral efficiency ranges.

f)	NRCellDU

g)	Valid for packet switching.

h)	5GS

5.1.1.X.2  	MCS Distribution in PUSCH

a)	This measurement provides the distribution of the MCS in different ranges of spectral efficiency. The spectral efficiency is obtained according to the MCS scheduled for PUSCH RB by NG-RAN, the RI (rank indicator) of the cell and Table 5.1.3.1-1, Table 5.1.3.1-2, Table 5.1.3.1-3 in clause 5.1.3.1, TS 38.214 [x]. Which table should be used depends on the configuration of PUSCH-Config described in TS 38.214 [x]. The ranges of spectral efficiency depends on the operator’s decisions.

b)	CC.

c)	This measurement is obtained by incrementing the appropriate measurement bin with the number of the PUSCH RBs according to the MCS scheduled for PUSCH RBs and the mapped spectral efficiency.

d)	A set of integers, each representing the (integer) number of PUSCH RBs in the range represented by that bin.

e)	CARR.PuschMcsDist.BinX
where X represents the index of the spectral efficiency ranges.

f)	NRCellDU

g)	Valid for packet switching.

h)	5GS

Second change

A.X	Monitoring of RF measurements

MCS represents the modulation and coding schemes scheduled for the physical resources by NG-RAN which have an impacts on the transmission efficieny of the scheduled physical resources. Scheduled spectral efficiency comprehensively considers the MCS of physical resources scheduled by NG-RAN and the spatial multiplexing technologies used in the cell. Therefore, the measurement of MCS distribution is a useful metric reflecting the efficiency for PDSCH and PUSCH RBs. It is helpful for operator to optimize the scheduling of physical resources to improve the network efficiency and overall service quality.

End of changes