3GPP TSG-SA5 Meeting #122	S5-187320

# Spokane,US, 12-16 November 2018												Revision of S5-187300

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.251 | CR | 0509 | rev | 2 | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Data Volume Uplink and Downlink definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14 |  |  |  |  |  | Date: |  |  | 2018-11-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Unclear definition of what data is counted in IP-CAN bearer / session charging. The use of “user plane payload” within GTP-U tunnelling can be interpreted in more than one way. For IP-CAN bearer charging & IP-CAN Session charging, when interworking with external IP networks, the inner IP packets of GTP-U tunnel should be counted, i.e. IP packets length. In some scenarios this isn’t equal to T-PDU with tailing garbage bytes behind the IP packets in T-PDU. TS29.281, clause 4.2.1 GTP-U Tunnel description has defined “inner IP packet in a GTPv1-U packet (T-PDU) is either - An IP packet sent to the UE/MS in the downlink direction over one or more tunnels from the external network identified by the APN. - An IP packet sent from a UE/MS in the uplink direction over one or more tunnels to the external network identified by the APN. “ Above inner IP packet of GTP-U tunnel description match with IP-CAN bearer / Session & service data flow concept. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change “the user plane payload” for tunnelling to refer to “inner IP packets when interworking with external IP networks, or T-PDU packets when interworking with external networks handling Non-IP data services”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | incorrect data volume counted for IP-CAN bearer charging & IP-CAN Session charging, when T-PDU contains tailing garbage bytes behind the inner IP packets (which cannot be transferred to external IP networks) as well as mismatch with data counted for Flow based Charging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 5.2.1.10.1, 5.3.1.1, 5.3.1.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.298 CR 0687 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[2] - [9]	Void.

[10]	3GPP TS 32.250: "Telecommunication management; Charging management; Circuit Switched (CS) domain charging".

[11] – [19]	Void.

[20]	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

[21] - [29]	Void.

[30]	3GPP TS 32.270: "Telecommunication management; Charging management; Multimedia Messaging Service (MMS) charging".

[31]	3GPP TS 32.271: "Telecommunication management; Charging management; Location Services (LCS) charging".

[32]	void.

[33]	3GPP TS 32.273: "Telecommunication management; Charging management; Multimedia Broadcast and Multicast Service (MBMS) charging"

[34] 	3GPP TS 32.274: "Telecommunication management; Charging management; Short Message Service (SMS) charging".

[35] - [49]	Void.

[50]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application".

[51]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) encoding rules description".

[52]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Records (CDR) file format and transfer".

[53]	3GPP TS 32.296: "Telecommunication management; Charging management; Online Charging System (OCS) applications and interfaces".

[54]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[55] - [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101]	3GPP TS 22.115: "Service aspects; Charging and billing".

[102] 	3GPP TS 22.011: "Service accessibility".

[103]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[104] - [199]	Void.

[200]	3GPP TS 22.060: "General Packet Radio Service (GPRS); Service description; Stage 1".

[201]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[202]	3GPP TS 29.078: "Customized Applications for Mobile network Enhanced Logic (CAMEL) 
Phase X;CAMEL Application Part (CAP) specification".

[203]	3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[204]	Void.

[205]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[206]	3GPP TS 23.078: "Customized Applications for Mobile network Enhanced Logic (CAMEL); 
Phase 4; Stage 2".

[207]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[208]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) acces".

[209]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[210]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[211]	3GPP TS 29.275: "Proxy Mobile IPv6 (PMIPv6) based Mobility and Tunnelling protocols; Stage 3".

[212]	3GPP TS 23.261: "IP flow mobility and seamless Wireless Local Area Network (WLAN) offload; Stage 2".

[213]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[214]	Void.

[215] 	3GPP TS 23.203: "Policy and charging control architecture".

[216]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[217]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[218] - [241]	Void.

[242]	3GPP TS 23.161: "Network-based IP flow mobility and Wireless Local Area Network (WLAN) offload; Stage 2".

[243] - [399]	Void.

[400]	Void.

[401]	Void.

[402]	IETF RFC 4006 (2005): "Diameter Credit-Control Application".

[403] - [599]	Void.

[600]	Broadband Forum TR-146: "Internet Protocol (IP) Sessions".

[601]	Broadband Forum TR-134: "Broadband Policy Control Framework (BPCF)".

Second change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations defined in TR 21.905 [100] and the following apply:

3G	3rd Generation

ABC	Application Based Charging

ADC	Application Detection and Control

AoC	Advice of Charge

APN	Access Point Name

ARP	Allocation and Retention Priority

BD	Billing Domain

CAMEL	Customised Applications for Mobile network Enhanced Logic

CCA	Credit-Control-Answer

CCR	Credit-Control-Request

CDF	Charging Data Function

CDR	Charging Data Record

CG	Charging Gateway

CGF	Charging Gateway Function

CI	Cell Identity

CIoT	Cellular Internet of Things

CP	Control PlaneCS	Circuit Switched

CSE	CAMEL Service Environment

CSG	Closed Subscriber Group

CSG ID	Closed Subscriber Group Identity

CTF	Charging Trigger Function

DCCA	Diameter Credit-Control Application

EPC	Evolved Packet Core

ePDG	Evolved Packet Data Gateway

EPS	Evolved Packet System

ECUR	Event Charging with Unit Reservation

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FBC	Flow Based bearer Charging

FQPC	Fully Qualified Partial CDR

GERAN	GSM EDGE Radio Access Network

GGSN	Gateway GPRS Support Node

GPRS	General Packet Radio Service

GSM	Global System for Mobile communication

GTP	GPRS Tunnelling Protocol

GTP'	The GPRS protocol used for CDR transport. 
It is derived from GTP with enhancements to improve transport reliability necessary for CDRs.

HeNB	Home eNode B

HNB	Home Node B

H(e)NB	HNB and HeNB

HLR	Home Location Register

HPLMN	Home PLMN

HRPD	High Rate Packet Data

HSGW	HRPD Serving Gateway

IE	Information Element

IEC	Immediate Event Charging

IETF	Internet Engineering Task Force

IFOM	IP Flow Mobility

IMEI	International Mobile Equipment Identity

IMSI	International Mobile Subscriber Identity

IP	Internet Protocol

IP-CAN	IP Connectivity Access Network

IPv4	Internet Protocol version 4

IPv6	Internet Protocol version 6

ISDN	Integrated Services Digital Network

ITU-T	International Telecommunication Union - Telecommunications standardization sector

LAC	Location Area Code

LR	Location Request

M-CDR	Mobility management generated CDR

MAPCON	Multi Access PDN Connectivity

MCC	Mobile Country Code (part of IMSI)

ME	Mobile Equipment

MLC	Mobile Location Center

MME	Mobility Management Entity

MMS	Multimedia Messaging Service

MNC	Mobile Network Code (part of IMSI)

MO	Mobile Originated

MO-LR	Mobile Originated Location Request

MS	Mobile Station

MSISDN	Mobile Station ISDN number

MT	Mobile Terminated

MT-LR	Mobile Terminated Location Request

NBIFOM	Network-based IP flow mobility

NE	Network Element

NI	Network Identifier (part of the APN)

NI-LR	Network Induced - Location Request

NSWO	Non-Seamless WLAN Offload

OAM&P	Operation, Administration, Maintenance and Provisioning

OCF	Online Charging Function

OCS	Online Charging System

OI	Operator Identifier (part of the APN)

P-GW	PDN Gateway

PCC	Policy and Charging Control

PCEF	Policy and Charging Enforcement Function

PCN	Packet switched Core network Node (SGSN, GGSN, S–GW, P–GW, TDF)

PDN	Packet Data Network

PDP	Packet Data Protocol (e.g. IP)

PDU	Packet Data Unit

PGW-CDR	P-GW (enhanced by FBC) generated CDR

PLMN	Public Land Mobile Network

PMIP	Proxy Mobile IP

PPP	Point-to-Point Protocol

PRA	Presence Reporting Area

PS	Packet Switched

QCI	QoS Class Identifier

QoS	Quality of Service

RAB	Radio Access Bearer

RAC	Routing Area Code

RANAP	Radio Access Network Application Part

RG	Residential Gateway

RNC	Radio Network Controller

RPC	Reduced Partial CDR

SAC	Service Area Code

S-CDR	SGSN (IP-CAN bearer) generated CDR

S-GW	Serving Gateway

SCUR	Session Charging with Unit Reservation

SDF	Service Data Flow

SGSN	Serving GPRS Support Node

SGW-CDR	S-GW (IP-CAN bearer) generated CDR

SIPTO	Selected IP Traffic Offload

SMS	Short Message Service

S-SMO-CDR	SGSN delivered Short message Mobile Originated CDR

S-SMT-CDR	SGSN delivered Short message Mobile Terminated CDR

T-PDU	Transport PDU

TDF	Traffic Detection Function

TDF-CDR	TDF generated CDR

TR	Technical Report

TS	Technical Specification

TWAG	Trusted WLAN Access Gateway

TWAN	Trusted WLAN Access Network

UMTS	Universal Mobile Telecommunications System

UP	User Plane

USIM	Universal Subscriber Identity Module

UTRAN	UMTS Terrestrial Radio Access Network

UWAN	Untrusted Wireless Access Network

Third change

##### 5.2.1.10.1	IP-CAN bearer charging

For the purpose of interoperator charging, the P-GW collects charging information per user per IP-CAN bearer. 
In case the P-GW is not aware of IP-CAN bearers, i.e. in case of PMIP based connectivity, P-GW collects charging information per IP-CAN session as it would be one IP-CAN bearer. IP-CAN bearer charging allows the P-GW to collect charging information related to data volumes sent to and received by the UE/MS, categorised by the QCI and ARP applied to the IP-CAN bearer. The user can be identified by MSISDN and/or IMSI, while the IP-CAN bearer can be determined by a unique identifier generated by the P-GW when creating an IP-CAN bearer. This identifier is forwarded to the S-GW/ePDG/ TWAG/SGSN so as to allow correlation of S-GW/ePDG/ TWAG/SGSN IP-CAN bearer CDRs with the matching P-GW charging information in the BD.

The amount of data counted in P-GW shall be based on:

-	In case of GTP based tunnelling and interworking with external IP networks: the inner IP packets in the GTP-U packet (T-PDU, see TS 29.281 [217])

-	In case of GTP based tunnelling and interworking with external networks handling other data services (e.g. non-IP, PPP): the T-PDU packets (see TS 29.281 [217])

-	In case of PMIP based protocol: the payload of the GRE tunnel

-	In case of DSMIPv6 based protocol: the payload of the tunnelling layer.

As minimum behaviour, the full payload shall be included. Time metering is started when IP-CAN bearer is activated.

NOTE 1:	The control plan address of the P-GW, together with the unique charging identifier assigned by the P-GW, enables the correlation of charging information. The control plane IP address of SGSN or P-GW (acting as GGSN) is the IP address used at Gn/Gp interface. 
The control plane IP address of S-GW or P-GW is the IP address used at S5/S8 interface. 
The control plane IP address of ePDG or P-GW is the IP address used at S2b interface. 
The control plane IP address of TWAG or P-GW is the IP address used at S2a interface.

When Charging per IP-CAN session is active and measurements for IP-CAN bearers are captured in the same CDR as FBC measurements, the following chargeable events are defined:

-	Start of the default bearer for an IP-CAN session when single access is used or start of the first default bearer for a multi-access PDN connection(i.e., when NBIFOM is accepted by PCRF). 
Upon encountering this event, a new CDR for the IP-CAN session is created and the data volume counts 
(i.e., uplink and downlink) are started and captured for the IP-CAN bearer.

NOTE 2:	Start of the default bearer or start of the first default bearer for an IP-CAN session is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	Addition of access to a PDN connection. 
Additional volume counts are started and captured for IP-CAN bearers of the access. New SGSN/S-GW/ePDG/TWAG address is added to data for the IP-CAN bearer in the CDR.

-	Start of a dedicated bearer for an IP-CAN session. 
Additional volume counts are started and captured for the dedicated bearer.

-	End of dedicated bearer in the P-GW. 
The counters and time stamps for the IP-CAN bearer are closed and resulting container added to the CDR.

-	Removal of access from a multi-access PDN connection. 
The counters and time stamps for the IP-CAN bearers of the removed access are closed and resulting containers added to CDR.

-	Access of a multi-access PDN connection becomes unusable. 
The counters and time stamps for the IP-CAN bearers of the unusable access are closed and resulting containers added to the CDR.

-	Access of a multi-access PDN connection becomes usable. 
New volume counts are started and captured for all bearers of the access that has become usable. These may not be the same as those that were previously active when the access became unusable due to changes in PCC Rules.

-	End of IP-CAN session (i.e. end of the default bearer for a single access PDN connection or end of the last default bearer for a multi-access PDN connection) in the P-GW. 
The counters and time stamps for all IP-CAN bearers and the resulting containers added to the CDR. The CDR is closed.

NOTE 3:	The End of IP-CAN session event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	Serving node (e.g. SGSN/S-GW/ePDG/TWAG) change in the P-GW. 
New SGSN/S-GW/ePDG/TWAG address is added to data for the IP-CAN bearer in the CDR.

-	Expiry of an operator configured time limit for keeping a CDR open. 
This event closes all counters. The resulting containers are added to the CDR and the CDR is closed. 
A new CDR is opened if the IP-CAN session is still active.

NOTE 4:	The expiry of an operator configured time limit for keeping a CDR open event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	Expiry of an operator configured time limit per IP-CAN bearer. 
The counters and time stamps for the IP-CAN bearer are closed and added to the CDR. A new IP-CAN bearer traffic volume container is opened if the IP-CAN bearer is still active.

-	Expiry of an operator configured data volume limit per IP-CAN session. 
This event closes the CDR and a new one is opened if the IP-CAN session is still active.

NOTE 5:	The expiry of an operator configured data volume limit per IP-CAN session event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	Expiry of an operator configured data volume limit per IP-CAN bearer. 
The counters and time stamps are closed and added to the CDR. A new IP-CAN bearer traffic volume container is opened if the IP-CAN bearer is still active.

-	Change of charging condition specific to APN-AMBR change. 
This event closes the CDR, and a new one is opened if the IP-CAN session is still active.

-	Change of charging condition specific to IP-CAN bearer modification QoS change. 
When this event is encountered, all counts and time stamps for the modified bearer are captured and new counts and time stamps for the specific bearer are started.

-	Change of charging condition. 
IP-CAN bearer modification except QoS change (e.g. SGSN change, S-GW change, user location change, user CSG information change, change of UE presence in Presence Reporting Area(s) , change of 3GPP PS Data Off status), or tariff time change. When this event is encountered, all current configured counts and time stamps are captured and new counts and time stamps for all active bearers are started. 
IP-CAN bearer modification except QoS change events are associated with one access of a multi-access PDN connection and all counts associated with affected access are identified with the specific event. All counts associated with the other access of a multi-access PDN connection are identified as an indirect change of charging condition.

Editor’s Note: The handling of  "Serving PLMN Rate Control change", "APN Rate Control change" events when charging per IP-CAN session applies is FFS.

NOTE 7:	The change of charging condition event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

Editor’s Note: whether the special case of user location reporting on dedicated bearer release triggers a change of charging condition for the IP-CAN session is ffs.

-	Intersystem change (e.g. change of radio interface from GSM to UMTS, RAT change) for any connected access visible in the P-GW. 
This event closes the CDR, and a new one is opened if the IP-CAN session is still active.

NOTE 8:	The intersystem change event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	PLMN change visible in the P-GW. This event closes the CDR. 
A new one is opened if the IP-CAN session is still active.

NOTE 9:	The PLMN change event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	MS Timezone change visible in the P-GW. This event closes the CDR. 
A new one is opened if the IP-CAN session is still active.

NOTE 10:	The MS Timezone change event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	Expiry of an operator configured limit of number of charging condition changes per IP-CAN session. 
This event closes the CDR, and a new one is opened if the IP-CAN session is still active.

NOTE 11:	The expiry of an operator configured limit of number of charging condition changes event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

Management intervention may also force trigger a chargeable event.

NOTE 12:	For non-IP type PDN connection, the following do not apply:

-	Addition of access to a PDN connection;

-	Start of a dedicated bearer for an IP-CAN session;

-	End of dedicated bearer in the P-GW;

-	Removal of access from a multi-access PDN connection;

-	Access of a multi-access PDN connection becomes unusable;

-	Access of a multi-access PDN connection becomes usable.

Fourth change

#### 5.3.1.1	IP-CAN bearer charging

IP-CAN bearer online charging is achieved by FBC online charging, see clause 5.3.1.2. When the IP-CAN bearer is online charged by means of FBC, the quota handling shall also be based on the use of a Rating Group/Service Identifier. 
The value of this IP-CAN bearer specific Rating Group/Service Identifier shall be vendor specific.

The amount of data counted with IP-CAN bearer specific Rating Group/Service Identifier shall be based on:

-	In case of GTP based tunnelling and nterworking with external IP networks: the inner IP packets in the GTP-U packet (T-PDU, see TS 29.281 [217])

-	In case of GTP based tunnelling and interworking with external networks handling other data services (e.g. non-IP, PPP): the T-PDU packets (see TS 29.281 [217])

-	In case of PMIP based protocol: the payload of the GRE tunnel

-	In case of DSMIPv6 based protocol: the payload of the tunnelling layer

As minimum behaviour, the full payload shall be included. Time metering is started when IP-CAN bearer is activated.

NOTE 1:	P-GW is aware of bearers in case of GTP based connectivity. In case of any other PMIP based connectivity, P-GW is aware of IP-CAN sessions only. If P-GW is not aware of IP-CAN bearers, P-GW collects charging information per IP-CAN session as it would be just one IP-CAN bearer.

NOTE 2: 	The control plane IP address of SGSN or P-GW (acting as GGSN) is the IP address used at Gn/Gp interface.

The control plane IP address of  S-GW or P-GW is the IP address used at S5/S8 interface. 
The control plane IP address of ePDG or P-GW is the IP address used at S2b interface.

The control plane IP address of TWAG or P-GW is the IP address used at S2a interface.

Fifth change

##### 5.3.1.6.1	IP-CAN bearer charging

IP-CAN bearer online charging is achieved by FBC online charging, see clause 5.3.1.6.2. When the IP-CAN session is online charged by means of FBC, the quota handling shall also be based on the use of a Rating Group. The value of this IP-CAN bearer specific Rating Group or combination of the Rating Group and Service Identifier shall be vendor specific.

The amount of data counted with IP-CAN bearer specific Rating Group/Service Identifier shall be based on:

-	In case of GTP based tunnelling and interworking with external IP networks: the inner IP packets in the GTP-U packet (T-PDU, see TS 29.281 [217])

-	In case of GTP based tunnelling and interworking with external networks handling other data services (e.g. non-IP, PPP): the T-PDU packets (see TS 29.281 [217])

-	In case of PMIP based protocol: the payload of the GRE tunnel

-	In case of DSMIPv6 based protocol: the payload of the tunnelling layer

As minimum behaviour, the full payload shall be included. Time metering is started when IP-CAN session is activated. 
All bearers in the IP-CAN session are included in the single vendor-specific Rating Group/Service Identifier.

NOTE 1:	P-GW is aware of bearers in case of GTP based connectivity. In case of any other PMIP based connectivity, P-GW is aware of IP-CAN sessions only. If P-GW is not aware of IP-CAN bearers, P-GW collects charging information per IP-CAN session as it would be just one IP-CAN bearer.

NOTE 2: 	The control plane IP address of SGSN or P-GW (acting as GGSN) is the IP address used at Gn/Gp interface. 
The control plane IP address of S-GW or P-GW is the IP address used at S5/S8 interface. 
The control plane IP address of ePDG or P-GW is the IP address used at S2b interface. 
The control plane IP address of TWAG or P-GW is the IP address used at S2a interface.

NOTE 3:	The vendor-specific Rating Group/Service Identifier may be configurable through the use of a predefined PCC rule in the P-GW.

End of changes