3GPP TSG-SA5 Meeting #122	S5-187322

# Spokane,US, 12-16 November 2018												Revision of S5-187076

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0007 | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Allow updating of Notify URI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-DCH |  |  |  |  |  | Date: |  |  | 2018-11-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For the Nchf_SpendingLimitControl service the Norify URI can be updated, but not in Nchf_ConvergedCharging |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the possibility to change Notify URI at update |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In a failover scenario in the NF it might be needed to have the possibility to update the Notify URI. This will also make the two services more consistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.290 CR 0020, TS 32.291 CR 0012 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

### 6.2.2	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by SMF for 5G data connectivity converged charging.

The Operation Types are listed in the following order: I (Initial)/U (Update)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as IUTE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IUT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. IU-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.2.2.1 defines the basic structure of the supported fields in the Charging Data Request message for 5G data connectivity converged charging.

Table 6.2.2.1: Supported fields in Charging Data Request message

| Information Element | Node Type | SMF (FBC) | SMF (QBC) |
| --- | --- | --- | --- |
|  | Supported Operation Types | I/U/T/E | I/U/T/E |
| Session Identifier |  | -UT- | -UT- |
| Subscriber Identifier |  | IUT- | IUT- |
| NF Consumer Identification |  | IUT- | IUT- |
| Invocation Timestamp |  | IUT- | IUT- |
| Invocation Sequence Number |  | IUT- | IUT- |
| Notify URI |  | IU-- | IU-- |
| Triggers |  | -UT- | -UT- |
| Multiple Unit Usage |  | IUT- | - |
| Rating Group |  | IUT- | - |
| Requested Unit |  | IU-- | - |
| Used Unit Container |  | -UT- | - |
| Triggers |  | -UT- | - |
| PDU Container Information |  | -UT- | - |
| UPF ID |  | IUT- | IUT- |
| PDU Session Charging Information |  | IUT- | IUT- |
| Charging Id |  | IUT- | IUT- |
| User Information |  | IUT- | IUT- |
| User Location Info |  | IUT- | IUT- |
| UE Time Zone |  | IUT- | IUT- |
| Presence Reporting Area Information |  | -UT- | -UT- |
| PDU Session Information |  | IUT- | IUT- |
| PDU Session ID |  | IUT- | IUT- |
| Network Slice Instance Identifier |  | IUT- | IUT- |
| PDU Type |  | IUT- | IUT- |
| PDU Address |  | IUT- | IUT- |
| SSC Mode |  | IUT- | IUT- |
| SUPI PLMN ID |  | IUT- | IUT- |
| Serving Network Function ID |  | IUT- | IUT- |
| Serving CN PLMN ID |  | IUT- | IUT- |
| RAT Type |  | IUT- | IUT- |
| Data Network Name Identifier |  | IUT- | IUT- |
| QoS Information |  | IUT- | IUT- |
| PDU session start Time |  | I--- | I--- |
| PDU session stop Time |  | --T- | --T- |
| Diagnostics |  | --T- | --T- |
| Charging Characteristics |  | IUT- | IUT- |
| Charging Characteristics Selection Mode |  | IUT- | IUT- |
| Charging Rule Base Name |  | IUT- | - |
| 3GPP PS Data Off Status |  | IUT- | IUT- |
| Session Stop Indicator |  | --T- | --T- |
| Unused Quota Timer |  | IU-- | - |
| Roaming QBC information |  | - | IUT- |
| Multiple QFI container |  | - | IUT- |
| UPF ID |  | - | IUT- |
| Roaming Charging Profile |  | - | IU-- |

Table 6.2.2.2 defines the basic structure of the supported fields in the Charging Data Response message for 5G data connectivity converged charging.

Table 6.2.2.1: Supported fields in Charging Data Response message

| Information Element | Node Type | SMF (FBC) | SMF (QBC) |
| --- | --- | --- | --- |
|  | Supported Operation Types | I/U/T/E | I/U/T/E |
| Session Identifier |  | I--- | I--- |
| Invocation Timestamp |  | IUT- | IUT- |
| Invocation Result |  | IUT- | IUT- |
| Invocation Sequence Number |  | IUT- | IUT- |
| Session Failover |  | IU-- | IUT- |
| Triggers |  | IU-- | IU-- |
| Multiple Unit information |  | IU-- | - |
| Result Code |  | IU-- | - |
| Rating Group |  | IU-- | - |
| UPF ID |  | IU-- | - |
| Granted Unit |  | IU-- | - |
| Validity Time |  | IU-- | - |
| Final Unit Indication |  | IU-- | - |
| Time Quota Threshold |  | IU-- | - |
| Volume Quota Threshold |  | IU-- | - |
| Unit Quota Threshold |  | IU-- | - |
| Quota Holding Time |  | IU-- | - |
| Triggers |  | IU-- | - |
| PDU Session Charging Information |  | IU-- | IU-- |
| Charging Id |  | - | - |
| User Information |  | - | - |
| User Location Info |  | - | - |
| UE Time Zone |  | - | - |
| Presence Reporting Area Information |  | IU-- | IU-- |
| PDU Session Information |  | IU-- | IU-- |
| PDU Session ID |  | - | - |
| Network Slice Instance Identifier |  | - | - |
| PDU Type |  | - | - |
| PDU Address |  | - | - |
| SSC Mode |  | - | - |
| SUPI PLMN ID |  | - | - |
| Serving Network Function ID |  | - | - |
| Serving CN PLMN ID |  | - | - |
| RAT Type |  | - | - |
| Data Network Name Identifier |  | - | - |
| QoS Information |  | - | - |
| PDU session start Time |  | - | - |
| PDU session stop Time |  | - | - |
| Diagnostics |  | - | - |
| Charging Characteristics |  | - | - |
| Charging Characteristics Selection Mode |  | - | - |
| Charging Rule Base Name |  | - | - |
| 3GPP PS Data Off Status |  | - | - |
| Session Stop Indicator |  | - | - |
| Unused Quota Timer |  | IU-- | - |
| Roaming QBC information |  | - | IU-- |
| Multiple QFI container |  | - | - |
| UPF ID |  | - | - |
| Roaming Charging Profile |  | - | IU-- |

End of changes