3GPP TSG-SA5 Meeting #122	S5-187337

# Spokane,US, 12th Nov 2018 -16th Nov 2018                                             revision of S5-187154

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0035 | rev | 1 | Current version: | 15.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR Rel-15 TS 28.541 Correction of missing 5G NRM NRSectorCarrier IOC attributes. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Pivotal Commware |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-5GNRM |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is a misalignment of the defined attributes for NR NRM IOC NRSectorCarrier with clause 5.1 Requirement REQ-NGRAN_NRM-CON-001 of TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM); Stage 1" [10]. Requirement is: “The NRM definitions shall support management of NG-RAN, containing gNB or/and ng-eNB.” NG-RAN 5G NR is beam-centric—if it is anything—and NR NRM of gNB transmission points include NR gNB beam needing to be represented by NRSectorCarrier IOC, which represents transmission points. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add missing beam-related attributes in NG-RAN 5G NR gNB’s NRSectorCarrier IOC. Add missing XML, JSON, and YANG 3GPP NR and NG-RAN NRM Solution Sets (Appendices C, D, & E). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-15 systems will not represent or support the new NR gNB beam resources from management perspective. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (2, 4.3.6, 4.4.1, C.4.3, D.3.2, D.4.3, E.4.2, E.4.3.9) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[6]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[7]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[8]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[9]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[10]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM);Stage 1".

[11]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[14]	3GPP TS 36.410: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 general aspects and principles".

[15]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol".

[16]	3GPP TS 36.425: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 interface user plane protocol".

[17]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS)".

[18]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

[19]	3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[20]	3GPP TS 28.702: "Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[21]	3GPP TS 28.708: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[22]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[23]	3GPP TS 29.510: "5G system; Network Function Repository Services; Stage 3".

[24]	3GPP TS 29.531: "5G System; Network Slice Selection Services Stage 3".

[25]	3GPP TS 32.152: "Telecommunication management; Integration Reference Point (IRP) Information Service (IS) Unified Modelling Language (UML) repertoire".

[26]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[27]	3GPP TS 28.554: "Management and orchestration; 5G End to end Key Performance Indicators (KPI)".

[28]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[29]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualisation (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[30]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[31]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[32]	3GPP TS 38.211: "NR; Physical channels and modulation".

[33]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[34]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[35]	3GPP TS 28.532: "Management and orchestration; Management services".

[36]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)"

[37]	IETF RFC 791: "Internet Protocol".

[38]	IETF RFC 2373: "IP Version 6 Addressing Architecture".

[39]	IEEE 802.1Q: "Media Access Control Bridges and Virtual Bridged Local Area Networks".

[40]	ETSI GS NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV); Management and Orchestration; Report on NFV Information Model".

[41]	3GPP TS 38.104: "Base Station (BS) radio transmission and reception".

[42]	3GPP TS 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz ".

[43]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

End of modified section

2nd modified section

### 4.3.6	NRSectorCarrier

#### 4.3.6.1	Definition

This <<IOC>>NRSectorCarrier represents the resources of each transmission point included in the cell. These in general have different physical locations (of the antennae), and possibly different frequencies or bandwidths. The UE is not directly aware of which NRSectorCarrier resources the network uses for its connection.

An NR sector-carrier can have downlink, uplink or both as specified by txDirection. Attributes related to unavailable direction (DL or UL) shall not be set.

Additional NRSectorCarriers not directly associated to one cell only can also be configured.

If a value of arfcnDL, arfcnUL, bsChannelBwDL or bsChannelBwUL can be derived unambiguously from the referring cell, then that attribute needs not be present. That will not be possible if the NRSectorCarrier is used for supplementary uplink, if it is not directly associated to a cell, or if the sector-carrier uses only a part of the cell's channel bandwidth. Thus, at least in those cases the applicable attributes must be present and their values need to be set.

#### 4.3.6.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| txDirection | M | T | T | F | T |
| configuredMaxTxPower | CM | T | T | F | T |
| arfcnDL | CM | T | T | F | T |
| arfcnUL | CM | T | T | F | T |
| bsChannelBwDL | CM | T | T | F | T |
| bsChannelBwUL | CM | T | T | F | T |
| nrBePtPhi | CM | T | T | F | T |
| nrBePtTheta | CM | T | T | F | T |
| nrBeWPhi | CM | T | T | F | T |
| nrBeWTheta | CM | T | T | F | T |
| attribute related to role |  |  |  |  |  |
| sectorEquipmentFunction | M | T | T | F | T |

#### 4.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| configuredMaxTxPower | Condition: The sector-carrier has a downlink. |
| arfcnDL | Condition: The sector-carrier has a downlink AND the value differs from the referring cell's value of arfcnDL. |
| arfcnUL | Condition: The sector-carrier has an uplink AND the value differs from the referring cell's value of arfcnUL. |
| bsChannelBwDL | Condition: The sector-carrier has a downlink AND the value differs from the referring cell's value of bsChannelBwDL. |
| bsChannelBwUL | Condition: The sector-carrier has an uplink AND the value differs from the referring cell's value of bsChannelBwUL. |

#### 4.3.6.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

End of modified section

3rd modified section

### 4.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |  |
| --- | --- | --- | --- |
| administrativeState | It indicates the administrative state of the NRCellDU. It describes the permission to use or prohibition against using the cell, imposed through the OAM services. allowedValues: "LOCKED", "SHUTTING DOWN", "UNLOCKED". The meaning of these values is as defined in ITU-T Recommendation X.731 [18]. See Annex A for Relation between the "Pre-operation state of the gNB-DU Cell" and administrative state relevant in case of 2-split and 3-split deployment scenarios. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Locked isNullable: False |  |
| operationalState | It indicates the operational state of the NRCellDU instance. It describes whether the resource is installed and partially or fully operable (Enabled) or the resource is not installed or not operable (Disabled). allowedValues: "ENABLED", "DISABLED". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| cellState | It indicates the usage state of the NRCellDU instance. It describes whether the cell is not currently in use (Idle), or currently in use but not configured to carry traffic (Inactive) or is currently in use and is configured to carry traffic (Active). The Inactive and Active definitions are in accordance with TS 38.401 [4]: "Inactive: the cell is known by both the gNB-DU and the gNB-CU. The cell shall not serve UEs; Active: the cell is known by both the gNB-DU and the gNB-CU. The cell should be able to serve UEs." "allowedValues: "IDLE", "INACTIVE", "ACTIVE". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| arfcnDL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for downlink allowedValues: See TS 38.104 [12] subclause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in subclause 5.4.2.3. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnUL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for uplink allowedValues: See TS 38.104 [12] subclause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in subclause 5.4.2.3. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnSUL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for supplementary uplink allowedValues: See TS 38.104 [12] subclause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in subclause 5.4.2.3. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwDL | BS Channel BW in MHz. for downlink allowedValues: See BS Channel BW in TS 38.104 [12], subclause 5.3.​ |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwUL | BS Channel BW in MHz.for uplink allowedValues: See BS Channel BW in TS 38.104 [12], subclause 5.3.​ |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwSUL | BS Channel BW in MHz.for supplementary uplink allowedValues: See BS Channel BW in TS 38.104 [12], subclause 5.3.​ |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| configuredMaxTxPower | This is the maximum possible for all downlink channels, used simultaneously in a cell, added together. allowedValues: TBD | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| cyclicPrefix | Cyclic prefix as defined in TS 38.211 [32], subclause 4.2. allowedValues: "NORMAL", "EXTENDED". | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| localAddress | This parameter specifies the localAddress including IP address and VLAN ID used for initialization of the underlying transport. First string is IP address, IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). Second string is VLAN Id. (See IEEE 802.1Q [39]), | type: String multiplicity: 2 isOrdered: True isUnique: N/A defaultValue: None isNullable: False |  |
| remoteAddress | Remote address including IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| gNBId | It identifies a gNB within a PLMN. The gNB ID is part of the NR Cell Identifier (NCI) of the gNB cells. See "gNB Identifier (gNB ID)" of subclause 8.2 of TS 38.300 [3]). See "Global gNB ID" in subclause 9.3.1.6 of TS 38.413 [5]. allowedValues: 0..4294967295 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| gNBIdLength | This indicates the number of bits for encoding the gNB ID. See "Global gNB ID" in subclause 9.3.1.6 of TS 38.413 [5]. allowedValues: 22 .. 32. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| gNBDUId | It uniquely identifies the DU at least within a gNB. See 'gNB-DU ID' in subclause 9.3.1.9 of 3GPP TS 38.473 [8]. allowedValues: Not applicable | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| gNBCUName | It identifies the Central Entity of a NR node, see subclause 9.2.1.4 of 3GPP TS 38.473 [8]. allowedValues: Not applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| gNBDUName | It identifies the Distributed Entity of a NR node, see subclause 9.2.1.5 of 3GPP TS 38.473 [8]. allowedValues: Not applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| nCI | It uniquely identifies a NR cell within a PLMN. The NR Cell Global identifier (NCGI) is constructed from the PLMN identity the cell belongs to and the NR Cell Identifier (NCI) of the cell. See relation between NCI and NCGI subclause 8.2 of TS 38.300 [3]. allowedValues: Not applicable | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |  |
| nRPCI | This holds the Physical Cell Identity (PCI) of the NR cell. allowedValues: See 3GPP TS 36.211 subclause 6.11 for legal values of pci. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| nRTAC | This holds the identity of the common Tracking Area Code for the PLMNs. allowedValues: a) It is the TAC or Extended-TAC. b) A cell can only broadcast one TAC or Extended-TAC. See TS 36.300, subclause 10.1.7 (PLMNID and TAC relation). c) TAC is defined in subclause 19.4.2.3 of 3GPP TS 23.003 [13] and Extended-TAC is defined in subclause 9.3.1.29 of 3GPP TS 38.473 [8]. | type: Bitstring multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| sNSSAIList | It represents the list of S-NSSAI the managed object is supporting, NSSAI is a set of supported S-NSSAI(s), an S-NSSAI is comprised of a SST (Slice/Service type) and an optional SD (Slice Differentiator) field, (See 3GPP TS 23.003 [13]). allowedValues: See 3GPP TS 23.003 [13] | type: <<S-NSSAI>> multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| rRMPolicyType | Type of the RRM policy. The value 0 denotes use of the rRMPolicy. The value 1 denotes use of the rRMPolicyNSSIId, rRMPolicyRatio allowedValues: 0 : 65535 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| rRMPolicyNSSIId | The list of NSSI identifiers (nSSIId values defined in the clause 6 ) for which rRMPolicyRatio values are specified | type: DN multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| rRMPolicyRatio | The RRM policy setting the ratio for the split of the Radio resources between the supported NSIs. It is the list of target percentage values assigned to the corresponding rRMPolicyNSSIId values. Every value specifies the percentage of PRBs to be allocated to the corresponding NSSI, in average over time. The sum of the values shall be less or equal 100. allowedValues: 0 : 100 NOTE: The averaging time interval is implementation dependent | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| rRMPolicy | It represents RRM policy which includes guidance for split of radio resources between multiple slices the cell supports in case when the rRMPolicyType is absent or equal to 0.. The RRM policy is implementation dependent. allowedValues: Not applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| subcarrierSpacing | Subcarrier spacing configuration for a BWP. See subclause 5 in TS 38.104 [12]. AllowedValues: [15, 30, 60, 120] depending on the frequency range FR1 or FR2. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| txDirection | Indicates if the transmission direction is downlink (DL), uplink (UL) or both downlink and uplink (DL and UL). allowedValues: "DL", "UL", "DL and UL" |  | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bwpContext | It identifies whether the object is used for downlink, uplink or supplementary uplink. allowedValues: "DL", "UL", "SUL" |  | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isInitialBwp | It identifies whether the object is used for initial or other BWP. allowedValues: "INITIAL", "OTHER" |  | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| startRB | Offset in common resource blocks to common resource block 0 for the applicable subcarrier spacing for a BWP. This corresponds to N_BWP_start, see subclause 4.4.5 in TS 38.211 [32]. allowedValues: 0 to N_grid_size – 1, where N_grid_size equals the number of resource blocks for the BS channel bandwidth, given the subcarrier spacing of the BWP. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| numberOfRBs | Number of physical resource blocks for a BWP. This corresponds to N_BWP_size, see subclause 4.4.5 in TS 38.211 [32]. allowedValues: 1 to N_grid_size – startRB of the BWP. Se startRB for definition of N_grid_size. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrBePtPhi | Horizontal beamforming pointing angle (beam peak direction) in the (Phi) φ-axis in degrees. See subclauses 3.2 in TS 38.104 [41] and 7.3 in TS 38.901 [42]. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrBePtTheta | Vertical beamforming pointing angle (beam peak direction) in the (Theta) θ-axis in degrees. See subclauses 3.2 in TS 38.104 [41] and 7.3 in TS 38.901 [42]. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrBeWPhi | Horizontal beamforming half-power (3dB down) beamwidth in the (Phi) φ-axis in degrees. See subclauses 3.2 in TS 38.104 [41] and 7.3 in TS 38.901 [42]. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrBeWTheta | Vertical beamforming half-power (3dB down) beamwidth in the (Theta) θ-axis in degrees. See subclauses 3.2 in TS 38.104 [41] and 7.3 in TS 38.901 [42]. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of modified section

4th modified section

## C.4.3	XML schema "nRNrm.xsd"

<?xml version="1.0" encoding="UTF-8"?>

<!--

3GPP TS 28.541 NR Network Resource Model

XML schema definition

nrNrm.xsd

-->

<schema xmlns="http://www.w3.org/2001/XMLSchema"

xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"

xmlns:nn="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"

xmlns:en="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"

xmlns:epc="http://www.3gpp.org/ftp/specs/archive/28_series/28.709#epcNrm"

xmlns:sm="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"

xmlns:ngc="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

xmlns:sp="http://www.3gpp.org/ftp/specs/archive/28_series/28.629#sonPolicyNrm"

targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm" elementFormDefault="qualified">

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.709#epcNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.629#sonPolicyNrm"/>

<simpleType name="GnbId">

<restriction base="unsignedLong">

<maxInclusive value="4294967295"/>

</restriction>

</simpleType>

<simpleType name="Nci">

<restriction base="unsignedLong">

<maxInclusive value="68719476735"/>

</restriction>

</simpleType>

<complexType name="Ncgi">

<sequence>

<element name="plmnId" type="en:PLMNId" minOccurs="0"/>

<element name="Nci" type="nn:Nci" minOccurs="0"/>

</sequence>

</complexType>

<simpleType name="Pci">

<restriction base="unsignedShort">

<maxInclusive value="503"/>

<!-- Minimum value is 0, maximum value is 3x167+2=503 -->

</restriction>

</simpleType>

<simpleType name="NgTac">

<restriction base="unsignedLong">

<maxInclusive value="16777215"/>

<!—5G TAC is 3-octets length -->

</restriction>

</simpleType>

<simpleType name="GnbDuId">

<restriction base="unsignedLong">

<maxInclusive value="68719476735"/>

<!-- Minimum value is 0, maximum value is 2^36-1=68719476735 -->

</restriction>

</simpleType>

<simpleType name="GnbName">

<restriction base="string">

<minLength value="1"/>

<maxLength value="150"/>

</restriction>

</simpleType>

<simpleType name="CyclicPrefix">

<restriction base="integer">

<enumeration value="15"/>

<enumeration value="30"/>

<enumeration value="60"/>

<enumeration value="120"/>

</restriction>

</simpleType>

<complexType name="IpEndPoint">

<sequence>

<element name="ipv4Address" type="string"/>

<element name="ipv6Address" type="string"/>

<element name="ipv6Prefix" type="string"/>

<element name="port" type="integer"/>

<element name="vlanId" type="integer"/>

</sequence>

</complexType>

<element name="GNBDUFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="gnbId" type="nn:GnbId"/>

<element name="gnbDUId" type="nn:GnbDuId"/>

<element name="gnbDuName" type="nn:GnbName" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="nn:NRCellDU"/>

<element ref="nn:EP_F1C"/>

<element ref="nn:EP_F1U"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="GNBCUCPFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="gnbId" type="nn:GnbId" />

<element name="gnbCuName" type=" nn:GnbName" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList" />

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="nn:NRCellCU"/>

<element ref="nn:EP_F1C"/>

<element ref="nn:EP_E1"/>

<element ref="nn:EP_XnC"/>

<element ref="nn:EP_X2C"/>

<element ref="nn:EP_NgC"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="GNBCUUPFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="gNBId" type="nn:GnbId"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="nn:EP_E1"/>

<element ref="nn:EP_F1U"/>

<element ref="nn:EP_XnU"/>

<element ref="nn:EP_NgU"/>

<element ref="nn:EP_X2U"/>

<element ref="nn:EP_S1U"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRCellCU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="nCGI" type="nn:Ncgi"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="nSSAIList" type="ngc:NssaiList" minOccurs="0"/>

<element name="rRMPolicy" type="string" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRCellDU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="nCGI" type="nn:Ncgi" minOccurs="0"/>

<element name="operationalState" type="sm:operationalStateType" minOccurs="0"/>

<element name="administrativeState" type="sm:administrativeStateType"

minOccurs="0"/>

<element name="availabilityStatus" type="sm:availabilityStatusType"

minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="nSSAIList" type="ngc:NssaiList" minOccurs="0"/>

<element name="nRpci" type="nn:Pci" />

<element name="nRTac" type="nn:NgTac" />

<element name="bSChannelBw" type="integer"/>

<element name="fDDCarrierDL" type="integer"/>

<element name="fDDCarrierUL" type="integer"/>

<element name="cyclicPrefixDL" type="nn:CyclicPrefix"/>

<element name="cyclicPrefixUL" type="nn:CyclicPrefix"/>

<element name="fDDBwpCommonCarrierFreqDL" type="integer" minOccurs="0"/>

<element name="fDDBwpCommonCarrierFreqUL" type="integer" minOccurs="0"/>

<element name="fDDBwpCommonChannelBwDL" type="integer" minOccurs="0"/>

<element name="fDDBwpCommonChannelBwUL" type="integer" minOccurs="0"/>

<element name="tDDBwpCommonCarrierFreq" type="integer" minOccurs="0"/>

<element name="tDDBwpCommonChannelBw" type="integer" minOccurs="0"/>

<element name="fDDBwpDedicatedCarrierFreqDL" type="integer" minOccurs="0"/>

<element name="fDDBwpDedicatedCarrierFreqUL" type="integer" minOccurs="0"/>

<element name="fDDBwpDedicatedChannelBwDL" type="integer" minOccurs="0"/>

<element name="fDDBwpDedicatedChannelBwUL" type="integer" minOccurs="0"/>

<element name="tDDBwpDedicatedCarrierFreq" type="integer" minOccurs="0"/>

<element name="tDDBwpDedicatedChannelBw" type="integer" minOccurs="0"/>

<element name="rRMPolicy" type="string"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

<element name="nRSectorCarrier" type="xn:dn" minOccurs="0"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRSectorCarrier">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="configuredMaxTxPower" type="integer"/>

<element name="pointAArfcnDLFDD" type="integer" minOccurs="0"/>

<element name="pointAArfcnULFDD" type="integer" minOccurs="0"/>

<element name="pointAArfcnTDD" type="integer" minOccurs="0"/>

<element name="fDDBwpTxStartDL" type="integer" minOccurs="0"/>

<element name="fDDBwpTxStartUL" type="integer" minOccurs="0"/>

<element name="fDDBwpTxBwDL" type="integer" minOccurs="0"/>

<element name="fDDBwpTxBwUL" type="integer" minOccurs="0"/>

<element name="tDDBwpTxStart" type="integer" minOccurs="0"/>

<element name="tDDBwpTxBw" type="integer" minOccurs="0"/>

<element name="nrBePtPhi" type="integer" minOccurs="0"/>

<element name="nrBePtTheta" type="integer" minOccurs="0"/>

<element name="nrBeWPhi" type="integer" minOccurs="0"/>

<element name="nrBeWTheta" type="integer" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

<element name="sectorEquipmentFunction" type="xn:dn" minOccurs="0"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_E1">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_XnC">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_XnU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NgC">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NgU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_F1C">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_F1U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_S1U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_X2C">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_X2U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

</schema>

End of modified section

5th modified section

## D.3.2	Attributes mapping

Mapping from the attributes of IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| Object Attributes | JSON definitions' Name | JSON Type |
| --- | --- | --- |
| NRCellDU.administrativeState | administrativeState | AdministrativeState |
| configuredMaxTxPower | configuredMaxTxPower | ConfiguredMaxTxPower |
| fDDCenterFrequencyUL | fddCenterFrequencyUL | FddCenterFrequencyUL |
| fDDCenterFrequencyDL | fddCenterFrequencyDL | FddCenterFrequencyDL |
| fDDBandwidthUL | fddBandwidthUL | FddBandwidthUL |
| fDDBandwidthDL | fddBandwidthDL | FddBandwidthDL |
| localAddress | localAddress | IpEndPoint |
| remoteAddress | remoteAddress | IpEndPoint |
| gNBId | gnbId | GnbId |
| gNBDUId | gnbDuId | GnbDUId |
| gNBCUName | gnbCuName | gNBName |
| gNBDUName | gnbDuName | gNBName |
| nCGI | nCgi | NCgi |
| nRPCI | nrPci | NrPCI |
| nRTac | nrTac | NrTac |
| nSSAI | nssai | Nssai |
| pointAArfcnULFDD | pointAArfcnULFDD | PointAArfcnULFDD |
| pointAArfcnDLFDD | pointAArfcnDLFDD | PointAArfcnDLFDD |
| pointAArfcnTDD | pointAArfcnTDD | PointAArfcnTDD |
| rRMPolicy | rrmPolicy | RrmPolicy |
| nrBePtPhi | nrBePtPhi | NrBePtPhi |
| nrBePtTheta | nrBePtTheta | NrBePtTheta |
| nrBeWPhi | nrBeWPhi | NrBeWPhi |
| nrBeWTheta | nrBeWTheta | NrBeWTheta |

End of modified section

6th modified section

## D.4.3	JSON schema "nrNrm.json"

{

"$schema": "http://json-schema.org/draft-05/schema#"

"id": "http://3gpp.org/28541/nrNrm.json",

"description": "JSON based solution set definitions for NR NRM",

"_referenced_schema":"http://3gpp.org/28623/genericNrm.json",

"_referenced_schema":"http://3gpp.org/28626/stateManagementNrm.json",

"_referenced_schema":"http://3gpp.org/28659/eutranNrm.json",

"_referenced_schema":"http://3gpp.org/28541/ngcNrm.json",

"definitions": {

"gnbDuFunction": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#definitions/vnfParametersListType"

},

"peeParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#definitions/peeParametersListType"

},

"gnbDuId": {

"$ref": "#/definitions/GnbDuId"

},

"gnbDuName": {

"$ref": "#/definitions/GnbName"

},

"gnbId": {

"$ref": "#/definitions/GnbId"

}

},

"required": [

"userLabel",

"gnbDuId",

"gnbId"

]

}

"gnbCuCpFunction": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#definitions/vnfParametersListType"

},

"peeParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#definitions/peeParametersListType"

"gnbId": {

"$ref": "#/definitions/GnbId"

},

"gnbCuName": {

"$ref": "#/definitions/GnbName"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28659/eutranNrm.json#definitions/PlmnIdList"

}

},

"required": [

"userLabel",

"gnbId",

"plmnIdList"

]

}

"gnbCuUpFunction": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#definitions/vnfParametersListType"

},

"peeParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#definitions/peeParametersListType"

"gnbId": {

"$ref": "#/definitions/GnbId"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28659/eutranNrm.json#definitions/PlmnIdList"

}

},

"required": [

"userLabel",

"gnbId",

"plmnIdList"

]

}

"nrCellCu": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#definitions/vnfParametersListType"

},

"nCgi": {

"$ref": "#/definitions/NCgi"

}

"plmnIdList": {

"$ref": "http://3gpp.org/28659/eutranNrm.json#definitions/PlmnIdList"

}

"nssai": {

"$ref": "#/definitions/Nssai"

}

"rrmPolicy": {

"$ref": "#/definitions/RrmPolicy"

}

},

"required": [

"userLabel",

"nCGI",

"plmnIdList"

]

}

"nrCellDu": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#definitions/vnfParametersListType"

},

"administrativeState": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#definitions/AdministrativeState"

},

"operationalState": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#definitions/OperationalState"

},

"availabilityStatus": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#definitions/AvailabilityStatus"

},

"cellState": {

"$ref": "#/definitions/CellState"

},

"nCgi": {

"$ref": "#/definitions/NCgi"

}

"plmnIdList": {

"$ref": "http://3gpp.org/28659/eutranNrm.json#definitions/PlmnIdList"

}

"nssai": {

"$ref": "#/definitions/Nssai"

}

"nrPci": {

"$ref": "#/definitions/NrPci"

}

"nrTac": {

"$ref": "#/definitions/NrTac"

}

"bSChannelBw": {

"type": "integer"

},

"fDDCarrierDL": {

"type": "integer"

},

"fDDCarrierUL": {

"type": "integer"

},

"cyclicPrefixDL": {

"$ref": "#/definitions/CyclicPrefix"

},

"cyclicPrefixUL": {

"$ref": "#/definitions/CyclicPrefix"

},

"fDDBwpCommonCarrierFreqDL": {

"type": "integer"

},

"fDDBwpCommonCarrierFreqUL": {

"type": "integer"

},

"fDDBwpCommonChannelBwDL": {

"type": "integer"

},

"fDDBwpCommonChannelBwUL": {

"type": "integer"

},

"tDDBwpCommonCarrierFreq": {

"type": "integer"

},

"tDDBwpCommonChannelBw": {

"type": "integer"

},

"fDDBwpDedicatedCarrierFreqDL": {

"type": "integer"

},

"fDDBwpDedicatedCarrierFreqUL": {

"type": "integer"

},

"fDDBwpDedicatedChannelBwDL": {

"type": "integer"

},

"fDDBwpDedicatedChannelBwUL": {

"type": "integer"

},

"tDDBwpDedicatedCarrierFreq": {

"type": "integer"

},

"tDDBwpDedicatedChannelBw": {

"type": "integer"

},

"rRMPolicy": {

"type": "string"

}

},

"required": [

"userLabel",

"nCgi",

"plmnIdList"

"nrPci",

"nrTac"

"bSChannelBw",

"fDDCarrierDL",

"fDDCarrierUL",

"tDDCarrier",

"subcarrierSpacingDL",

"subcarrierSpacingUL",

"cyclicPrefixDL",

"cyclicPrefixUL",

]

}

"nrSectorCarrier": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#definitions/vnfParametersListType"

},

"configuredMaxTxPower": {

"type": "integer"

},

"pointAArfcnDLFDD": {

"type": "integer"

},

"pointAArfcnULFDD": {

"type": "integer"

},

"pointAArfcnTDD": {

"type": "integer"

},

"fDDBwpTxStartDL": {

"type": "integer"

},

"fDDBwpTxStartUL": {

"type": "integer"

},

"fDDBwpTxBwDL": {

"type": "integer"

},

"fDDBwpTxBwUL": {

"type": "integer"

},

"tDDBwpTxStart": {

"type": "integer"

},

"tDDBwpTxBw": {

"type": "integer"

},

"nrBePtPhi": {

"type": "integer"

},

"nrBePtTheta": {

"type": "integer"

},

"nrBeWPhi": {

"type": "integer"

},

"nrBeWTheta": {

"type": "integer"

},

},

"required": [

"userLabel",

"configuredMaxTxPower"

]

}

"eP_E1": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint "

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint "

}

},

}

"eP_XnC": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint "

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint "

}

},

}

"eP_XnU": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint "

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint "

}

},

}

"eP_NgC": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint "

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint "

}

},

}

"eP_NgU": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint "

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint "

}

},

}

"eP_F1C": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint "

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint "

}

},

}

"eP_F1U": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint "

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint "

}

},

}

"eP_S1U": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint "

}

},

}

"eP_X2C": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint "

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint "

}

},

}

"eP_X2U": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint "

}

},

}

"GnbId": {

"type": "string",

"description": "gNBId identifies a gNB within a PLMN."

}

"GnbName": {

"type": "string",

"maxLength": "150"

}

"GnbDuId": {

"type": "number",

"description": "GnbDuId represents the gNB-DU ID defined in TS 38.473.",

"minimum": 0,

"maximum": 68719476735

}

"NCgi": {

"type": "object",

"description": "NCgi uniquely identifies a NR cell globally.",

"properties": {

"plmnId": {

"$ref": "http://3gpp.org/28659/eutranNrm.json#definitions/PlmnId"

},

"nCI": {

"$ref": "#/definitions/NrCellId"

}

}

},

"Nssai": {

"type": "array",

"item": {

"$ref": "#/definitions/Snssai"

}

},

"RrmPolicy": {

"type": "string",

"description": "RrmPolicy represents RRM policy which includes guidance for split of radio resources between multiple slices the cell supports. The RRM policy is implementation dependent.",

}

"IpEndPoint": {

"type": "object",

"properties": {

"ipv4Address": {

"type": "string"

},

"ipv6Address": {

"type": "string"

},

"ipv6Prefix": {

"type": "string"

},

"transport": {

"type": "string"

},

"port": {

"type": "integer"

}

"vlanId": {

"type": "integer"

}

}

},

"NrPci": {

"type": "integer",

"maximum": "503"

},

"NrTac": {

"type": "integer",

"maximum": "16777215"

},

"NrCellId": {

"type": "integer",

"maximum": "68719476735"

},

"Snssai": {

"type": "object",

"properties": {

"sst": {

"$ref": "http://3gpp.org/28541/ngcNrm.json#definitions/Sst"

},

"sd": {

"type": "string"

}

}

},

}

}

End of modified section

7th modified section

## E.4.2	Graphical representation

module: ngran

+--rw ManagedElement

+--ro objectClass               string

+--ro objectInstance            nrm-type:t_dn

+--ro id                        nrm-type:t_dn

+--ro vendorName                string

+--rw userDefinedState          string

+--ro swVersion                 string

+--ro dnPrefix                  string

+--rw userLabel                 string

+--ro locationName              string

+--ro managedBy*                nrm-type:t_dn

+--ro managedElementTypeList*   string

+--rw GNBDUFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw gNBDUId              uint32

|  +--rw gNBDUName           string

|  +--ro gNBId                uint32

|  +--rw EP_F1C

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_F1U

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw NRCellDU* [k_nCGI]

|  |  +--rw k_nCGI                          string

|  |  +--ro objectClass                     string

|  |  +--ro objectInstance                  nrm-type:t_dn

|  |  +--ro id                              nrm-type:t_dn

|  |  +--rw userLabel                      string

|  |  +--rw vnfParametersList

|  |  |  +--rw vnfInstanceId    string

|  |  |  +--rw vnfdId          string

|  |  |  +--rw flavourId       string

|  |  |  +--rw autoScalable    boolean

|  |  +--rw peeParametersList

|  |  |  +--rw siteIdentification    string

|  |  |  +--rw siteLatitude         decimal64

|  |  |  +--rw siteLongitude        decimal64

|  |  |  +--rw siteDescription       string

|  |  |  +--rw equipmentType         enumeration

|  |  |  +--rw environmentType       enumeration

|  |  |  +--rw powerInterface        enumeration

|  |  +--rw nCGI

|  |  |  +--rw MCC   t_mcc

|  |  |  +--rw MNC   t_mnc

|  |  |  +--rw nCI   t_NCI

|  |  +--rw pLMNId* [MCC]

|  |  |  +--rw MCC    t_mcc

|  |  |  +--rw MNC   t_mnc

|  |  +--rw s-NSSAI*                        nrm-type:t_s-NSSAI

|  |  +--ro operationalState               nrm-type:t_operationalState

|  |  +--rw administrativeState            nrm-type:t_administrativeState

|  |  +--ro availabilityStatus             nrm-type:t_availabilityStatus

|  |  +--ro cellState                       nrm-type:t_cellState

|  |  +--rw nRPCI                           nrm-type:t_nRPCI

|  |  +--rw nRTAC                           nrm-type:t_tAC

|  |  +--rw bSChannelBw                     uint16

|  |  +--rw fDDCarrierDL                    uint16

|  |  +--rw fDDCarrierUL                    uint16

|  |  +--rw tDDCarrier                      uint16

|  |  +--rw subcarrierSpacingDL             nrm-type:t_subcarrierSpacing

|  |  +--rw subcarrierSpacingUL             nrm-type:t_subcarrierSpacing

|  |  +--rw cyclicPrefixDL                  nrm-type:t_cyclicPrefix

|  |  +--rw cyclicPrefixUL                  nrm-type:t_cyclicPrefix

|  |  +--rw fDDBwpCommonCarrierFreqDL      uint16

|  |  +--rw fDDBwpCommonCarrierFreqUL      uint16

|  |  +--rw fDDBwpCommonChannelBwDL        uint16

|  |  +--rw fDDBwpCommonChannelBwUL        uint16

|  |  +--rw tDDBwpCommonCarrierFreq        uint16

|  |  +--rw tDDBwpCommonChannelBw          uint16

|  |  +--rw fDDBwpDedicatedCarrierFreqDL   uint16

|  |  +--rw fDDBwpDedicatedCarrierFreqUL   uint16

|  |  +--rw fDDBwpDedicatedChannelBwDL     uint16

|  |  +--rw fDDBwpDedicatedChannelBwUL     uint16

|  |  +--rw tDDBwpDedicatedCarrierFreq     uint16

|  |  +--rw tDDBwpDedicatedChannelBw       uint16

|  |  +--rw NRSectorCarrier*                nrm-type:t_dn

|  +--rw NRSectorCarrier* [sectorEquipmentFunction]

|     +--ro configuredMaxTxPower       uint16

|     +--ro pointAArfcnDLFDD          uint32

|     +--ro pointAArfcnULFDD          uint32

|     +--ro pointAArfcnTDD            uint32

|     +--ro fDDBwpTxStartDL           uint16

|     +--ro fDDBwpTxStartUL           uint16

|     +--ro fDDBwpTxBwDL              uint16

|     +--ro fDDBwpTxBwUL              uint16

|     +--ro tDDBwpTxStart             uint16

|     +--ro tDDBwpTxBw                uint16

|     +--ro nrBePtPhi		         uint16

|     +--ro nrBePtTheta		         uint16

|     +--ro nrBeWPhi			         uint16

|     +--ro nrBeWTheta	             uint16

|     +--rw sectorEquipmentFunction    nrm-type:t_dn

+--rw GNBCUCPFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw gNBCUName           string

|  +--ro gNBId                uint32

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--rw (gNB-type)

|  |  +--:(gNB)

|  |  |  +--rw EP-NgC

|  |  |  |  +--ro objectClass       string

|  |  |  |  +--ro objectInstance    nrm-type:t_dn

|  |  |  |  +--rw id                nrm-type:t_dn

|  |  |  |  +--rw userLabel        string

|  |  |  |  +--rw farEndEntity     nrm-type:t_dn

|  |  |  |  +--rw localAddress

|  |  |  |  |  +--rw ip_address   inet:ip-address

|  |  |  |  |  +--rw vlan_id      uint16

|  |  |  |  +--rw remoteAddress    inet:ip-address

|  |  |  +--rw EP-XnC

|  |  |     +--ro objectClass       string

|  |  |     +--ro objectInstance    nrm-type:t_dn

|  |  |     +--rw id                nrm-type:t_dn

|  |  |     +--rw userLabel        string

|  |  |     +--rw farEndEntity     nrm-type:t_dn

|  |  |     +--rw localAddress

|  |  |     |  +--rw ip_address   inet:ip-address

|  |  |     |  +--rw vlan_id      uint16

|  |  |     +--rw remoteAddress    inet:ip-address

|  |  +--:(en-gNB)

|  |     +--rw EP-X2C

|  |        +--ro objectClass       string

|  |        +--ro objectInstance    nrm-type:t_dn

|  |        +--rw id                nrm-type:t_dn

|  |        +--rw userLabel        string

|  |        +--rw farEndEntity     nrm-type:t_dn

|  |        +--rw localAddress

|  |        |  +--rw ip_address   inet:ip-address

|  |        |  +--rw vlan_id      uint16

|  |        +--rw remoteAddress    inet:ip-address

|  +--rw EP_F1C

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_E1

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw NRCellCU* [k_nCGI]

|     +--rw k_nCGI               string

|     +--ro objectClass          string

|     +--ro objectInstance       nrm-type:t_dn

|     +--ro id                   nrm-type:t_dn

|     +--rw userLabel           string

|     +--rw vnfParametersList

|     |  +--rw vnfInstanceId    string

|     |  +--rw vnfdId          string

|     |  +--rw flavourId       string

|     |  +--rw autoScalable    boolean

|     +--rw peeParametersList

|     |  +--rw siteIdentification    string

|     |  +--rw siteLatitude         decimal64

|     |  +--rw siteLongitude        decimal64

|     |  +--rw siteDescription       string

|     |  +--rw equipmentType         enumeration

|     |  +--rw environmentType       enumeration

|     |  +--rw powerInterface        enumeration

|     +--rw nCGI

|     |  +--rw MCC   t_mcc

|     |  +--rw MNC   t_mnc

|     |  +--rw nCI   t_NCI

|     +--rw pLMNId* [MCC]

|     |  +--rw MCC    t_mcc

|     |  +--rw MNC   t_mnc

|     +--rw s-NSSAI*             nrm-type:t_s-NSSAI

|     +--rw rRMPolicy           string

+--rw GNBCUUPFunction

+--ro objectClass          string

+--ro objectInstance       nrm-type:t_dn

+--ro id                   nrm-type:t_dn

+--rw userLabel           string

+--rw vnfParametersList

|  +--rw vnfInstanceId    string

|  +--rw vnfdId          string

|  +--rw flavourId       string

|  +--rw autoScalable    boolean

+--rw peeParametersList

|  +--rw siteIdentification    string

|  +--rw siteLatitude         decimal64

|  +--rw siteLongitude        decimal64

|  +--rw siteDescription       string

|  +--rw equipmentType         enumeration

|  +--rw environmentType       enumeration

|  +--rw powerInterface        enumeration

+--ro gNBId                uint32

+--rw pLMNId* [MCC MNC]

|  +--rw MCC    t_mcc

|  +--rw MNC    t_mnc

+--rw EP_F1U

|  +--ro objectClass       string

|  +--ro objectInstance    nrm-type:t_dn

|  +--rw id                nrm-type:t_dn

|  +--rw userLabel        string

|  +--rw farEndEntity     nrm-type:t_dn

|  +--rw localAddress

|  |  +--rw ip_address   inet:ip-address

|  |  +--rw vlan_id      uint16

|  +--rw remoteAddress    inet:ip-address

+--rw EP_E1

|  +--ro objectClass       string

|  +--ro objectInstance    nrm-type:t_dn

|  +--rw id                nrm-type:t_dn

|  +--rw userLabel        string

|  +--rw farEndEntity     nrm-type:t_dn

|  +--rw localAddress

|  |  +--rw ip_address   inet:ip-address

|  |  +--rw vlan_id      uint16

|  +--rw remoteAddress    inet:ip-address

+--rw (gNB-type)

+--:(gNB)

|  +--rw EP_NgU

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_XnU

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

+--:(en-gNB)

+--rw EP_X2U

|  +--ro objectClass       string

|  +--ro objectInstance    nrm-type:t_dn

|  +--rw id                nrm-type:t_dn

|  +--rw userLabel        string

|  +--rw farEndEntity     nrm-type:t_dn

|  +--rw localAddress

|  |  +--rw ip_address   inet:ip-address

|  |  +--rw vlan_id      uint16

|  +--rw remoteAddress    inet:ip-address

+--rw EP_S1U

+--ro objectClass       string

+--ro objectInstance    nrm-type:t_dn

+--rw id                nrm-type:t_dn

+--rw userLabel        string

+--rw farEndEntity     nrm-type:t_dn

+--rw localAddress

|  +--rw ip_address   inet:ip-address

|  +--rw vlan_id      uint16

+--rw remoteAddress    inet:ip-address

End of modified section

8th modified section

### E.4.3.9	NGRAN submodule, "ngran-nRSectorCarrier.yang"

submodule ngran-nRSectorCarrier {

belongs-to ngran { prefix ngran; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

description "NRSectorCarrier";

revision 2018-08-02 {

description "Initial revision";

}

grouping NRSectorCarrier {

description "";

leaf configuredMaxTxPower {

mandatory "true";

config "false";

description "";

type uint16;

}

leaf pointAArfcnDLFDD {

config "false";

description "CM";

type uint32;

}

leaf pointAArfcnULFDD {

config "false";

description "CM";

type uint32;

}

leaf pointAArfcnTDD {

config "false";

description "CM";

type uint32;

}

leaf fDDBwpTxStartDL {

config "false";

description "CM";

type uint16;

}

leaf fDDBwpTxStartUL {

config "false";

description "CM";

type uint16;

}

leaf fDDBwpTxBwDL {

config "false";

description "CM";

type uint16;

}

leaf fDDBwpTxBwUL {

config "false";

description "CM";

type uint16;

}

leaf tDDBwpTxStart {

config "false";

description "CM";

type uint16;

}

leaf tDDBwpTxBw {

config "false";

description "CM";

type uint16;

}

leaf nrBePtPhi {

config "false";

description "CM";

type uint16;

}

leaf nrBePtTheta {

config "false";

description "CM";

type uint16;

}

leaf nrBeWPhi {

config "false";

description "CM";

type uint16;

}

leaf nrBeWTheta {

config "false";

description "CM";

type uint16;

}

leaf sectorEquipmentFunction {

mandatory "true";

config "true";

description "fqBand for NR should be defined";

reference "23.622";

type nrm-type:t_dn;

}

}

}

End of modified section