3GPP TSG-SA5 Meeting #122	S5-187396

# Spokane,US, 12-16 November 2018 												Revision of S5-187099

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0683 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction Data Volume Reporting for Option 4&7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2018-11-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA2 has agreed CR#0661rev1 to TS 23.501 (LS S2-1811547), introducing the support for secondary RAT reporting for Option 4/7 in 5G Core, which was missing to specify the corresponding charging mechanism. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the secondary RAT reporting to CHF for Option 4/7 in CHF CDR for PDU session |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing essential functionality for Option 4/7 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 32.255 CR#0013 |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.298 CR#0021 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.2.5.2	CHF CDRs

This subclause contains the abstract syntax definitions that are specific to the CHF CDR types defined in this document.

.$CHFChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) chfChargingDataTypes (15) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallDuration,

CauseForRecClosing,

DataVolumeOctets,

Diagnostics,

IPAddress,

LocalSequenceNumber,

ManagementExtensions,

MSTimeZone,

RecordType,

ServiceSpecificInfo,

SubscriberEquipmentType,

SubscriptionID,

ThreeGPPPSDataOffStatus,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

ChargingCharacteristics,

ChargingID,

ChargingRuleBaseName,

ChChSelectionMode,

EventBasedChargingInformation,

PresenceReportingAreaInfo,

PLMN-Id,

QoSInformation,

RatingGroupId,

RATType,

ServiceIdentifier,

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version1 (0)}

InvolvedParty,

Session-Id

FROM IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version1 (0)}

;

--

--  CHF RECORDS

--

CHFRecord	::= CHOICE

--

-- Record values 200..201 are specific

--

{

chargingFunctionRecord			[200] ChargingRecord,

}

ChargingRecord 	::= SET

{

recordType						[0] RecordType,

recordingNetworkFunctionID		[1] NetworkFunctionName,

subscriberIdentifier			[2] SubscriptionID OPTIONAL,

networkFunctionInformation		[3] NetworkFunctionInformation,

triggers						[4] SEQUENCE OF Trigger OPTIONAL,

listOfMultipleUnitUsage			[5] SEQUENCE OF MultipleUnitUsage OPTIONAL,

recordOpeningTime				[6] TimeStamp,

duration						[7] CallDuration,

recordSequenceNumber			[8] INTEGER OPTIONAL,

causeForRecClosing				[9] CauseForRecClosing,

diagnostics						[10] Diagnostics OPTIONAL,

localRecordSequenceNumber		[11] INTEGER OPTIONAL,

recordExtensions				[12] ManagementExtensions OPTIONAL,

pDUSessionChargingInformation	[13] PDUSessionChargingInformation OPTIONAL,

roamingQBCInformation			[14] RoamingQBCInformation OPTIONAL

}

--

-- PDU Session Charging Information

--

PDUSessionChargingInformation 	::= SET

{

pDUSessionChargingID			[0] ChargingID,

userIdentifier					[1] InvolvedParty OPTIONAL,

userEquipmentInfo				[2] SubscriberEquipmentType OPTIONAL,

userLocationInformation			[3] OCTET STRING OPTIONAL,

userRoamerInOut					[4] RoamerInOut OPTIONAL,

presenceReportingAreaInfo		[5]	PresenceReportingAreaInfo OPTIONAL,

pDUSessionId					[6] Session-Id,

networkSliceInstanceID			[7] NetworkSliceInstanceID OPTIONAL,

pDUType							[8] PDUSessionType OPTIONAL,

sSCMode							[9] SSCMode OPTIONAL,

sUPIPLMNIIdetifier				[10] PLMN-ID OPTIONAL,

servingNetworkFunctionID		[11] SEQUENCE OF NetworkFunctionID OPTIONAL,

rATType							[12] RATType OPTIONAL,

dataNetworkNameIdentifier		[13] DataNetworkNameIdentifier OPTIONAL,

pDUAddress						[14] IPAddress OPTIONAL,

QoSInformation					[15] QoSInformation OPTIONAL,

uETimeZone 						[16] MSTimeZone OPTIONAL,

pDUSessionstartTime				[17] TimeStamp OPTIONAL,

pDUSessionstopTime				[18] TimeStamp OPTIONAL,

diagnostics						[19] Diagnostics OPTIONAL,

chargingCharacteristics			[20] ChargingCharacteristics,

chChSelectionMode				[21] ChChSelectionMode OPTIONAL,

threeGPPPSDataOffStatus			[22] ThreeGPPPSDataOffStatus OPTIONAL,

rANSecondaryRATUsageReport 	[23] SEQUENCE OF NGRANSecondaryRATUsageReport OPTIONAL

}

--

-- Roaming QBC Information

--

RoamingQBCInformation 	::= SET

{

multipleQFIcontainer			[0] SEQUENCE OF MultipleQFIContainer OPTIONAL,

uPFID							[1]	NetworkFunctionName OPTIONAL,

roamingChargingProfiles			[2] RoamingChargingProfile OPTIONAL,

}

--

-- PDU Container Information

--

PDUContainerInformation 		::= SEQUENCE

{

chargingRuleBaseName				[0] ChargingRuleBaseName OPTIONAL,

aFCorrelationInformation			[1] OCTET STRING OPTIONAL,

timeOfFirstUsage					[2] TimeStamp OPTIONAL,

timeOfLastUsage						[3] TimeStamp OPTIONAL,

qoSInformation						[4] QoSInformation OPTIONAL,

userLocationInformation				[5] OCTET STRING OPTIONAL,

presenceReportingAreaInfo			[6] PresenceReportingAreaInfo OPTIONAL,

rATType								[7] RATType OPTIONAL,

sponsorIdentity						[8] OCTET STRING OPTIONAL,

applicationServiceProviderIdentity	[9] OCTET STRING OPTIONAL

}

--

-- QFI Container Information

--

MultipleQFIContainer 		::= SEQUENCE

{

qosFlowId							[0] QoSFlowId OPTIONAL

triggers							[1] SEQUENCE OF Trigger,

triggerTimeStamp					[2] TimeStamp OPTIONAL,

dataTotalVolume						[3] DataVolumeOctets OPTIONAL,

dataVolumeUplink					[4] DataVolumeOctets OPTIONAL,

dataVolumeDownlink					[5] DataVolumeOctets OPTIONAL,

localSequenceNumber					[6]	INTEGER OPTIONAL,

timeOfFistUsage						[8] TimeStamp OPTIONAL,

timeOfLastUsage						[9] TimeStamp OPTIONAL,

qoSInformation						[10] QoSInformation OPTIONAL,

userLocationInformation				[11] OCTET STRING OPTIONAL,

uETimeZone	 						[12] MSTimeZone OPTIONAL,

presenceReportingAreaInfo			[13] PresenceReportingAreaInfo OPTIONAL,

rATType								[14] RATType OPTIONAL,

reportTime							[15] TimeStamp,

servingNetworkFunctionID			[16] SEQUENCE OF NetworkFunctionID OPTIONAL,

threeGPPPSDataOffStatus				[17] ThreeGPPPSDataOffStatus OPTIONAL

}

--

--  CHF CHARGING TYPES

--

AMFID	::= OCTET STRING (SIZE(6))

-- See subclause 2.10.1 of 3GPP TS 23.003 [7] for encoding.

DataNetworkNameIdentifier	::= IA5String (SIZE(1..63))

--

-- Network Identifier part of DNN in dot representation.

-- For example, if the complete DNN is 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs'

-- The Identifier is 'apn1a.apn1b.apn1c' and is presented in this form in the CDR.

--

MultipleUnitUsage 		::= SEQUENCE

{

ratingGroup							[0] RatingGroupId,

usedUnitContainer					[1] UsedUnitContainer OPTIONAL,

uPFID								[2]	NetworkFunctionName OPTIONAL

}

NGRANSecondaryRATType	::= OCTET STRING

--

-- "NR" or "EUTRA"

--

NGRANSecondaryRATUsageReport	::= SEQUENCE

{

nGRANSecondaryRATType			[0] NGRANSecondaryRATType OPTIONAL,

qosFlowsUsageReports			[1] SEQUENCE OF QosFlowsUsageReport OPTIONAL

}

QosFlowsUsageReport		::= SEQUENCE

{

qosFlowId						[0] QoSFlowId OPTIONAL,

startTime						[1] TimeStamp,

endTime							[2] TimeStamp,

dataVolumeDownlink				[3] DataVolumeOctets,

dataVolumeUplink				[4] DataVolumeOctets

}

NetworkFunctionInformation	::= SEQUENCE

{

networkFunctionality				[0]	NetworkFunctionality,

networkFunctionName					[1] NetworkFunctionName,

networkFunctionAddress				[2]	IPAddress,

networkFunctionPLMNIdentifier		[3] PLMN-Id OPTIONAL

}

NetworkFunctionName	::= IA5String (SIZE(1..20))

-- Shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 4122 [410]

NetworkFunctionality	::= ENUMERATED

{

cHF			(0),

sMF			(1)

}

NetworkSliceInstanceID	::= OCTET STRING (SIZE(1..4))

-- See S-NSSAI subclause 5.15.2 of 3GPP TS 23.501 [247] for encoding.

PartialRecordMethod	::= ENUMERATED

{

default			(0),

individual		(1)

}

PDUSessionType		::= ENUMERATED

{

iPv4v6			(0),

iPv4			(1),

iPv6			(2),

unstructured	(3),

ethernet		(4)

}

-- See 3GPP TS 29.571 [249] for details.

QoSFlowId		::= INTEGER

RatingIndicator	::= BOOLEAN

-- Included if the units have been rated.

RoamingChargingProfile 		::= SEQUENCE

{

roamingTriggers			[0] SEQUENCE OF RoamingTrigger OPTIONAL,

partialRecordMethod		[1] PartialRecordMethod OPTIONAL

}

RoamerInOut	::= ENUMERATED

{

roamerInBound		(0),

roamerOutBound		(1)

}

RoamingTrigger 		::= SEQUENCE

{

Trigger					[0] SMFTrigger OPTIONAL,

triggerCategory			[1] TriggerCategory	 OPTIONAL,

timeLimit				[2] CallDuration OPTIONAL,

volumeLimit				[3] DataVolumeGPRS OPTIONAL,

maxNbChargingConditions	[4] INTEGER OPTIONAL

}

SSCMode	::= INTEGER

{

sSCMode1				(1),

sSCMode2				(2),

sSCMode3				(3),

}

-- See 3GPP TS 29.501 [248] for details.

SMFTrigger				::= INTEGER

{

startOfPDUSession							(1),

atartOfServiceDataFlowNoSession				(2),

-- Change of Charging conditions

qoSChange									(100),

userLocationChange							(101),

aMFChange									(102),

presenceReportingAreaChange					(103),

threeGPPPSDataOffStatusChange				(104),

tariffTimeChange							(105),

uETimeZoneChange							(106),

pLMNChange									(107),

rATTypeChange								(108),

sessionAMBRChange							(109),

additionOfUPF								(110),

removalOfUPF 								(111),

-- Limit per PDU session

pDUSessionExpiryDataTimeLimit				(200),

pDUSessionExpiryDataVolumeLimit				(201),

pDUSessionExpiryDataEventLimit				(202),

pDUSessionExpiryChargingConditionChanges	(203),

-- Limit per Rating group

ratingGroupDataTimeLimit					(300),

ratingGroupDataVolumeLimit					(301),

ratingGroupDataEventLimit					(302),

-- Quota management

timeThresholdReached						(400),

volumeThresholdReached						(401),

unitThresholdReached						(402),

timeQuotaExhausted							(403),

volumeQuotaExhausted						(404),

unitQuotaExhausted							(405),

expiryOfQuotaValidityTime					(406),

reAuthorizationRequest						(407),

startOfServiceDataFlowNoValidQuota			(408),

-- Others

terminationOfServiceDataFlow				(500),

managementIntervention						(501),

unusedQuotaTimerExpiry						(502),

endOfPDUSession								(503),

cHFResponseWithSessionTermination			(504),

cHFAbortRequest								(505),

-- Limit per QoS Flow

qoSFlowExpiryDataTimeLimit					(600),

qoSFlowExpiryDataVolumeLimit				(601)

}

-- See TS 32.255 [15] for details.

Trigger	::= CHOICE

{

sMFTrigger		[0] SMFTrigger

}

TriggerCategory	::= ENUMERATED

{

immediateReport		(0),

deferredReport		(1)

}

UsedUnitContainer 		::= SEQUENCE

{

serviceIdentifier					[0] ServiceIdentifier OPTIONAL,

time								[1] CallDuration OPTIONAL,

triggers							[2] SEQUENCE OF Trigger,

triggerTimeStamp					[3] TimeStamp OPTIONAL,

dataTotalVolume						[4] DataVolumeOctets OPTIONAL,

dataVolumeUplink					[5] DataVolumeOctets OPTIONAL,

dataVolumeDownlink					[6] DataVolumeOctets OPTIONAL,

serviceSpecificUnits				[7] INTEGER OPTIONAL,

eventTimeStamp						[8] TimeStamp OPTIONAL,

localSequenceNumber					[9]	INTEGER OPTIONAL,

RatingIndicator						[10] RatingIndicator OPTIONAL,

pDUContainerInformation				[11] PDUContainerInformation OPTIONAL

}

.#END

End of changes