3GPP TSG-SA5 Meeting #123	S5-191081

# Montreal,Canada, 21-25 January 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0042 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add MCS distribution measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2018/12/20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The performance of AMC affects the spectrum effectiveness.The MCS distribution can indicate the spectrum effectiveness. The measurement of MCS distribution can be used to assess the performance of AMC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definitions of measurement for PDSCH/PUSCH MCS distribution are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The assessment of AMC’s performance is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1.1.X (new), A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[3]	3GPP TS 32.404: "Performance Management (PM); Performance measurements - Definitions and template".

[4]	3GPP TS 23.501: "System Architecture for the 5G System".

[5]	IETF RFC 5136: "Defining Network Capacity".

[6]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

[7]	3GPP TS 23.502: "Procedures for the 5G System".

[8]	3GPP TS 28.554: “Management and orchestration; 5G end to end Key Performance Indicators (KPI)”.

[9]	3GPP TS 32.425: “Performance Management (PM); Performance measurements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN)”

[10]	3GPP TS 32.451: "Key Performance Indicators (KPI) for Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Requirements”

[x]	3GPP TS 38.214: "NR; Physical layer procedures for data”

Next change

5.1.1.X MCS Distribution Measurement

5.1.1.X.1	PDSCH MCS Index Distribution

a)  This measurement provides the PDSCH MCS RB distribution of the gNodeB scheduled.

b)  CC.

c)  This measurement is obtained by incrementing the number of PDSCH RB with MCS index to the appropriate measurement bin when gNodeB scheduling. In case there are more than one RIs (rank indicator), separate counters are maintained for the RIs.  In case multiple MCS index tables (Table 5.1.3.1-1, Table 5.1.3.1-2, Table 5.1.3.1-3 in clause 5.1.3.1, TS 38.214 [x]) are used, separate counters are maintained for the MCS index tables.  The used MCS index table is optional. The RBs used for broadcast should be excluded.

d)  A set of integers.

e)  CARR.PdschMcsIndexDist.RankX.TableY.BinZ

where X represents the rank (1~8 ), Y represents the MCS index table (1 to 3 for  PDSCH), Z represents the MCS index (0 to 28  for table 1, 3;  0 to 27  for table 2).

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

5.1.1.X.2 PUSCH MCS Index Distribution

a)  This measurement provides the PUSCH MCS RB distribution of the gNodeB scheduled.

b)  CC.

c)  This measurement is obtained by incrementing the number of PUSCH RB with MCS index to the appropriate measurement bin when gNodeB scheduling.  In case there are more than one RIs (rank indicator), separate counters are maintained for the rank.  In case multiple MCS index table (Table 5.1.3.1-1, Table 5.1.3.1-2, Table 5.1.3.1-3 in clause 5.1.3.1, TS 38.214 [x]) are used, separate counters are maintained for the MCS index tables.  The used MCS index table is optional.

d)  A set of integers.

e)  CARR.PuschMcsIndexDist.RankX.TableY.BinZ

where X represents the rank (1~4 ), Y represents the MCS index table (1 to 3 same to PDSCH, 4,5  refers to PUSCH table 1,2), Z represents the MCS index (0 to 28  for table 1,3;  0 to 27  for table 2,4,5).

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

Next modified section

A.x Monitoring PDSCH/PUSCH scheduled MCS RB distribution

MCS represents the modulation and coding schemes scheduled for the physical resources by NG-RAN which have an impacts on the transmission efficieny of the scheduled physical resources. Scheduled spectral efficiency comprehensively considers the MCS of physical resources scheduled by NG-RAN and the spatial multiplexing technologies used in the cell. The measurement of MCS distribution is a useful metric reflecting the efficiency for PDSCH and PUSCH RBs. The distribution of MCS combined with BLER, CQI (PMI, RI) can be used for evaluate AMC performance.  So it is helpful for operator to optimize the scheduling of physical resources to improve the network efficiency and overall service quality

End of modifications