3GPP TSG-SA5 Meeting #124 	S5-192059

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0043 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of dnn data type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-DCH |  |  |  |  |  | Date: |  |  | 2019-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Data Network Name (dnnId) is not referring to the associated common TS 29.571 data type. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Re-use dnn TS 29.571 data type for dnnid attribute |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency in the format for DNN between different interfaces |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1, 6.1.6.2.2.8, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

#### 6.1.6.1	General

This subclause specifies the application data model supported by the API.

The Nchf_ConvergedCharging Service API allows the NF consumer to consume the converged charging service from the CHF as defined in 3GPP TS 32.290 [58].

Table 6.1.6.1-1 specifies the data types defined for the ConvergedCharging service based interface protocol.

Table 6.1.6.1-1: Nchf_ ConvergedCharging specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ChargingDataRequest | 6.1.6.2.1.1 6.1.6.2.2.1 | Describes the attributes of Charging Data Request to CHF for initial, update and termination of the charging session. |  |
| ChargingDataResponse | 6.1.6.2.1.2 6.1.6.2.2.2 | Describes the attributes of Charging Data Response from CHF on charging session initial, update and termination. |  |
| ChargingNotifyRequest | 6.1.6.2.1.3 | Describes Notifications about events that occurred in request message. |  |

Table 6.1.6.1-2 specifies data types re-used by the Nchf_ConvergedCharging service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nchf_ConvergedCharging service based interface.

Table 6.1.6.1-2: Nchf_ConvergedCharging re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Supi | 3GPP TS 29.571 [371] | The identification of the user (i.e. IMSI, NAI). |  |
| Uint32 | 3GPP TS 29.571 [371] | Unsigned 32-bit integers |  |
| Uint64 | 3GPP TS 29.571 [371] | Unsigned 64-bit integers |  |
| PduSessionId | 3GPP TS 29.571 [371] | The identification of the PDU session. |  |
| PduSessionType | 3GPP TS 29.571 [371] | the type of a PDU session |  |
| Uri | 3GPP TS 29.571 [371] | String providing an URI |  |
| AccessType | 3GPP TS 29.571 [371] | The identification of the type of access network. |  |
| RatType | 3GPP TS 29.571 [371] | The identification of the RAT type. |  |
| Ipv4Addr | 3GPP TS 29.571 [371] | The Ipv4 address allocated for the user. |  |
| Ipv6Prefix | 3GPP TS 29.571 [371] | The Ipv6 prefix allocated for the user. |  |
| Pei | 3GPP TS 29.571 [371] | The Identification of a Permanent Equipment. |  |
| TimeZone | 3GPP TS 29.571 [371] | Time zone information |  |
| NfInstanceId | 3GPP TS 29.571 [371] | String uniquely identifying a NF instance. |  |
| Gpsi | 3GPP TS 29.571 [371] | String identifying a Gpsi |  |
| DefaultQosInformation | 3GPP TS 29.571 [371] | Identifies the information of the default QoS. |  |
| UserLocation | 3GPP TS 29.571 [371] | User location information |  |
| PlmnId | 3GPP TS 29.571 [371] | PLMN id |  |
| Guami | 3GPP TS 29.571 [371] | Globally Unique AMF Identifier |  |
| DurationSec | 3GPP TS 29.571 [371] | Identifies a period of time in units of seconds. |  |
| Snssai | 3GPP TS 29.571 [371] | SNSSAI |  |
| ProblemDetails | 3GPP TS 29.571 [371] | additional details of the error |  |
| SscMode | 3GPP TS 29.571 [371] | SSC Mode type |  |
| PresenceInfoPraInfo | 3GPP TS 29.571 [371]3GPP TS 29.512 [204] | PRA information including PRAId, PRA element list and PRA status |  |
| Qfi | 3GPP TS 29.571 [371] | QoS flow identifier designated as "Qfi". |  |
| Dnn | 3GPP TS 29.571 [371] | Data Network Name |  |

Next change

###### 6.1.6.2.2.8	Type PDUSessionInformation

Table  6.1.6.2.2.8-1: Definition of type PDUSessionInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| networkSlicingInfo | NetworkSlicingInfo | OM | 0..1 | information of network slice serving the PDU session |  |
| pduSessionID | PduSessionId | M | 1 |  |  |
| pduType | PduSessionType | OM | 0..1 | type of the PDU session |  |
| sscMode | SscMode | OC | 0..1 | information of SSC Mode type. |  |
| hPlmnId | PlmnId | OC | 0..1 | PLMN identifier of the home network |  |
| servingNetworkFunctionID | ServingNetworkFunctionID | Oc | 0..1 | This field holds serving Network Function identifier. |  |
| servingCNPlmnId | PlmnId | Oc | 0..1 | Serving Core Network Operator PLMN ID selected by the UE in shared networks. |  |
| ratType | RatType | OC | 0..1 | the RAT Type of the PDU session |  |
| dnnId | Dnn | M | 1 | a Data Network Name |  |
| chargingCharacteristics | string | OC | 0..1 | the Charging Characteristics for this PDU session. |  |
| chargingCharacteristicsSelectionMode | ChargingCharacteristicsSelectionMode | OC | 0..1 | information about how the "Charging Characteristics" was selected. |  |
| startTime | dateTime | OC | 0..1 | the time in UTC format which represents the start of a PDU session at the SMF |  |
| stopTime | dateTime | OC | 0..1 | the time in UTC format which represents the stop of a PDU session at the SMF |  |
| 3gppPSDataOffStatus | 3GPPPSDataOffStatus | OC | 0..1 | This field holds the 3GPP Data off Status when UE’s 3GPP Data Off status is Activated or Deactivated. |  |
| sessionStopIndicator | boolean | OC | 0..1 | This field indicates to the CHF that the PDU session has been terminated. |  |
| pduAddress | PDUAddress | OC | 0..1 | Group of user ip address |  |
| diagnostics | Diagnostics | OC | 0..1 | provides a more detailed cause value from SMF. |  |
| qoSInformation | DefaultQosInformation | OC | 0..1 | This field holds the authorized QoS applied to PDU session. |  |

Next change

## A.2	Nchf_ ConvergedCharging API

openapi: 3.0.0

info:

description: ConvergedCharging Service

version: 1.R15.0.0

title: Nchf_ConvergedCharging

externalDocs:

description: >-

3GPP TS 32.291 Telecommunication management; Charging management;

5G system, charging service; Stage 3.

url: 'http://www.3gpp.org/ftp/Specs/archive/32_series/32.291/'

servers:

- url: '{apiRoot}/nchf-convergedcharging/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 4.4 of 3GPP TS 29.501.

paths:

/chargingdata:

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataRequest'

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataResponse'

'400':

description: Bad request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'403':

description: Forbidden

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: Not Found

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'410':

$ref: 'TS29571_CommonData.yaml#/components/responses/410'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifyUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingNotifyRequest'

responses:

'204':

description: 'No Content, Notification was succesfull'

'400':

description: Bad request

content:

application/json:

schema:

$ref: >-

TS29571_CommonData.yaml#/components/schemas/ProblemDetails

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

'/chargingdata/{ChargingDataRef}/update':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataRequest'

parameters:

- name: ChargingDataRef

in: path

description: a unique identifier for a charging data resource in a PLMN

required: true

schema:

type: string

responses:

'200':

description: OK. Updated Charging Data resource is returned

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataResponse'

'400':

description: Bad request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'403':

description: Forbidden

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: Not Found

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'410':

$ref: 'TS29571_CommonData.yaml#/components/responses/410'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

'/chargingdata/{ChargingDataRef}/release':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataRequest'

parameters:

- name: ChargingDataRef

in: path

description: a unique identifier for a charging data resource in a PLMN

required: true

schema:

type: string

responses:

'204':

description: No Content.

'404':

description: Not Found

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'410':

$ref: 'TS29571_CommonData.yaml#/components/responses/410'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

schemas:

ChargingDataRequest:

type: object

properties:

subscriberIdentifier:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

nfConsumerIdentification:

$ref: '#/components/schemas/NFConsumerIdentification'

invocationTimeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

invocationSequenceNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

oneTimeEvent:

type: boolean

notifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

multipleUnitUsage:

type: array

items:

$ref: '#/components/schemas/MultipleUnitUsage'

minItems: 0

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

pDUSessionChargingInformation:

$ref: '#/components/schemas/PDUSessionChargingInformation'

roamingQBCInformation:

$ref: '#/components/schemas/RoamingQBCInformation'

sMSChargingInformation:

$ref: '#/components/schemas/SMSChargingInformation'

required:

- nfConsumerIdentification

- invocationTimeStamp

- invocationSequenceNumber

ChargingDataResponse:

type: object

properties:

invocationTimeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

invocationSequenceNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

invocationResult:

$ref: '#/components/schemas/InvocationResult'

sessionFailover:

$ref: '#/components/schemas/SessionFailover'

multipleUnitInformation:

type: array

items:

$ref: '#/components/schemas/MultipleUnitInformation'

minItems: 0

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

pDUSessionChargingInformation:

$ref: '#/components/schemas/PDUSessionChargingInformation'

roamingQBCInformation:

$ref: '#/components/schemas/RoamingQBCInformation'

required:

- invocationTimeStamp

- invocationSequenceNumber

ChargingNotifyRequest:

type: object

properties:

notificationType:

$ref: '#/components/schemas/NotificationType'

reauthorizationDetails:

type: array

items:

$ref: '#/components/schemas/ReauthorizationDetails'

minItems: 0

required:

- notificationType

NFConsumerIdentification:

type: object

properties:

nFName:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

nFIPv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

nFIPv6Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

nFPLMNID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

nodeFunctionality:

$ref: '#/components/schemas/NodeFunctionality'

required:

- nFName

- nFIPv4Address

- nFIPv6Address

- nodeFunctionality

MultipleUnitUsage:

type: object

properties:

ratingGroup:

$ref: '#/components/schemas/RatingGroup'

requestedUnit:

$ref: '#/components/schemas/RequestedUnit'

UsedUnitContainer:

type: array

items:

$ref: '#/components/schemas/UsedUnitContainer'

minItems: 0

uPFID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

required:

- ratingGroup

InvocationResult:

type: object

properties:

error:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

failureHandling:

$ref: '#/components/schemas/FailureHandling'

Trigger:

type: object

properties:

triggerType:

$ref: '#/components/schemas/TriggerType'

triggerCategory:

$ref: '#/components/schemas/TriggerCategory'

timeLimit:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

volumeLimit:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

maxNumberOfccc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

required:

- triggerType

- triggerCategory

MultipleUnitInformation:

type: object

properties:

resultCode:

$ref: '#/components/schemas/ResultCode'

ratingGroup:

$ref: '#/components/schemas/RatingGroup'

grantedUnit:

$ref: '#/components/schemas/GrantedUnit'

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

validityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

quotaHoldingTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

finalUnitIndication:

$ref: '#/components/schemas/FinalUnitIndication'

timeQuotaThreshold:

type: integer

volumeQuotaThreshold:

type: integer

unitQuotaThreshold:

type: integer

uPFID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

required:

- ratingGroup

RequestedUnit:

type: object

properties:

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

serviceSpecificUnits:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

UsedUnitContainer:

type: object

properties:

serviceId:

$ref: '#/components/schemas/ServiceId'

quotaManagementIndicator:

$ref: '#/components/schemas/QuotaManagementIndicator'

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

triggerTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

serviceSpecificUnits:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

eventTimeStamps:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

localSequenceNumber:

type: integer

pDUContainerInformation:

$ref: '#/components/schemas/PDUContainerInformation'

required:

- localSequenceNumber

GrantedUnit:

type: object

properties:

tariffTimeChange:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

serviceSpecificUnits:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

FinalUnitIndication:

type: object

properties:

finalUnitAction:

$ref: '#/components/schemas/FinalUnitAction'

restrictionFilterRule:

$ref: '#/components/schemas/IPFilterRule'

filterId:

type: string

redirectServer:

$ref: '#/components/schemas/RedirectServer'

required:

- finalUnitAction

RedirectServer:

type: object

properties:

redirectAddressType:

$ref: '#/components/schemas/RedirectAddressType'

redirectServerAddress:

type: string

required:

- redirectAddressType

- redirectServerAddress

ReauthorizationDetails:

type: object

properties:

serviceId:

$ref: '#/components/schemas/ServiceId'

ratingGroup:

$ref: '#/components/schemas/RatingGroup'

quotaManagementIndicator:

$ref: '#/components/schemas/QuotaManagementIndicator'

PDUSessionChargingInformation:

type: object

properties:

chargingId:

$ref: '#/components/schemas/ChargingId'

userInformation:

$ref: '#/components/schemas/UserInformation'

userLocationinfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

userLocationTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

presenceReportingAreaInformation:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 0

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

pduSessionInformation:

$ref: '#/components/schemas/PDUSessionInformation'

unitCountInactivityTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'
        rANSecondaryRATUsageReport:

$ref: '#/components/schemas/RANSecondaryRATUsageReport'

required:

- userInformation

- pduSessionInformation

UserInformation:

type: object

properties:

servedGPSI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

servedPEI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

unauthenticatedFlag:

type: boolean

roamerInOut:

$ref: '#/components/schemas/RoamerInOut'

required:

- servedGPSI

PDUSessionInformation:

type: object

properties:

networkSlicingInfo:

$ref: '#/components/schemas/NetworkSlicingInfo'

pduSessionID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

pduType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

sscMode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SscMode'

hPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

servingNetworkFunctionID:

$ref: '#/components/schemas/ServingNetworkFunctionID'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

dnnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

chargingCharacteristics:

type: string

chargingCharacteristicsSelectionMode:

$ref: '#/components/schemas/ChargingCharacteristicsSelectionMode'

startTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

stopTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

3gppPSDataOffStatus:

$ref: '#/components/schemas/3GPPPSDataOffStatus'

sessionStopIndicator:

type: boolean

pduAddress:

$ref: '#/components/schemas/PDUAddress'

diagnostics:

$ref: '#/components/schemas/Diagnostics'

qoSInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DefaultQosInformation'

servingCNPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

required:

- pduSessionID

- dnnId

PDUContainerInformation:

type: object

properties:

timeofFirstUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

timeofLastUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

qoSInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DefaultQosInformation'

aFCorrelationInformation:

type: string

userLocationInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

servingNodeID:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

minItems: 0

presenceReportingAreaInformation:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 0

3gppPSDataOffStatus:

$ref: '#/components/schemas/3GPPPSDataOffStatus'

sponsorIdentity:

type: string

applicationserviceProviderIdentity:

type: string

chargingRuleBaseName:

type: string

NetworkSlicingInfo:

type: object

properties:

sNSSAI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

required:

- sNSSAI

PDUAddress:

type: object

properties:

pduIPv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

pduIPv6Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

pduAddressprefixlength:

type: integer

IPv4dynamicAddressFlag:

type: boolean

IPv6dynamicAddressFlag:

type: boolean

ServingNetworkFunctionID:

type: object

properties:

servingNetworkFunctionName:

type: string

servingNetworkFunctionInstanceid:

type: string

gUAMI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

RoamingQBCInformation:

type: object

properties:

multipleQFIcontainer:

type: array

items:

$ref: '#/components/schemas/MultipleQFIcontainer'

minItems: 0

uPFID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

roamingChargingProfile:

$ref: '#/components/schemas/RoamingChargingProfile'

MultipleQFIcontainer:

type: object

properties:

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

triggerTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

localSequenceNumber:

type: integer

qFIContainerInformation:

$ref: '#/components/schemas/QFIContainerInformation'

required:

- localSequenceNumber

QFIContainerInformation:

type: object

properties:

qFI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

timeofFirstUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

timeofLastUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

qoSInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DefaultQosInformation'

userLocationInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

presenceReportingAreaInformation:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 0

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

servingNetworkFunctionID:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

minItems: 0

3gppPSDataOffStatus:

$ref: '#/components/schemas/3GPPPSDataOffStatus'

RoamingChargingProfile:

type: object

properties:

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

partialRecordMethod:

$ref: '#/components/schemas/PartialRecordMethod'

SMSChargingInformation:

type: object

properties:

originatorInfo:

$ref: '#/components/schemas/OriginatorInfo'

recipientInfo:

type: array

items:

$ref: '#/components/schemas/RecipientInfo'

minItems: 0

userEquipmentInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

userLocationinfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

sMSCAddress:

type: string

sMDataCodingScheme:

type: integer

sMMessageType:

$ref: '#/components/schemas/SMMessageType'

sMReplyPathRequested:

$ref: '#/components/schemas/ReplyPathRequested'

sMUserDataHeader:

type: string

sMStatus:

type: string

sMDischargeTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

numberofMessagesSent:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

sMServiceType:

$ref: '#/components/schemas/SMServiceType'

sMSequenceNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

sMSresult:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

submissionTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

sMPriority:

$ref: '#/components/schemas/SMPriority'

messageReference:

type: string

messageSize:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

messageClass:

$ref: '#/components/schemas/MessageClass'

deliveryReportRequested:

$ref: '#/components/schemas/DeliveryReportRequested'

OriginatorInfo:

type: object

properties:

originatorSUPI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

originatorGPSI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

originatorOtherAddress:

$ref: '#/components/schemas/SMAddressInfo'

originatorReceivedAddress:

$ref: '#/components/schemas/SMAddressInfo'

originatorSCCPAddress:

type: string

sMOriginatorInterface:

$ref: '#/components/schemas/SMInterface'

sMOriginatorProtocolId:

type: string

RecipientInfo:

type: object

properties:

recipientSUPI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

recipientGPSI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

recipientOtherAddress:

$ref: '#/components/schemas/SMAddressInfo'

recipientReceivedAddress:

$ref: '#/components/schemas/SMAddressInfo'

recipientSCCPAddress:

type: string

sMDestinationInterface:

$ref: '#/components/schemas/SMInterface'

sMrecipientProtocolId:

type: string

SMAddressInfo:

type: object

properties:

sMaddressType:

$ref: '#/components/schemas/SMAddressType'

sMaddressData:

type: string

sMaddressDomain:

$ref: '#/components/schemas/SMAddressDomain'

RecipientAddress:

type: object

properties:

recipientAddressInfo:

$ref: '#/components/schemas/SMAddressInfo'

sMaddresseeType:

$ref: '#/components/schemas/SMAddresseeType'

MessageClass:

type: object

properties:

classIdentifier:

$ref: '#/components/schemas/ClassIdentifier'

tokenText:

type: string

SMAddressDomain:

type: object

properties:

domainName:

type: string

3GPPIMSIMCCMNC:

type: string

SMInterface:

type: object

properties:

interfaceId:

type: string

interfaceText:

type: string

interfacePort:

type: string

interfaceType:

$ref: '#/components/schemas/InterfaceType'

RANSecondaryRATUsageReport:

type: object

properties:

rANSecondaryRATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

qosFlowsUsageReports:

type: array

items:

$ref: '#/components/schemas/QosFlowsUsageReport'

RatingGroup:

type: Uint32

ServiceId:

type: Uint32

Diagnostics:

type: integer

IPFilterRule:

type: string

ChargingId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

QosFlowsUsageReport:

type: object

properties:

qFI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

startTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

endTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

NotificationType:

anyOf:

- type: string

enum:

- REAUTHORIZATION

- ABORT_CHARGING

- type: string

NodeFunctionality:

anyOf:

- type: string

enum:

- SMF

- type: string

ChargingCharacteristicsSelectionMode:

anyOf:

- type: string

enum:

- HOME_DEFAULT

- ROAMING_DEFAULT

- VISITING_DEFAULT

- type: string

TriggerType:

anyOf:

- type: string

enum:

- QUOTA_THRESHOLD

- QHT

- FINAL

- QUOTA_EXHAUSTED

- VALIDITY_TIME

- OTHER_QUOTA_TYPE

- FORCED_REAUTHORISATION

- UNUSED_QUOTA_TIMER

- ABNORMAL_RELEASE

- QOS_CHANGE

- VOLUME_LIMIT

- TIME_LIMIT

- PLMN_CHANGE

- USER_LOCATION_CHANGE

- RAT_CHANGE

- UE_TIMEZONE_CHANGE

- TARIFF_TIME_CHANGE

- MAX_NUMBER_OF_CHANGES_IN CHARGING_CONDITIONS

- MANAGEMENT_INTERVENTION

- CHANGE_OF_UE_PRESENCE_IN PRESENCE_REPORTING_AREA

- CHANGE_OF_3GPP_PS_DATA_OFF_STATUS

- SERVING_NODE_CHANGE

- REMOVAL_OF_UPF

- ADDITION_OF_UPF

- type: string

FinalUnitAction:

anyOf:

- type: string

enum:

- TERMINATE

- REDIRECT

- RESTRICT_ACCESS

- type: string

RedirectAddressType:

anyOf:

- type: string

enum:

- IPV4

- IPV6

- URL

- type: string

TriggerCategory:

anyOf:

- type: string

enum:

- IMMEDIATE_REPORT

- DEFERRED_REPORT

- type: string

QuotaManagementIndicator:

anyOf:

- type: string

enum:

- ONLINE_CHARGING

- OFFLINE_CHARGING

- type: string

FailureHandling:

anyOf:

- type: string

enum:

- TERMINATE

- CONTINUE

- RETRY_AND_TERMINATE

- type: string

SessionFailover:

anyOf:

- type: string

enum:

- FAILOVER_NOT_SUPPORTED

- FAILOVER_SUPPORTED

- type: string

3GPPPSDataOffStatus:

anyOf:

- type: string

enum:

- ACTIVE

- INACTIVE

- type: string

ResultCode:

anyOf:

- type: string

enum:

- SUCCESS

- END_USER_SERVICE_DENIED

- QUOTA_MANAGEMENT _NOT_APPLICABLE

- QUOTA_LIMIT_REACHED

- END_USER_SERVICE_REJECTED

- USER_UNKNOWN

- RATING_FAILED

- type: string

PartialRecordMethod:

anyOf:

- type: string

enum:

- DEFAULT

- INDIVIDUAL

- type: string

RoamerInOut:

anyOf:

- type: string

enum:

- IN_BOUND

- OUT_BOUND

- type: string

SMMessageType:

anyOf:

- type: string

enum:

- SUBMISSION

- DELIVERY_REPORT

- SM_SERVICE_REQUEST

- type: string

SMPriority:

anyOf:

- type: string

enum:

- LOW

- NORMAL

- HIGH

- type: string

DeliveryReportRequested:

anyOf:

- type: string

enum:

- YES

- NO

- type: string

InterfaceType:

anyOf:

- type: string

enum:

- UNKNOWN

- MOBILE_ORIGINATING

- MOBILE_TERMINATING

- APPLICATION_ORIGINATING

- APPLICATION_TERMINATING

- type: string

ClassIdentifier:

anyOf:

- type: string

enum:

- PERSONAL

- ADVERTISEMENT

- INFORMATIONAL

- AUTO

- type: string

SMAddressType:

anyOf:

- type: string

enum:

- EMAIL_ADDRESS

- MSISDN

- IPV4_ADDRESS

- IPV6_ADDRESS

- NUMERIC_SHORTCODE

- ALPHANUMERIC_SHORTCODE

- OTHER

- IMSI

- type: string

SMAddresseeType:

anyOf:

- type: string

enum:

- TO

- CC

- BCC

- type: string

SMServiceType:

anyOf:

- type: string

enum:

- VAS4SMS_SHORT_MESSAGE_CONTENT_PROCESSING

- VAS4SMS_SHORT_MESSAGE_FORWARDING

- VAS4SMS_SHORT_MESSAGE_FORWARDING_MULTIPLE_SUBSCRIPTIONS

- VAS4SMS_SHORT_MESSAGE_FILTERING

- VAS4SMS_SHORT_MESSAGE_RECEIPT

- VAS4SMS_SHORT_MESSAGE_NETWORK_STORAGE

- VAS4SMS_SHORT_MESSAGE_TO_MULTIPLE_DESTINATIONS

- VAS4SMS_SHORT_MESSAGE_VIRTUAL_PRIVATE_NETWORK(VPN)

- VAS4SMS_SHORT_MESSAGE_AUTO_REPLY

- VAS4SMS_SHORT_MESSAGE_PERSONAL_SIGNATURE

- VAS4SMS_SHORT_MESSAGE_DEFERRED_DELIVERY

- VAS4SMS_SHORT_MESSAGE_CONTENT_PROCESSING

- type: string

ReplyPathRequested:

anyOf:

- type: string

enum:

- NO_REPLY_PATH_SET

- REPLY_PATH_SET

- type: string

End of changes