3GPP TSG-SA5 Meeting #124	S5-192088

# Taipei, Taiwan 25 February-1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0058 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements related to Secondary Node Change for MR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-02-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the MR-DC scenario, the Secondary Node Change procedure is initiated either by MN or SN and used to transfer a UE context from a source SN to a target SN and to change the SCG configuration in UE from one SN to another.The Secondary Node Change procedure always involves signalling over MCG SRB towards the UE. The performance of Secondary Node Change procedure needs to be monitored. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added measurements related to the Secondary Node Change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance of Secondary Node Change procedure cannot be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.X(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

5.1.1.X	Secondary Node Change

5.1.1.X.1 	Number of Secondary Node Change Attempts initiated by SN

a)	This measurement provides the number of Secondary Node Change attempts initiated by the SN.

b)	CC

c)	On Receipt of by the MN of an SN Change Required message from SN.

d)	Each measurement is an integer value.

e)	The measurement name has the form MRDC.SNChangeAttInitiatedBySN.

f)	NRCellCU

g)	Valid for packet switching

h)	5GS

5.1.1.X.2	Successful Secondary Node Change initiated by SN

a)	This measurement provides the number of successful Secondary Node Changes initiated by the SN.

b)	CC

c)	On transmission by the MN of an UE Context Release message to SN (after MN receives RRCConnectionReconfigurationComplete message from UE) for SN change.

c)	Each measurement is an integer value.

d)	The measurement name has the form MRDC.SNChangeSuccInitiatedBySN.

f)	NRCellCU

g)	Valid for packet switching

h)	5GS

Next change

A.X     Monitor of Secondary Node Change for Multi-Radio Dual Connectivity

In the Multi-Radio Dual Connectivity scenario, the Secondary Node Change procedure is initiated either by MN or SN and used to transfer a UE context from a source SN to a target SN and to change the SCG configuration in UE from one SN to another.The failed SN Change similar to partial failed handover, which directly impacts the QoS delivered by the networks, and the satisfaction degree of the end user.

End of changes