3GPP TSG-SA5 Meeting #124 	S5-192122

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0045 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Offline charging service procedures. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OFSBI_CH |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The 5G data network offline charging service procedure is not specified in TS 32.255. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution is to add text and figures for 5G data connectivity charging, including offline charging service procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure for offline only charging is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change to TS 32.255

5.2.2.2.2	PDU session establishment

The following figure 5.2.2.2.1 describes a PDU session establishment charging, based on clause 4.3.2.2.1 TS 23.502 [202] description:

Figure 5.2.2.2.2.1: PDU session establishment

1.	The UE initiates a PDU Session establishment procedure (PDU Session ID, PDU Type, SSC mode, User location information and Access Technology Type Information).

[2 to 3]. SMF selection by AMF, and sending of SM Request (PDU Session Establishment Request).

[4a-b] to 5.	Subscription Data Request to UDM, based on (Subscriber Permanent ID, DNN): Default QoS profile.

6.	PDU session authentication/authorization.

[7a-b].	If dynamic PCC is deployed, the SMF performs PCF selection, and requests for PCC Rules, and SMF selection of UPF.

8. The SMF selects an SSC mode for the PDU Session and also selects one or more UPFs.

9. The SMF may perform a Session Management Policy Modification procedure to report some event to the PCF that has previously subscribed.

9ch-a.The SMF creates a Charging Id for the PDU session, and sends Charging Data Request [initial] to CHF for authorization for the subscriber to start the PDU session which is triggered by start of PDU session charging event.

9ch-b. The CHF opens CDR for this PDU session.

9ch-c. The CHF acknowledges by sending Charging Data Response [Initial] to the SMF.

10. the SMF initiates an N4 Session Establishment procedure with the selected UPF, otherwise it initiates an N4 Session Modification procedure with the selected UPF.

11.	SMF-AMF interaction.

[12 to 14].	AMF-(R)AN-UE interaction.

15.	AMF to SMF: SM Request (N2 SM information).

[16a-b]. N4 session Modification: the SMF provides AN Tunnel Info to the UPF. From this step, the UPF delivers any down-link packets to the UE that may have been buffered for this PDU Session.

16ch-a. This step may occur in case "start of service data flow" needs quota from CHF, for the SMF to request quota.

16ch-b. The CHF updates CDR for this PDU session.

16ch-c. The CHF acknowledges by sending Charging Data Response [Update] to the SMF.

NOTE 1: The steps from 16ch-a to 16ch-c for quota request from CHF is not applicable for offline only charging.

17.	SM Request with PDU session Update Response to AMF.

18. SMF sends SM Context Status Notify to AMF.

19.	In case of PDU Type IPv6, IPv6 Address Configuration.

End of Change