3GPP TSG-SA5 Meeting #124 	S5-192283

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-192202

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0004 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add message flows for converged charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1_NEFCH |  |  |  |  |  | Date: |  |  | 2019-02-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Message flow for converged charging is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding of the message flows for the converged charging both IEC and ECUR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No message flows for the converged charging of northbound API |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x.2 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.x.2	Message flows

#### 5.x.2.1	Introduction

The different scenarios below focus on the different messages from/to the NEF and corresponding interaction with the CHF, based on scenarios specified in TS 23.222 [202].

#### 5.x.2.2	API Invocation - IEC

Figure 5.x.2.2.1 describes the scenario where there is an API invocation request at the NEF for IEC mode

Figure 5.x.2.2.1: API Invocation Request to NEF using IEC

1.	NEF receives an API invocation Request from an AF.

2.	NEF performs the actions needed to fulfil the API invoked.

3.	If authorized, the NEF continues the API invocation processing and sends the API Invocation Response.

3ch-a. The NEF sends Charging Data Request [Event] to CHF for the received API Invocation.

3ch-b. The CHF creates a CDR for this API Invocation.

3ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the NEF.

#### 5.x.2.3	API Invocation - ECUR

Figure 5.x.2.3.1 describes the scenario where there is an API invocation request at the NEF for ECUR mode.

Figure 5.x.2.3.1: API Invocation Request to NEF using ECUR

1.	NEF receives an API invocation Request from an AF.

1ch-a.	The NEF sends Charging Data Request [Initial] to CHF for the received API Invocation.

1ch-b. The CHF opens CDR for this API Invocation.

1ch-c. The CHF acknowledges by sending Charging Data Response [Initial] to the NEF.

2.	NEF performs the actions needed to fulfil the API invoked.

3.	If authorized, the NEF continues the API invocation processing and sends the API Invocation Response.

3ch-a. The NEF sends Charging Data Request [Termination] to the CHF for terminating the charging associated with the API Invocation.

3ch-b. The CHF closes the CDR for this API Invocation.

3ch-c. The CHF acknowledges by sending Charging Data Response [Termination] to the NEF.

#### 5.x.2.4	API Notification - IEC

Figure 5.x.2.4.1 describes the scenario where an API Notification is delivered from the NEF for IEC mode

Figure 5.x.2.4.1 API Notification from NEF using IEC

1.	The NEF receives a notification from an NF.

2.	The NEF sends the notification to AF.

2ch-a. The NEF sends Charging Data Request [Event] to CHF for the Notification.

2ch-b. The CHF creates a CDR for this Notification.

2ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the NEF.

3.	The NEF receives acknowledgement for the notification.

#### 5.x.2.5	API event Notification - ECUR

Figure 5.x.2.5.1 describes the scenario where an API event Notification is delivered from the NEF for ECUR mode.

Figure 5.x.2.4.1: API Notification from NEF using ECUR

1.	The NEF receives a notification from an NF.

1ch-a.	The NEF sends Charging Data Request [Initial] to CHF for the Notification.

1ch-b. The CHF opens CDR for this API Notification.

1ch-c. The CHF acknowledges by sending Charging Data Response [Initial] to the NEF.

2. 	The NEF sends the notification to AF.

3.	The NEF receives acknowledgement for the notification.

3ch-a. The NEF sends Charging Data Request [Termination] to the CHF for terminating the charging associated with the API event Notification.

3ch-b. The CHF closes the CDR for this API Notification.

3ch-c. The CHF acknowledges by sending Charging Data Response [Termination] to the NEF.

End of changes