3GPP TSG-SA5 Meeting #124	S5-192418

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-192226

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0069 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to sST attribute stage 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-5GNRM |  |  |  |  |  | Date: |  |  | 2019-02-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Stage 3 for the sST attribute is not aligned with Stage 2 attribute definitions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To align Stage 3 with Stage 2: - sST attribute is added to ServiceProfile |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If correction not performed, the Stage 3 for the sST attribute will not be correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | I.4.3, J.4.3, K4.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

## I.4.3	XML schema "sliceNrm.xsd"

<?xml version="1.0" encoding="UTF-8"?>

<!--

3GPP TS 28.541 network slice Network Resource Model

XML schema definition

sliceNrm.xsd

-->

<schema xmlns="http://www.w3.org/2001/XMLSchema"

xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"

xmlns:sl="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#sliceNrm"

xmlns:nn="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"

xmlns:ngc="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

xmlns:en="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"

xmlns:sm="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"

targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#sliceNrm" elementFormDefault="qualified">

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"/>

<simpleType name="MobilityLevel">

<restriction base="string">

<enumeration value="STATIONARY"/>

<enumeration value="NOMADIC"/>

<enumeration value="RESTRICTED MOBILITY"/>

<enumeration value="FULLY MOBILITY"/>

</restriction>

</simpleType>

<simpleType name="SharingLevel">

<restriction base="string">

<enumeration value="SHARED"/>

<enumeration value="NON-SHARED"/>

</restriction>

</simpleType>

<complexType name="PerfReq">

<!-- Refer to definitions in TS 22.261-->

<sequence>

<choice minOccurs="0" maxOccurs="1">

<element name="perfReqEMBB" type="sl:PerfReqEmbb"/>

<element name="perfReqUrllc" type="sl:PerfReqUrllc"/>

</choice>

</sequence>

</complexType>

<complexType name="PerfReqEmbb">

<sequence>

<element name="expDataRateDL" type="integer" minOccurs="0"/>

<element name="expDataRateUL" type="integer" minOccurs="0"/>

<element name="areaTrafficCapDL" type="integer" minOccurs="0"/>

<element name="areaTrafficCapUL" type="integer" minOccurs="0"/>

<element name="userDensity" type="integer" minOccurs="0"/>

<element name="activityFactor" type="integer " minOccurs="0"/>

<element name="uESpeed" type="integer" minOccurs="0"/>

<element name="coverage" type="string" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="PerfReqUrllc">

<sequence>

<element name="e2eKatency" type="integer" minOccurs="0"/>

<element name="jitter" type="integer" minOccurs="0"/>

<element name="survivalTime" type="integer" minOccurs="0"/>

<element name="cSAvailability" type="float" minOccurs="0"/>

<element name="reliability" type="float" minOccurs="0"/>

<element name="expDataRate" type="integer " minOccurs="0"/>

<element name="payloadSize" type="integer" minOccurs="0"/>

<element name="trafficDensity" type="string" minOccurs="0"/>

<element name="connDensity" type="float" minOccurs="0"/>

<element name="serviceDimension" type="string " minOccurs="0"/>

</sequence>

</complexType>

<complexType name="ServiceProfile">

<sequence>

<element name="serviceProfileId" type="string"/>

<element name="sNSSAIList" type="ngc:SnssaiList"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="perfReq" type="sl:PerfReq"/>

<element name="maxNumberofUEs" type="long" minOccurs="0"/>

<element name="coverageAreaTAList" type="ngc:NrTACList" minOccurs="0"/>

<element name="latency" type="integer" minOccurs="0"/>

<element name="uEMobilityLevel" type="integer" minOccurs="0"/>

<element name="resourceSharingLevel" type="integer" minOccurs="0"/>

<element name="sst" type="ngc:Sst"/>

</sequence>

</complexType>

<complexType name="ServiceProfileList">

<sequence>

<element name="serviceProfile" type="sl:ServiceProfile"/>

</sequence>

</complexType>

<complexType name="SliceProfile">

<sequence>

<element name="sliceProfileId" type="string"/>

<element name="sNSSAIList" type=" ngc:SnssaiList"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="perfReq" type="sl:PerfReq"/>

<element name="maxNumberofUEs" type="long" minOccurs="0"/>

<element name="coverageAreaTAList" type="ngc:NrTACList" minOccurs="0"/>

<element name="latency" type="integer" minOccurs="0"/>

<element name="uEMobilityLevel" type="sl: MobilityLevel" minOccurs="0"/>

<element name="resourceSharingLevel" type="integer" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="SliceProfileList">

<sequence>

<element name="sliceProfile" type="sl:SliceProfile"/>

</sequence>

</complexType>

<complexType name="NsInfo">

<!-- Refer to definitions in subclause 8.3.3.2.2 of ETSI NFV IFA013 -->

<sequence>

<element name="nsInstanceId" type="string"/>

<element name="nsName" type="string"/>

<element name="description" type="string"/>

</sequence>

</complexType>

<element name="NetworkSlice" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from SubNetwork -->

<element name="dnPrefix" type="string" minOccurs="0"/>

<element name="userLabel" type="string"/>

<element name="userDefinedNetworkType" type="string"/>

<element name="setOfMcc" type="string" minOccurs="0"/>

<!-- End of inherited attributes from SubNetwork -->

<element name="nSSIId" type="xn:dn"/>

<element name="operationalState" type="sm:operationalStateType"/>

<element name="administrativeState" type="sm:administrativeStateType"/>

<element name="serviceProfileList" type="sl:ServiceProfileList"/>

<element name="sst" type="ngc:Sst"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="sl:NetworkSliceSubnet"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NetworkSliceSubnet" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from SubNetwork -->

<element name="dnPrefix" type="string" minOccurs="0"/>

<element name="userLabel" type="string"/>

<element name="userDefinedNetworkType" type="string"/>

<element name="setOfMcc" type="string" minOccurs="0"/>

<!-- End of inherited attributes from SubNetwork -->

<element name="mFIdList" type="xn:dnList"/>

<element name="ConstituentNSSIIdList" type="xn:dnList"/>

<element name="operationalState" type="sm:operationalStateType"/>

<element name="administrativeState" type="sm:administrativeStateType"/>

<element name="nsInfo" type="sl:NsInfo" minOccurs="0"/>

<element name="sliceProfileList" type="sl:SliceProfileList"/>

<element name="sst" type="ngc:Sst"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="sl:NetworkSliceSubnet"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

</schema>

Next Modified Section

## J.4.3	JSON schema "sliceNrm.json"

{

"$schema": "http://json-schema.org/draft-04/schema#"

"id": "http://3gpp.org/28541/sliceNrm.json",

"description": "JSON based solution set definitions for network slice and network slice subnet",

"_referenced_schema":"http://3gpp.org/28623/genericNrm.json",

"_referenced_schema":"http://3gpp.org/28626/stateManagementNrm.json",

"_referenced_schema":"http://3gpp.org/28541/nrNrm.json",

"_referenced_schema":"http://3gpp.org/28541/ngcNrm.json",

"definitions": {

"networkSlice": {

"type": "object",

"properties": {

"dnPrefix": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"userDefinedNetworkType": {

"type": "string"

},

"setOfMcc": {

"type": "string"

},

"nSSIId": {

"$ref": "http://3gpp.org/28623/genericNrm.json#definitions/dn"

},

"operationalState": {

"$ref": "http://3gpp.org/28659/stateManagementNrm.json#definitions/operationalState"

},

"administrativeState": {

"$ref": "http://3gpp.org/28659/stateManagementNrm.json#definitions/administrativeState"

},

"serviceProfileList": {

"$ref": "#/definitions/ServiceProfileList"

},

"sst": {

"$ref": "http://3gpp.org/28541/ngcNrm.json#definitions/Sst"

}

},

"required": [

"userLabel",

"userDefinedNetworkType",

"nSSIId",

"operationalState",

"administrativeState",

"serviceProfileList",

"sst"

]

}

"networkSliceSubnet": {

"type": "object",

"properties": {

"dnPrefix": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"userDefinedNetworkType": {

"type": "string"

},

"setOfMcc": {

"type": "string"

},

"mFIdList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#definitions/dnlist"

},

"constituentNSSIIdList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#definitions/dnlist"

},

"operationalState": {

"$ref": "http://3gpp.org/28659/stateManagementNrm.json#definitions/operationalState"

},

"administrativeState": {

"$ref": "http://3gpp.org/28659/stateManagementNrm.json#definitions/administrativeState"

},

"nsInfo": {

"$ref": "#/definitions/NsInfo"

},

"sliceProfileList": {

"$ref": "#/definitions/ServiceProfileList"

},

"sst": {

"$ref": "http://3gpp.org/28541/ngcNrm.json#definitions/Sst"

}

},

"required": [

"userLabel",

"",

"mFIdList",

"constituentNSSIIdList",

"operationalState",

"administrativeState",

"sliceProfileList",

"sst"

]

},

"MobilityLevel": {

"type": "string",

"enum": [

"STATIONARY",

"NOMADIC",

"RESTRICTED MOBILITY",

"FULLY MOBILITY",

]

},

"SharingLevel": {

"type": "string",

"enum": [

"SHARED",

"NON-SHARED"

]

},

"PrefReq": {

"type": "object",

"properties": {

"prefReqEmbb": {

"$ref": "#/definitions/PrefReqEmbb"

},

"prefReqUrllc": {

"$ref": "#/definitions/PrefReqUrllc"

}

}

},

"PerfReqEmbb":{

"type": "object",

"properties": {

"expDataRateDL": {

"type": "number"

},

"expDataRateUL": {

"type": "number"

},

"areaTrafficCapDL": {

"type": "number"

},

"areaTrafficCapUL": {

"type": "number"

},

"userDensity": {

"type": "number"

},

"activityFactor": {

"type": "number"

},

"uESpeed": {

"type": "number"

},

"coverage": {

"type": "string"

}

}

},

"PerfReqUrrlc":{

"type": "object",

"properties": {

"e2eLatency": {

"type": "number"

},

"jitter": {

"type": "number"

},

"survivalTime": {

"type": "number"

},

"areaTrafficCapUL": {

"type": "number"

},

"cSAvailability": {

"type": "number"

},

"reliability": {

"type": "number"

},

"expDataRate": {

"type": "number"

},

"payloadSize": {

"type": "number"

}

"trafficDensity": {

"type": "string"

},

"connDensity": {

"type": "number"

},

"serviceDimension": {

"type": "string"

}

}

},

"ServiceProfile": {

"type": "object",

"properties": {

"serviceProfileId": {

"type": "string"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#definitions/SnssaiList"

},

"pLMNIdList": {

"$ref": "http://3gpp.org/28659/eutranNrm.json#definitions/PLMNIdList"

},

"perfReq": {

"$ref": "#/definitions/PrefReq"

},

"maxNumberofUEs": {

"type": "number"

},

"coverageAreaTAList": {

"$ref": "http://3gpp.org/28541/ngcNrm.json#definitions/TACList"

},

"latency": {

"type": "number"

},

"uEMobilityLevel": {

"type": "number "

},

"sst": {

"$ref": "http://3gpp.org/28541/ngcNrm.json#definitions/Sst"

}

"resourceSharingLevel": {

"type": "integer "

}

}

},

"ServiceProfileList": {

"type": "array",

"item": {

"$ref": "#/definitions/ServiceProfile"

}

}

"SliceProfile": {

"type": "object",

"properties": {

"sliceProfileId": {

"type": "string"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#definitions/SnssaiList"

},

"pLMNIdList": {

"$ref": "http://3gpp.org/28659/eutranNrm.json#definitions/PLMNIdList"

},

"perfReq": {

"$ref": "#/definitions/PrefReq"

},

"maxNumberofUEs": {

"type": "number"

},

"coverageAreaTAList": {

"$ref": "http://3gpp.org/28541/ngcNrm.json#definitions/TACList"

},

"latency": {

"type": "number"

},

"uEMobilityLevel": {

"type": "number "

}

"resourceSharingLevel": {

"type": "integer "

}

}

},

"SliceProfileList": {

"type": "array",

"item": {

"$ref": "#/definitions/SliceProfile"

}

}

"NsInfo": {

"type": "object",

"properties": {

"nsInstanceId": {

"type": "string"

},

"nsName": {

"type": "string"

},

"description ": {

"type": "string"

}

}

}

}

}

Next Modified Section

## K.4.2	Graphical representation

module: NetworkSlice

+--rw NetworkSlice

+--ro objectClass               string

+--ro objectInstance            nrm-type:t_dn

+--ro id                        nrm-type:t_dn

+--ro dnPrefix                  string

+--rw userLabel                 string

+--rw userDefinedNetworkType    string

+--ro setOfMcc*                 nrm-type:t_mcc

+--ro nSSIId                    nrm-type:t_dn

+--ro operationalState          nrm-type:t_operationalState

+--rw administrativeState       nrm-type:t_administrativeState

+--rw serviceProfile* [serviceProfileId]

|  +--rw serviceProfileId        nrm-type:t_dn

|  +--rw s-NSSAI*                nrm-type:t_s-NSSAI

|  +--rw pLMNId

|  |  +--rw MCC   t_mcc

|  |  +--rw MNC   t_mnc

|  +--rw perfReq

|  |  +--rw (SST)

|  |     +--:(eMBB)

|  |     |  +--rw expDataRateDL          uint16

|  |     |  +--rw expDataRateU           uint16

|  |     |  +--rw areaTrafficCapDL       uint16

|  |     |  +--rw areaTrafficCapUL       uint16

|  |     |  +--rw userDensity            uint16

|  |     |  +--rw activityFactor         uint16

|  |     |  +--rw uESpeed                uint16

|  |     |  +--rw coverage               string

|  |     +--:(uRLLC)

|  |        +--rw e2eLatency             uint16

|  |        +--rw jitter                 uint16

|  |        +--rw survivalTime           uint16

|  |        +--rw cSAvailability         decimal64

|  |        +--rw reliability            decimal64

|  |        +--rw expDataRate            uint16

|  |        +--rw payloadSize            string

|  |        +--rw trafficDensity         uint16

|  |        +--rw connDensity            uint16

|  |        +--rw serviceAreaDimension   string

|  +--rw maxNumberofUEs         uint64

|  +--rw coverageAreaTA*         nrm-type:t_tAC

|  +--rw latency                 uint16

|  +--rw uEMobilityLevel        nrm-type:t_uEMobilityLevel

|  +--rw resourceSharingLevel   nrm-type:t_resourceSharingLevel

|  +--rw sST                    nrm-type:t_SST

module: NetworkSliceSubnet

+--rw NetworkSliceSubnet

+--ro objectClass               string

+--ro objectInstance            nrm-type:t_dn

+--ro id                        nrm-type:t_dn

+--ro dnPrefix                  string

+--rw userLabel                 string

+--rw userDefinedNetworkType    string

+--ro setOfMcc*                 nrm-type:t_mcc

+--ro mFId*                     nrm-type:t_dn

+--ro constituentNSSIIdList*    nrm-type:t_dn

+--rw State                     string

+--ro nsInfo                   nrm-type:t_dn

+--rw sliceProfile* [sliceProfileId]

|  +--rw sliceProfileId          nrm-type:t_dn

|  +--rw s-NSSAI*                nrm-type:t_s-NSSAI

|  +--rw pLMNId

|  |  +--rw MCC   t_mcc

|  |  +--rw MNC   t_mnc

|  +--rw perfReq

|  |  +--rw (SST)

|  |     +--:(eMBB)

|  |     |  +--rw expDataRateDL          uint16

|  |     |  +--rw expDataRateU           uint16

|  |     |  +--rw areaTrafficCapDL       uint16

|  |     |  +--rw areaTrafficCapUL       uint16

|  |     |  +--rw userDensity            uint16

|  |     |  +--rw activityFactor         uint16

|  |     |  +--rw uESpeed                uint16

|  |     |  +--rw coverage               string

|  |     +--:(uRLLC)

|  |        +--rw e2eLatency             uint16

|  |        +--rw jitter                 uint16

|  |        +--rw survivalTime           uint16

|  |        +--rw cSAvailability         decimal64

|  |        +--rw reliability            decimal64

|  |        +--rw expDataRate            uint16

|  |        +--rw payloadSize            string

|  |        +--rw trafficDensity         uint16

|  |        +--rw connDensity            uint16

|  |        +--rw serviceAreaDimension   string

|  +--rw maxNumberofUEs         uint64

|  +--rw coverageAreaTA*         nrm-type:t_tAC

|  +--rw latency                 uint16

|  +--rw uEMobilityLevel        nrm-type:t_uEMobilityLevel

|  +--rw resourceSharingLevel   nrm-type:t_resourceSharingLevel

+--rw sST                       nrm-type:t_SST

Next Modified Section

### K.4.3.3	NetworkSlice Module, "NetworkSlice.yang"

module NetworkSlice {

namespace "urn:3gpp:tsg:sa5:nrm:NetworkSlice";

prefix "ns";

import SubNetwork { prefix subnet; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

include ServiceProfile;

description "";

revision 2018-08-09 {

description "Initial revision";

}

container NetworkSlice {

uses subnet:SubNetwork;

leaf nSSIId {

mandatory "true";

config "false";

description "";

type nrm-type:t_dn;

}

leaf operationalState {

mandatory "true";

config "false";

description "";

type nrm-type:t_operationalState;

}

leaf administrativeState {

mandatory "true";

config "true";

description "";

type nrm-type:t_administrativeState;

}

list serviceProfile {

min-elements "1";

config "true";

description "";

key serviceProfileId;

uses ServiceProfile;

}

leaf sST {

mandatory "true";

config "true";

description "";

type nrm-type:t_SST;

}

}

}

Next Modified Section

### K.4.3.4	ServiceProfile Submodule, "ServiceProfile.yang"

submodule ServiceProfile {

belongs-to NetworkSlice { prefix ns; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import perfReq { prefix perf; revision-date "2018-08-10"; }

revision 2018-08-09 {

description "Initial revision";

}

grouping ServiceProfile {

leaf serviceProfileId {

mandatory "true";

description "";

type nrm-type:t_dn;

}

leaf-list s-NSSAI  {

min-elements "1";

config "true";

description "";

type nrm-type:t_s-NSSAI;

}

list pLMNId {

min-elements "1";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

container perfReq {

presence "true";

config "true";

description "";

uses perf:perfReq;

}

leaf maxNumberofUEs {

mandatory "false";

config "true";

description "";

type uint64;

}

leaf-list coverageAreaTA {

min-elements "1";

config "true";

description "";

type nrm-type:t_tAC;

}

leaf latency {

mandatory "true";

config "true";

description "";

type uint16;

}

leaf uEMobilityLevel {

mandatory "false";

config "true";

description "";

type nrm-type:t_uEMobilityLevel;

}

leaf resourceSharingLevel {

mandatory "false";

config "true";

description "";

type nrm-type:t_resourceSharingLevel;

}

leaf sST {

mandatory "true";

config "true";

description "";

type nrm-type:t_SST;

}

}

}

Next Modified Section

### K.4.3.5	NetworkSlice Module, "NetworkSliceSubnet.yang"

module NetworkSliceSubnet {

namespace "urn:3gpp:tsg:sa5:nrm:NetworkSliceSubnet";

prefix "nss";

import SubNetwork { prefix subnet; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

include SliceProfile;

description "";

revision 2018-08-09 {

description "Initial revision";

}

container NetworkSliceSubnet {

uses subnet:SubNetwork;

leaf-list mFId {

min-elements  "0";

config "false";

description "";

type nrm-type:t_dn;

}

leaf-list constituentNSSIIdList {

min-elements  "0";

config "false";

description "";

type nrm-type:t_dn;

}

leaf operationalState {

mandatory "true";

config "false";

description "";

type nrm-type:t_operationalState;

}

leaf administrativeState {

mandatory "true";

config "true";

description "";

type nrm-type:t_administrativeState;

}

leaf nsInfo {

mandatory "false";

config "false";

description "";

type nrm-type:t_dn;

}

list sliceProfile {

min-elements  "1";

config "true";

description "";

key sliceProfileId;

uses SliceProfile;

}

leaf sST {

mandatory "true";

config "true";

description "";

type nrm-type:t_SST;

}

}

}

End of Modified Sections