3GPP TSG-SA5 Meeting #124	S5-192140

# Taipei,Taiwan, 25 Februry-1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0008 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use case and definitions of packet loss rate measurement over N3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI, KT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019/2/25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In order to provide performance assurance of 5G services, it is important to measure end-to-end performance. Performance measurement at gNB level may not satisfy this requirement, Packet loss, delay, or latency can happen between gBN and UPF and their measurement thus is also very important. Performance measurements from UE to gNB is well defined but those of N3 is currently missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following measurements: - Loss rate of the incoming GTP data packets total and per QoS flow on the N3 interface, from (R)AN to UPF - Loss rate of the outgoing GTP data packets total and per QoS flow on the N3 interface, from UPF to (R)AN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance measurement beside data volume for UP traffic on N3 cannot be made. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.X (new), A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

5.4.1.x  Incoming GTP Data Packet Loss Rate

a)	This measurement provides the fraction of GTP data packets which are not successfully received at UPF. It is a measure of the incoming GTP data packet loss on the UPF interface.  The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

b)	SI

c)	This measurement is obtained as:  1000000* Number of missing incoming GTP sequence numbers (TS 29.281), representing packets that are not delivered to higher layers divided by Total number of incoming GTP sequence numbers (also including missing sequence numbers) of a bearer, starting from the GTP sequence number of the first packet delivered by gNB to UPF until the GTP sequence number of the last packet. Separate counters are optionally maintained for mapped 5QI (or QCI for option 3).

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS level measurement is perfomed, the measurements are equal to the number of mapped 5QIs.

e)	The measurement name has the form GTP.InDattaPktPacketLossRateN3UPF or optionally GTP.InDattaPktPacketLossRateN3UPF.QoS where QoS identifies the target quality of service class.

f)	EP_N3

g)	Valid for packet switched traffic

h)	5GS

5.4.1.x  Outgoing GTP Packet Loss Rate

a)	This measurement provides the fraction of GTP data packets which are not successfully received at gNB over N3. It is a measure of the outgoing GTP data packet loss on the UPF interface.  The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

b)	SI

c)	This measurement is obtained as:  1000000* Number of missing incoming GTP sequence numbers (TS 29.281), representing packets that are not delivered to higher layers divided by Total number of outgoing GTP sequence numbers (also including missing sequence numbers) of a bearer, starting from the GTP sequence number of the first packet delivered by UPF to gNB until the GTP sequence number of the last packet. Separate counters are optionally maintained for mapped 5QI (or QCI for option 3).

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS level measurement is perfomed, the measurements are equal to the number of mapped 5QIs.

e)	The measurement name has the form GTP.OutDattaPktPacketLossRateN3UPF or optionally GTP.OutDattaPktPacketLossRateN3UPF.QoS where QoS identifies the target quality of service class.

f)	EP_N3

g)	Valid for packet switched traffic

h)	5GS

Next modified section

A.x	Monitoring of incoming/outgoing GTP packet loss on N3

Keeping track of GTP data packet loss over N3 is essential, since for certain services packets that are lost along the way through the system may have a noticeable impact on the end user. Incoming/outgoing GTP data packet loss measurements can be useful for evaluation, optimization and for performance assurance between RAN and UPF in the core.  It is also important for the performance measurement of end-to-end point of view from UE to Core. Performance degradation can happen any point although the focus is centered more at UE and RAN.

End of modifications