3GPP TSG-SA5 Meeting #125	S5-193151

# Newport Beach, US, 8-12 April 2019

# Source:		Ericsson

# Title:	Mapping rule between DN and URI

# Document for:	Endorsement

Agenda Item:	TBD

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

# [1]	3GPP 32.300 Name Convention

# [2]	IETF RFC 3986 “Uniform Resource Identifier (URI): Generic Syntax

# 3	Background

In the context of network management, all managed object instance has a DN [1]. In some management solutions, the use of URI (Universal Resource Identifier) [2] is common for identification of managed resources. In the context of network management, the semantics of managed object instance is equivalent to that of managed resources.

A management system can be using multiple management solutions for the management of its network.  Its managed nodes/functions can be named by DN or by URI or, yet, by both DN or URI. For example, a management system can receive PM files of Measurements of a managed function identified by DN. This management system may need to configure the same managed function supplying the managed function’s URI. The management system can also receive alarms bearing the DN of this same managed function.

In order for the management system to correlate network management information received, some carrying DN while others carrying URI, it is essential that a predictive mapping rule be standardized, between DN and URI.

# 4	Detailed proposal

# It is proposed to endorse the creation of a new subsection in [1] to capture the following:

“

# 10	DN URI mapping

A DN is defined (in [1]) as: “RegularDN = DNPrefixPlusRDNSeparator, LocalDN; “

An URI is defined in [2] as:

“URI = scheme ":" hier-part [ "?" query ] [ "#" fragment ]

hier-part   = "//" authority path-abempty

/ path-absolute

/ path-rootless

/ path-empty

A DN is mapped semantically to an URI’s authority and path components. Specifically, the DNPrefixPlusRDNSeparator of RegularDN is mapped semantically to the URI authority component and the LocalDN of RegularDN is mapped semantically to the URI path-abempty component.

“