3GPP TSG-SA5 Meeting #125 	S5-193255

# Newport Beach, US, 8-12 April 2019

# Source:	GSMA

# Title:	LS to SA2 and SA5 on VoWiFi – VoLTE handover

# Document for:	Information, Discussion

Agenda Item:	7.1

# Liaison Statement

| Liaison Statement Title: | LS to SA2 and SA5 on VoWiFi – VoLTE handover |
| --- | --- |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| RILTE #67 | 2018-11-08 | Tokyo, Japan |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Author: |
| RILTE #67 Doc 116 | 2018-11-16 | Mari Melander (Telekom Deutschland GmbH) |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| NG RILTE | RILTE#68(see below) | mihaela.ambrozie@vodafone.com |
| Security Classification | Non-confidential |  |

| Action Required by Recipient |  |
| --- | --- |
| Internal Recipients: | None |
| External Recipients: | To: 3GPP SA WG2, 3GPP SA WG5 CC: 3GPP CT WG1 3GPPLiaison@etsi.org) |

NG RILTE has been made aware by several operators about roaming related scenarios, whereby a UE that started a voice session using VoWiFi subsequently performs a handover to VoLTE. This situation was observed to occur regardless of whether the visited network indicated VoLTE support with VoPS (Voice over PS) indication or not. NG RILTE has some questions related to the cited scenario that seem not to be addressed by existing 3GPP specifications:

Q1: How can the handover be prevented in situations where the visited network does not send VoPS indication, i.e. it either does not support VoLTE or it supports VoLTE but does not have VoLTE roaming agreement with the home network of the inbound roamer?

Q2: How can a handover be prevented in situations where the visited network wants to block handovers in certain TACs from all users (including own users) regardless of roaming agreements?

Q3: How can the visited network properly charge a session that is handed over between VoWiFi and VoLTE assuming the visited network supports VoLTE and has a VoLTE roaming agreement with the home network of the inbound roamer?

ACTION to SA2:

NG RILTE kindly asks SA2 to respond to the above questions.

ACTION to SA5:

NG RILTE kindly asks SA5 to respond to Q3 in co-ordination with SA2.

Next Meetings:

RILTE#68 – 7th February, London, UK

RILTE#69 – 4th April, Split, Croatia