3GPP TSG-SA5 Meeting #125 	S5-193262

# Newport Beach, USA,  8 – 12 April 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0097 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add datatype definition for NfProfile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNRM |  |  |  |  |  | Date: |  |  | 2019-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the new methodology the datatype have the definitions documented in separate clauses. Currently there are no datatype definitions other than descriptive text in the attribute definition table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New sub-clause for datatype NfProfile is added, attribute definition table is updated accordingly |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no definitions for datatype NfProfile. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.x(New), 5.3.x.1 (New), 5.3.x.2 (New), 5.3.x.3 (New), 5.3.x.4 (New), 5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First modified section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[6]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[7]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[8]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[9]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[10]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM);Stage 1".

[11]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[14]	3GPP TS 36.410: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 general aspects and principles".

[15]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol".

[16]	3GPP TS 36.425: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 interface user plane protocol".

[17]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS)".

[18]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

[19]	3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[20]	3GPP TS 28.702: "Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[21]	3GPP TS 28.708: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[22]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[23]	3GPP TS 29.510: "5G system; Network Function Repository Services; Stage 3".

[24]	3GPP TS 29.531: "5G System; Network Slice Selection Services Stage 3".

[25]	3GPP TS 32.152: "Telecommunication management; Integration Reference Point (IRP) Information Service (IS) Unified Modelling Language (UML) repertoire".

[26]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[27]	3GPP TS 28.554: "Management and orchestration; 5G End to end Key Performance Indicators (KPI)".

[28]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[29]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualisation (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[30]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[31]	Void.

[32]	3GPP TS 38.211: "NR; Physical channels and modulation".

[33]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[34]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[35]	3GPP TS 28.532: "Management and orchestration; Management services".

[36]	Void.

[37]	IETF RFC 791: "Internet Protocol".

[38]	IETF RFC 2373: "IP Version 6 Addressing Architecture".

[39]	IEEE 802.1Q: "Media Access Control Bridges and Virtual Bridged Local Area Networks".

[40]	ETSI GS NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV); Management and Orchestration; Report on NFV Information Model".

[Y1]	3GPP TS 23.527: "5G System; Restoration procedures".

[Y2]	IETF RFC 2782: "A DNS RR for specifying the location of services (DNS SRV), February 2000".

First modified section

# 5		Information Model definitions for 5GC NRM

## 5.1 	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.622 [30], IOC, ManagedElement | ManagedElement |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [30], IOC, EP_RP | EP_RP |
| TS 28.708 [21], IOC, ServingGWFunction | ServingGWFunction |
| TS 28.702 [20], IOC, SmsIwmscFunction | SmsIwmscFunction |
| TS 28.702 [20], IOC, SmsGmscFunction | SmsGmscFunction |
| TS 28.702 [20], IOC, GmlcFunction | GmlcFunction |
| TS 28.658 [19], attribute, pLmnIdList | pLMNIDList |
| TS 29.510 [23], attribute, plmnList | pLMNIDList |
| TS 29.510 [23], attribute, sNssais | sNssais |
| TS 29.510 [23], attribute, allowedPlmns | allowedPlmns |
| TS 29.510 [23], attribute, allowedNssais | allowedNssais |

Second modified section

### 5.3.x	NfProfile <<dataType>>

#### 5.3.x.1	Definition

This datatype represents the properties of network function related requirement that should be supported by the network function instance in a 5G network. NfProfile is defined in 3GPP TS 29.510 [23].

#### 5.3.x.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nfInstanceID | M | T | T | T | T |
| nfType | M | T | T | T | T |
| nfStatus | M | T | T | F | T |
| heartBeatTimer | O | T | T | F | T |
| plmnList | CO | T | T | F | T |
| sNssais | O | T | T | F | T |
| nsiList | O | T | T | F | T |
| fqdn | CM | T | T | F | T |
| interPlmnFqdn | CM | T | T | F | T |
| ipv4Addresses | CM | T | T | F | T |
| ipv6Addresses | CM | T | T | F | T |
| allowedPlmns | O | T | T | F | T |
| allowedNfTypes | O | T | T | F | T |
| allowedNfDomains | O | T | T | F | T |
| allowedNssais | O | T | T | F | T |
| priority | O | T | T | F | T |
| capacity | O | T | T | F | T |
| load | O | T | F | F | N |
| locality | O | T | T | F | T |
| udrInfo | O | T | T | F | T |
| udmInfo | O | T | T | F | T |
| ausfInfo | O | T | T | F | T |
| amfInfo | O | T | T | F | T |
| smfInfo | O | T | T | F | T |
| upfInfo | O | T | T | F | T |
| pcfInfo | O | T | T | F | T |
| bsfInfo | O | T | T | F | T |
| chfInfo | O | T | T | F | T |
| nrfInfo | O | T | T | F | T |
| customInfo | O | T | T | F | T |
| recoveryTime | O | T | T | F | T |
| nfServicePersistence | O | T | T | T | T |
| nfServices | O | T | T | F | T |

#### 5.3.x.3	Attribute constraints

| Name | Definition |
| --- | --- |
| plmnList CO support qualifier | Condition: this attribute shall be present if this information is available for the NF. |
| Fqdn CM support qualifier | Condition: at least one of the addressing parameters (Fqdn, ipv4addresses or ipv6adresses) shall be included in the NF Profile. |
| interPlmnFqdn CM support qualifier | Condition: the NF needs to be discoverable by other NFs in a different PLMN |
| ipv4Addresses CM support qualifier | Condition: at least one of the addressing parameters (Fqdn, ipv4addresses or ipv6adresses) shall be included in the NF Profile. |
| ipv6Addresses CM support qualifier | Condition: at least one of the addressing parameters (Fqdn, ipv4addresses or ipv6adresses) shall be included in the NF Profile. |

#### 5.3.x.4               Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

Second modified section

## 5.4	Attribute definitions

### 5.4.1	Attribute properties

The following table defines the attributes that are present in several Information Object Classes (IOCs) of the present document.

| Attribute Name | Documentation and Allowed Values | Properties |  |
| --- | --- | --- | --- |
| aMFIdentifier | The AMFI is constructed from an AMF Region ID, an AMF Set ID and an AMF Pointer. The AMF Region ID identifies the region, the AMF Set ID uniquely identifies the AMF Set within the AMF Region, and the AMF Pointer uniquely identifies the AMF within the AMF Set. (Ref. 3GPP TS 23.003 [13]) | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| aMFSetId | It represents the AMF Set ID, which is uniquely identifies the AMF Set within the AMF Region. allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 [13]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| aMFSetMemberList | It is the list of DNs of AMFFunction instances of the AMFSet. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |  |
| aMFRegionId | It represents the AMF Region ID, which identifies the region. allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 [13]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| localAddress | This parameter specifies the localAddress including IP address and VLAN ID used for initialization of the underlying transport. First string is IP address, IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). Second string is VLAN Id (See IEEE 802.1Q [39]). | type: String multiplicity: 2 isOrdered: True isUnique: N/A defaultValue: None isNullable: False |
| remoteAddress | Remote address including IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nfProfileList | It is a set of NFProfile(s) to be registered in the NRF instance. NFProfile is defined in 3GPP TS 29.510 [23]. | type: NfProfile multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| nSIIdList | It is a set of NSI Id. The NSI ID is defined in subclause 6.1.6.2.8 of 3GPP TS 29.531 [24]. | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| sNSSAIList | See subclause 4.4.1. |  |  |
| sBIFQDN | It is used to indicate the FQDN of the registered NF instance in service-based interface, for example, NF instance FQDN structure is: nftype<nfnum>.slicetype<sliceid>.mnc<MNC>.mcc<MCC>.3gppnetwork.org | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| sBIServiceList | It is used to indicate the all supported NF services registered on service-based interface. | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| nRTACList | It is the list of Tracking Area Code (either legacy TAC or extended TAC) where the represented management function serving. allowedValues: Legacy TAC and Extended TAC are defined in clause 9.3.3.10 of TS 38.413 [5]. | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| weightFactor | The weight factor is typically set according to the capacity of local node (AMF) relative to other nodes in the same type. And it is used to achieve load balancing among a set of same type of network functions. (Ref. 3GPP TS 23.501 [2]) | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| nfInstanceID | It represents the identity of an NF instance, provided by the NF Service Consumer, that shall be globally unique inside the PLMN of the NRF where the NF is being registered. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 4122 [X1] See attribute nfInstanceID in TS 29.510 [23] subsection 5.2.2.2.2 | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| nfType | It represents the type of Network Function that can be found in the 5GC, as specified in Table 6.1.6.3.3-1 in TS 29.510 [23]. See attribute nfType in TS 29.510 [23] subsection 6.1.6.3.3 | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| nfStatus | It represents he status of an NF instance (NOTE X1) as specified in Table 6.6.3.7-1 in TS 29.510 [23]. See attribute nfStatus in TS 29.510 [23] subsection 6.1.6.3.7 | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| heartBeatTimer | It represents the time in seconds expected between 2 consecutive heart-beat messages from an NF Instance to the NRF [23]. See attribute heartBeatTimer in TS 29.510 [23] subsection 6.1.6.2.2 | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| plmnList | It represents the PLMN(s) of the Network Function (NOTE X2). This attribute shall be present if this information is available for the NF [23]. If not provided, PLMN ID(s) of the PLMN of the NRF are assumed for the NF [23] | type: PLMNId multiplicity: 1..N isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| sNssais | It represents S-NSSAIs of the Network Function. If not provided, the NF can serve any S-NSSAI. When present this attribute represents the list of S-NSSAIs supported in all the PLMNs listed in the plmnList attribute [23]. See attribute sNssais in TS 29.510 [23] subsection 6.1.6.2.2 | type: S-NSSAI multiplicity: 1..N isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| nsiList | NSI identities of the Network Function. If not provided, the NF can serve any NSI [23]. See attribute nsiList in TS 29.510 [23] subsection 6.2.6.2.3 | type: String multiplicity: 1..N isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| fQDN | It represents the Fully Qualified Domain Name of a Network Function [23]. See attribute fQDN in TS 29.510 [23] subsection 6.1.6.3.2 | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| interPlmnFqdn | It represents If the NF needs to be discoverable by other NFs in a different PLMN [23]. See attribute interPlmnFqdn in TS 29.510 [23] subsection 6.1.6.2.2 | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| ipv4Addresses | It represents the IPv4 address(es) of the Network Function (NOTE X3) [23]. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| ipv6Addresses | It represents the IPv6 address(es) of the Network Function (NOTE X3) [23] | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| allowedPlmns | It represents the PLMNs allowed to access the service instance (NOTE X4). The absence of this attribute indicates that any PLMN is allowed to access the service instance [23]. When included, the allowedPlmns attribute needs not include the PLMN ID(s) registered in the plmnList attribute of the NF Profile, i.e. the PLMN ID(s) registered in the NF Profile shall be considered to be allowed to access the service instance [23]. See attribute allowedPlmns in TS 29.510 [23] subsection 6.1.6.2.2 | type: PLMNId multiplicity: 1..N isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| allowedNfTypes | It represents the type of the NFs allowed to access the service instance (NOTE X4). [23] The absence of this attribute indicates that any NF type is allowed to access the service instance [23] See attribute heartBeatTimer in TS 29.510 [23] subsection 6.1.6.2.2 | type: String multiplicity: 1..N isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| allowedNfDomains | It represents the pattern (regular expression according to the ECMA-262 dialect [Y]) representing the NF domain names allowed to access the service instance (NOTE X4). The absence of this attribute indicates that any NF domain is allowed to access the service instance [23]. See attribute heartBeatTimer in TS 29.510 [23] subsection 6.1.6.2.2 | type: String multiplicity: 1..N isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| allowedNssais | It represents the S-NSSAI of the allowed slices to access the service instance (NOTE X4). The absence of this attribute indicates that any slice is allowed to access the service instance [23]. See attribute heartBeatTimer in TS 29.510 [23] subsection 6.1.6.2.2 | type: S-NSSAI multiplicity: 1..N isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| priority | It represents priority (relative to other services of the same type) in the range of 0-65535, to be used for NF Service selection; lower values indicate a higher priority. (NOTE X5). Further details can be found in Table 6.1.6.2.2-1 in TS 29.510 [23] See attribute priority in TS 29.510 [23] subsection 6.1.6.2.2 | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| capacity | It represents static capacity information in the range of 0-65535, expressed as a weight relative to other services of the same type. (NOTE X5) [23]. See attribute capacity in TS 29.510 [23] subsection 6.1.6.2.2 | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| load | It represents dynamic load information, ranged from 0 to 100, indicates the current load percentage of the NF Service [23]. See attribute load in TS 29.510 [23] subsection 6.1.6.2.2 | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| locality | It represents Operator defined information about the location of the NF instance (e.g. geographic location, data center) (NOTE X6) [23] See attribute localty in TS 29.510 [23] subsection 6.1.6.2.2 | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| udrInfo | It represents specific data for the UDR. The definition of type UdrInfo can be found in Table 6.1.6.2.6-1 in TS 29.510 [23] See attribute udrInfo in TS 29.510 [23] subsection 6.1.6.2.6 | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| udmInfo | It represents specific data for the UDM. The definition of type UdmInfo can be found in Table 6.1.6.2.7-1 in TS 29.510 [23] See attribute udmInfo in TS 29.510 [23] subsection 6.1.6.2.7 | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| ausfInfo | It represents specific data for the AUS. The definition of type AusInfo can be found in Table 6.1.6.2.8-1 in TS 29.510 [23] See attribute ausInfo in TS 29.510 [23] subsection 6.1.6.2.8 | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| amfInfo | It represents specific data for the AMF. The definition of type AmfInfo can be found in Table 6.1.6.2.11-1 in TS 29.510 [23] See attribute amfInfo in TS 29.510 [23] subsection 6.1.6.2.11 | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| smfInfo | It represents specific data for the SMF. The definition of type SmfInfo can be found in Table 6.1.6.2.12-1 in TS 29.510 [23] See attribute smfInfo in TS 29.510 [23] subsection 6.1.6.2.12 | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| upfInfo | It represents specific data for the UPF. The definition of type UpfInfo can be found in Table 6.1.6.2.13-1 in TS 29.510 [23] See attribute upfInfo in TS 29.510 [23] subsection 6.1.6.2.13 | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| pcfInfo | It represents specific data for the PCF. The definition of type Pcf Info can be found in Table 6.1.6.2.20-1 in TS 29.510 [23] See attribute pcfInfo in TS 29.510 [23] subsection 6.1.6.2.20 | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| bsfInfo | It represents specific data for the BSF. The definition of type BsfInfo can be found in Table 6.1.6.2.21-1 in TS 29.510 [23] See attribute bsfInfo in TS 29.510 [23] subsection 6.1.6.2.21 | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| chfInfo | It represents specific data for the CHF. The definition of type UdrInfo can be found in Table 6.1.6.2.32-1 in TS 29.510 [23] See attribute chfInfo in TS 29.510 [23] subsection 6.1.6.2.32 | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| nrfInfo | It represents specific data for the NRF. The definition of type NrfInfo can be found in Table 6.1.6.2.31-1 in TS 29.510 [23] See attribute nrfInfo in TS 29.510 [23] subsection 6.1.6.2.31 | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| customInfo | It represents specific data for custom Network Functions. Further details can be found in clause 5.3.2.2.2 in TS 29.510 [23] See attribute customInfo in TS 29.510 [23] subsection 5.3.2.2.2 | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| recoveryTime | It represents Timestamp when the NF was (re)started. Further details can be found in Table 6.1.6.2.2-1 in TS 29.510 [23] See attribute recoveryTime in TS 29.510 [23] subsection 6.1.6.2.2 | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| nfServicePersistence | If present, and set to true, it indicates that the different service instances of a same NF Service in this NF instance, supporting a same API version, are capable to persist their resource state in shared storage and therefore these resources are available after a new NF service instance supporting the same API version is selected by a NF Service Consumer (see 3GPP TS 23.527 [Y1]). Otherwise, it indicates that the NF Service Instances of a same NF Service are not capable to share resource state inside the NF Instance. See attribute nfServicePersistence in TS 29.510 [23] subsection 6.1.6.2.3 | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| nfServices | It represents list of NF Service Instances. It shall include the services produced by the NF that can be discovered by other NFs, if any. See attribute nfServices in TS 29.510 [23] subsection 6.1.6.2.3 | type: String multiplicity: 1..N isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| NOTE X1: The NRF shall notify NFs subscribed to receiving notifications of changes of the NF profile, if the NF recoveryTime or the nfStatus is changed. See subclause 6.2 of 3GPP TS 23.527 [Y1]. NOTE X2: A NF may register multiple PLMN IDs in its profile within a PLMN comprising multiple PLMN IDs. If so, all the attributes of the NF Profile shall apply to each PLMN ID registered in the plmnList. As an exception, attributes including a PLMN ID, e.g. IMSI-based SUPI ranges, TAIs and GUAMIs, are specific to one PLMN ID and the NF may register in its profile multiple occurrences of such attributes for different PLMN IDs (e.g. the UDM may register in its profile SUPI ranges for different PLMN IDs). NOTE X3: At least one of the addressing parameters (fqdn, ipv4address or ipv6adress) shall be included in the NF Profile. NOTE X4: If this attribute is present in the NFService and in the NF profile, the attribute from the NFService shall prevail. The absence of this attribute in the NFService and in the NFProfile indicates that there is no corresponding restriction to access the service instance. NOTE X5: The capacity and priority parameters, if present, are used for NF selection and load balancing. The priority and capacity attributes shall be used for NF selection in the same way that priority and weight are used for server selection as defined in IETF RFC 2782 [Y2]. NOTE X6: A requester NF may use this information to select a NF instance (e.g. a NF instance preferably located in the same data center). |  |  |  |

## 5.5	Common notifications

This subclause presents a list of notifications, defined in [35], that provisioning management service consumer can receive. The notification parameter objectClass/objectInstance, defined in [26], would capture the DN of an instance of an IOC defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOIAttributeValueChanges | O |  |
| notifyMOICreation | O |  |
| notifyMOIDeletion | O |  |

End of change