3GPP TSG-SA5 Meeting #125 	S5-193423

# Newport Beach, US, 8 12 April 2019												Revision of S5-193098

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0063 | rev | 1 | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on errors description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SBI_CH |  |  |  |  |  | Date: |  |  | 2019-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The application error value is missing when the Response code contains "400 Bad Request" value for the Notify. The ResultCode is defined for MultipleUnitInformation and it is not clear the corresponding behaviour applies at Rating Group level only. Whether the result code needs the service to always be terminated is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce one application error for Notify Indicate applicability to rating group Remove "terminate services" |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misinterpretation of errors could lead to service rejection instead of service continuation or service continuation instead of service rejection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.7.3, 6.1.6.3.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

#### 6.1.7.3	Application errors

The application errors defined for the Nchf_ConvergedCharging API are listed in table 6.1.7.3-1. The CHF shall include in the HTTP status code a "ProblemDetails" data structure with the "cause" attribute indicating the application error as listed in table 6.1.7.3-1. The common application errors defined in the Table 5.2.7.2-1 in 3GPP TS 29.500 [7] may also be used for the Npcf_ConvergedCharging service.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| CHARGING_FAILED | 400 Bad Request | The HTTP request is rejected because the set of session or subscriber information needed by the CHF for charging or CDR creation is incomplete or erroneous or not available. (E.g. Rating Group, subscriber information) |
| RE_AUTHORIZATION_FAILED | 400 Bad Request | The HTTP request is rejected because the set of information needed by the NF Consumer (CTF) to report the usage is incomplete or erroneous or not available. |
| CHARGING_NOT_APPLICABLE | 403 Forbidden | The HTTP request is rejected by the CHF since it has been determined that the service can be allowed to the end user without any charging or CDR creation. |
| USER_UNKNOWN | 404 Not Found | The HTTP request is rejected because the end user specified in the request cannot be served by the CHF. |
| END_USER REQUEST_DENIED | 403 Forbidden | The HTTP request denied by the CHF due to restrictions or limitations related to the end-user. |
| QUOTA_LIMIT_REACHED | 403 Forbidden | The HTTP request denied by the CHF because the end user's account could not cover the requested service. If the request contained used units they are deducted, if applicable. |
| END_USER_REQUEST_REJECTED | 403 Forbidden | The HTTP request rejected by the CHF due to end-user restrictions or limitations. |

Next change

##### 6.1.6.3.14	Enumeration: ResultCode

Table 6.1.6.3.14-1: Enumeration ResultCode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SUCCESS | The CHF grants the service to the end-user. This applies to the rating group. |  |
| END_USER_SERVICE_DENIED | The CHF denies the service request due to end-user service restrictions or limitations related to the end-user . If the request contained used units they shall be deducted, if applicable. This applies to the rating group. |  |
| QUOTA_MANAGEMENT_NOT_APPLICABLE | The CHF determines that the service can be granted to the end user without quota management,and used units shall be reported. This applies to the rating group. |  |
| QUOTA_LIMIT_REACHED | The CHF denies the service request since the end user's account could not cover the requested service. If the request contained used units they shall be deducted, if applicable. This applies to the rating group. |  |
| END_USER_SERVICE_REJECTED | The CHF denies the service request in order to terminate the service for which credit is requested. This applies to the rating group. |  |
| RATING_FAILED | The CHF determines that the service cannot be rated due to insufficient rating input, incorrect parameter combination or unrecognized parameter, or parameter value. This applies to the rating group. |  |

End of changes