3GPP TSG-SA5 Meeting #125 	S5-193388

# Newport Beach, USA,  8 – 12 April 2019								      		Revision of S5-193218

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0013 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add KPI for NG-RAN Handover Success Rate |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-03-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When a service is used it is important that it is not interrupted or aborted. One of the fault cases in a radio system for this is handovers/mobility. If a mobility KPI is not considered OK, then the network operator can investigate which steps that are required to improve the mobility towards their services. Thus, a KPI should be added for observing the impact of mobility in NG-RAN on end-users. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add KPI for NG-RAN Handover Success Rate and Mobility KPI use case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to monitor the success rate of handovers within NG-RAN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 6.X(new), 6.X.1(new), A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

# 4	End to end KPI concept and overview

The following KPI categories are included in the present document:

-	Accessibility (see the definition in [3]).

-	Integrity (see the definition in [3]).

-	Utilization.

-	Retainability (see the definition in [3]).

-	Mobility.

For future update of the document it will also include:

-	Availability.

Next change

## 6.x	Mobility KPI

### 6.x.1	NG-RAN Handover Success Rate

- NG-RAN Handover Success Rate

- A KPI that shows how often a handover within NR-RAN is successful, regardless if the handover was made due to bad coverage or any other reason.

- This KPI is obtained by successful handovers to the same or another gNB divided by attempted handovers to the same or another gNB.

- NGRANHOSR=(MM.HoExeInterSucc+MM.HoExeIntraSucc)(MM.HoExeInterReq+MM.HoExeIntraReq)×(MM.HoPrepInterSucc+MM.HoPrepIntraSucc)(MM.HoPrepInterReq+MM.HoPrepIntraReq)×100%

- MM.HoExeInterSucc, MM.HoExeIntraSucc, MM.HoExeInterReq, MM.HoExeIntraReq, MM.HoPrepInterSucc, MM.HoPrepIntraSucc, MM.HoPrepInterReq and MM.HoPrepIntraReq

- 5GS

- Mobility

- Percentage

- RATIO

Next change

A.x	Use case for Mobility KPI

When a service is used it is important that it is not interrupted or aborted. One of the fault cases in a radio system for this is handovers/mobility.

If a mobility KPI is not considered OK, then the network operator can investigate which steps that are required to improve the mobility towards their services.

This KPI should be used for observing the impact of mobility in NG-RAN on end-users.

End of changes