3GPP TSG-SA5 Meeting #125Adhoc 	S5-194132

# Sapporo, Japan 25-28 June 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0035 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add RESTful HTTP-based solution set for file-based performance assurance MnS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE |  |  |  |  |  | Date: |  |  | 2019-06-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In 3GPP TS 28.532(v 15.2.0), the IS of file-based performance assurance MnS has been defined. But the implementation for the IS solution is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new RESTful HTTP-based solution set for file-based performance assurance MnS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The file-based performance assurance MnS will be published without solution set. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Change

X. RESTful HTTP-based solution set of file-based performance assurance

X.1 Mapping of operation

X.1.1 Introduction

The IS operations are mapped to SS equivalents according to table X.1.1-1.

Table X.1.1-1: Mapping of IS operations to SS equivalents

| IS operations | HTTP Method | Resource URI | SQ |
| --- | --- | --- | --- |
| listAvailableFiles | GET | /files | M |
| subscribe | POST | /subscriptions |  |
| unsubscribe | DELETE | /subscriptions |  |
|  | DELETE | /subscriptions/{subscriptionId} |  |

X.1.2 Operation “listAvailableFiles”

This operation lists all or specified available management data files.

Table X.1.2-1: Mapping of IS operation input parameters to SS equivalents(HTTP GET)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| managementDataType | query | filter | filter-QueryType | M |
| beginTime | query | filter | filter-QueryType | M |
| endTime | query | filter | filter-QueryType | M |

Table X.1.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP GET)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| fileInfoList | response body | n/a | files-ResponseType | M |
| status | response status codes | n/a | n/a | M |

The message flow is as follows:

- The Service Consumer sends a HTTP GET request to the Service Provider.

The URI identifies the "…/files" collection resource.

The query part may contain filter parameter. Absence of the query part means all available management data files shall be returned.

The request message body shall be empty.

- The Service Provider sends a HTTP GET response to the Service Consumer.

On success "200 OK" shall be returned. The response message body shall carry the information of available files. The response format is defined by "files-ResponseType ".

On failure, an appropriate error code shall be returned. The response message body may carry an error object.

X.1.3 Operation "subscribe"

See sub-clause 9.1.8.

X.1.4 Operation "unSubscribe"

See sub-clause 9.1.9.

X.2 Mapping of notification

X.2.1 Introduction

The IS notifications are mapped to SS equivalents according to table X.2.1-1.

Table X.2.1-1: Mapping of IS notifications to SS equivalents

| IS operations | HTTP Method | Resource URI | SQ |
| --- | --- | --- | --- |
| notifyFileReady | POST | /notificationSink | M |
| notifyFilePreparationError | POST | /notificationSink | M |

X.2.2 Notification notifyFileReady

The IS notifications parameters are mapped to SS equivalents according to table X.2.2-1.

Table X.2.2-1: Mapping of IS notification parameters to SS equivalents

| IS notification parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| objectClass, objectInstance | request body | href | uri-Type | M |
| notificationId | request body | notificationId | notificationId-Type | M |
| notificationType | request body | notificationType | notificationType-Type | M |
| eventTime | request body | eventTime | dateTime-Type | M |
| fileInfoList | request body | fileInfoList | FileInfoList-Type | M |
| additionalText | request body | additionalText | additionalText-Type | O |

X.2.3 Notification notifyFilePreparationError

The IS notifications parameters are mapped to SS equivalents according to table X.2.3-1.

Table X.2.2-1: Mapping of IS notification parameters to SS equivalents

| IS notification parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| objectClass, objectInstance | request body | href | uri-Type | M |
| notificationId | request body | notificationId | notificationId-Type | M |
| notificationType | request body | notificationType | notificationType-Type | M |
| eventTime | request body | eventTime | dateTime-Type | M |
| fileInfoList | request body | fileInfoList | fileInfoList-Type | M |
| reason | request body | reason | reason-Type | M |
| additionalText | request body | additionalText | additionalText-Type | O |

X.3 Resource

X.3.1 Resource structure

Figure X.3.1-1 shows the resource structure of the performance assurance MnS. The "files" resource is a collection resource.

Figure X.3.1-1: Resource URI structure of the Performance assurance MnS

Table X.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table X.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| files | /files | GET | Retrieve the information about all or filtered available management data files |

X.3.2 Resource definitions

X.3.2.1 Resource “/files”

X.3.2.1.1 Description

This resource represents a collection of files.

X.3.2.1.2 URI

Resource URI: {DN_prefix_authority_part}/{DN_prefix_remainder}/PerformanceassuranceMnS/v1520/files

The resource URI variables a defined in the following table.

Table X.3.2.1.2-1: URI variables

| Name | Definition |
| --- | --- |
| DN_prefix_authority_part | See subclause 4.4 of TS 32.158 [15] |
| DN_prefix_remainder | See subclause 4.4 of TS 32.158 [15] |

X.3.2.1.3 HTTP methods

X.3.2.1.3.1 HTTP GET

This method shall support the URI query parameters specified in the following table.

Table X.3.2.1.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| filter | filter-QueryType |  | O |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table X.3.2.1.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | Description | Qualifier |
| --- | --- | --- |
| n/a | n/a | n/a |

Table X.3.2.1.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | Response codes | Description | Qualifier |
| --- | --- | --- | --- |
| files-ResponseType | 200 OK | The files returned. | M |
| error-Type | 4xx/5xx | Returned in case of an error | O |

X.3.2.2 Resource "/subscriptions"

X.3.2.2.1 Description

This resource is a container resource for individual subscriptions.

X.3.2.2.2	URI

The resource URI is:

Resource URI: {DN_prefix_authority_part}/{DN_prefix_remainder}/PerfMnS/v1520/subscriptions

X.3.2.2.2	HTTP methods

X.3.2.2.2.1	POST

This method shall support the URI query parameters specified in table X.3.2.2.2.1-1.

Table 8.2.1.3.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table X.3.2.2.2.1-2 and the response data structures and response codes specified in table X.3.2.2.2.1-3.

Table X.3.2.2.2.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| subscription-RequestType | Details of the subscription to be created | M |

Table X.3.2.2.2.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| subscription-ResponseType | 201 Created | In case of success the representation of the created subscription is returned. | M |
| error-Type | 4xx/5xx | In case of failure the error object is returned. | M |

X.3.2.2.2.2	DELETE

This method shall support the URI query parameters specified in table X.3.2.2.2.2-1.

Table X.3.2.2.2.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| consumerReferenceId | consumerReferenceId-QueryType | Identifies the consumer whose subscriptions shall be deleted | M |

This method shall support the request data structures specified in table X.3.2.2.2.2-2 and the response data structures and response codes specified in table X.3.2.2.2.2-3.

Table X.3.2.2.2.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| n/a | n/a | n/a |

Table X.3.2.2.2.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | n/a |
| error-Type | 4xx/5xx | In case of failure the error object is returned. | M |

X.3.2.3	Resource "/subscriptions /{subscriptionId}"

X.3.2.3.1 	Description

This resource represents a subscription.

X.3.2.3.2 	URI

The resource URI is:

Resource URI: {DN_prefix_authority_part}/{DN_prefix_remainder}/PerfMnS/v1520/ subscriptions/{subscriptionId}

Table 3.2.3.2-1: URI variables

| Name | Definition |
| --- | --- |
| DN_prefix_authority_part | See subclause 4.4 of TS 32.158 [15] |
| DN_prefix_remainder | See subclause 4.4 of TS 32.158 [15] |
| subscriptionId | Subscription identifier |

X.3.2.3.3 	HTTP methods

X.3.2.3.3.1	DELETE

This method shall support the URI query parameters specified in table X.3.2.3.3.1-1.

Table X.3.2.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table X.3.2.3.3.1-2 and the response data structures and response codes specified in table X.3.2.3.3.1-3.

Table X.3.2.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| n/a | n/a | n/a |

Table X.3.2.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| error-Type | 4xx/5xx | In case of failure the error object is returned. | M |

X.4 Data type definitions

X.4.1 General

Table X.4.1-1: Data types defined in the present document

| Data type | Reference | Description |
| --- | --- | --- |
| General types |  |  |
| uri-Type |  | Type of an URI |
| dateTime-Type |  | Date and time |
| Types in query parts |  |  |
| filter-QueryType | X.4.2.1 | Used in the query part of HTTP GET on /files to discriminate files to be returned |
| Types used in response bodies |  |  |
| files-ResponseType | X.4.2.2 | Used in the response body of HTTP GET on /files to return file information |
| Types used in notifications |  |  |
| notifyFileReady-NotifType | X.4.2.3 | Used in the request body of HTTP POST for the notification type notifyFileReady |
| notifyFilePreparationError-NotifType | X.4.2.4 | Used in the request body of HTTP POST for the notification type notifyPreparationError |
| Types referenced by the definitions above |  |  |
| notificationId-Type | X.4.4.2 | Notification identifier as defined in ITU-T Rec. X. 733 [4] |
| notificationType-Type | X.4.4.3 | Notification type (notifyFileReady, etc.) |
| reason-Type | X.4.4.2 | Used in the request body of HTTP POST for specifing the reason of the error occurred during the performance data file preparation |
| additionalText-Type | X.4.4.2 | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] |
| fileInfoList-Type | X.4.3.1 | Used in the request body of HTTP POST for specifing the information of each available file |

X.4.2 Query, message body

X.4.2.1 Type filter-QueryType

Table X.4.2.1-1: Definition of type filter-QueryType

| Type | Definition | Description |
| --- | --- | --- |
| filter-QueryType | string | Used in the query part of HTTP GET to filter scoped resource objects |

X.4.2.2 Type files-ResponseType

Table X.4.2.2-1: Definition of type files-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| fileInfoList | array(file-ResourceType) | Used in the request body of HTTP GET on /files to return file information | M |

X.4.2.3 Type notifyFileReady-NotifyType

Table X.4.2.3-1: Definition of type notifyFileReady-NotifyType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| header |  |  |  |
| > href | uri-Type | URI of the resource where the event (fileready) occurred | M |
| > notificationId | notificationId-Type | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| > notificationType | notificationType-Type | Notification type (notifyNewAlarm, etc.) | M |
| > eventTime | dateTime-Type | Event (alarm) occurrence time | M |
| body |  |  |  |
| > fileInfoList | fileInfoList-Type | Specifies the information of each available file. | M |
| > additionalText | additionalText-Type | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |

X.4.2.4 Type notifyFilePreparationError-NotifyType

Table X.4.2.3-1: Definition of type notifyFileReady-NotifyType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| header |  |  |  |
| > href | uri-Type | URI of the resource where the event (fileready) occurred | M |
| > notificationId | notificationId-Type | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| > notificationType | notificationType-Type | Notification type (notifyNewAlarm, etc.) | M |
| > eventTime | dateTime-Type | Event (alarm) occurrence time | M |
| body |  |  |  |
| > fileInfoList | fileInfoList-Type | Specifies the information of each available file. | M |
| > additionalText | additionalText-Type | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| > reason | reason-Type | Specifies the reason of the error occurred during the performance data file preparation. |  |

X.4.3 Referenced structured data types

X.4.3.1 Type fileInfoList-Type

Table X.4.3.1-1: Definition of type fileInfoList-Type

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| fileLocation | string | The location of the file | M |
| fileSize | integer | The size of the file |  |
| fileReadyTime | dateTime-Type | The date and time when the file was last closed and made available in the management service producer and the file content will not be changed. |  |
| fileExpirationTime | dateTime-Type | The date and time beyond which the file may be deleted. It shall not be empty and shall be later than fileReadyTime. |  |
| fileCompression | string | The name of the compression algorithm used for the file. |  |
| fileFormat | string | The encoding technique used by the file. |  |

X.4.3.2 Type header-Type

Table X.4.3.2-1: Definition of type header-Type

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| herf | uri-Type | URI of the resource where the event (alarm) occurred | M |
| notificationId | notificationId-Type | Notification identifier as defined in ITU-T Rec. X. 733 [4] | O |
| notificationType | notificationType-Type | Notification type (notifyNewAlarm, etc.) | M |
| eventTime | dateTime-Type | Event (alarm) occurrence time | M |

X.4.4 Simple Data types and enumerations

X.4.4.1 General

This subclause defines simple data types and enumerations that are used by the data structures defined in the previous subclauses.

X.4.4.2 simple data types

Table X.4.4.2: Simple data types

| Type name | Type definition | Description |
| --- | --- | --- |
| notificationId-Type | long | Notification identifier as defined in ITU-T Rec. X. 733 [4] |
| reason-Type | string | Reason of the error occurred during the performance data file preparation |
| additionalText-Type | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] |

X.4.4.3 Enumeration notificationType-type

TableX.4.4.3-1: Enumeration notificationType-Type

| Enumeration value | Description |
| --- | --- |
| notifyFileReady | Notification type is notifyFileReady |
| notifyFilePreparationError | Notification type is notifyFilePreparationError |

2nd Change

A.x Performance assurance service

{

"openapi": "3.0.1",

"info": {

"title": "TS 28.532 Perfoamance assurance Service",

"version": "15.2.0",

"description": "OAS 3.0.0 specification of the Perfoamance assurance Service"

},

"servers": [

{

"url": "http://{DN_prefix_authority_part}/{DN_prefix_remainder}/PerfMnS/v1500",

"variables": {

"DN_prefix_authority_part": {

"description": "See subclause 4.4 of TS 32.158",

"default": "example.com"

},

"DN_prefix_remainder": {

"description": "See subclause 4.4 of TS 32.158",

"default": ""

}

}

}

],

"paths": {

"/files": {

"get": {

"summary": "Retrieve files",

"description": "Retrieve the information about all or filtered available management data files.",

"parameters": [

{

"name": "managementDataType",

"in": "query",

"required": false,

"schema": {

"$ref": "#/components/schemas/filter-QueryType"

}

},

{

"name": "beginTime",

"in": "query",

"required": false,

"schema": {

"$ref": "#/components/schemas/filter-QueryType"

}

},

{

"name": "endTime",

"in": "query",

"required": false,

"schema": {

"$ref": "#/components/schemas/filter-QueryType"

}

}

],

"responses": {

"200": {

"description": "Success case (\"200 OK\"). Returns the file information identified in the request.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/files-ResponseType"

}

}

}

},

"default": {

"description": "Response in case of error.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

},

"/subscriptions": {

"post": {

"summary": "Create a subscription",

"description": "To create a subscription the representation of the subscription is POSTed on the /subscriptions collection resource.",

"requestBody": {

"required": true,

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/subscription-RequestType"

}

}

}

},

"responses": {

"201": {

"description": "Success case (\"201 Created\"). The representation of the newly created subscription resource shall be returned.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/subscription-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

},

"callbacks": {

"notifyFileReady": {

"{request.body#/consumerReference}": {

"post": {

"requestBody": {

"required": true,

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/notifyFileReady-NotifType"

}

}

}

},

"responses": {

"204": {

"description": "Success case (\"204 No Content\"). The notification is successfully delivered. The response message body is absent."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

}

},

"notifyFilePreparationError": {

"{request.body#/consumerReference}": {

"post": {

"requestBody": {

"required": true,

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/notifyFilePreparationError-NotifType"

}

}

}

},

"responses": {

"204": {

"description": "Success case (\"204 No Content\"). The notification is successfully delivered. The response message body is absent."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

}

},

"delete": {

"summary": "Delete all subscriptions made with a specific consumerReferenceId",

"description": "The subscriptions are deleted by deleting the corresponding subscription resources. The resources to be deleted are identified with the path component of the URI pointing to the /subscription collection resource and filtering on the consumerReferenceId provided in the query part.",

"parameters": [

{

"name": "consumerReferenceId",

"in": "query",

"description": "Identifies the subscriptions to be deleted.",

"required": true,

"schema": {

"$ref": "#/components/schemas/consumerReferenceId-QueryType"

}

}

],

"responses": {

"204": {

"description": "Success case (\"204 No Content\"). The subscription resources have been deleted. The response message body is absent."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

},

"/subscriptions/{subscriptionId}": {

"delete": {

"summary": "Delete a single subscription",

"description": "The subscription is deleted by deleting the corresponding subscription resource. The resource to be deleted is identified with the path component of the URI.",

"parameters": [

{

"name": "subscriptionId",

"in": "path",

"description": "Identifies the subscription to be deleted.",

"required": true,

"schema": {

"$ref": "#/components/schemas/subscriptionId-PathType"

}

}

],

"responses": {

"204": {

"description": "Success case (\"204 No Content\"). The subscription resource has been deleted. The response message body is absent."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

}

},

"components": {

"schemas": {

"dateTime-Type": {

"type": "string",

"format": "date-Time"

},

"long-Type": {

"type": "string",

"format": "long"

},

"uri-Type": {

"type": "string"

},

"error-ResponseType": {

"type": "object",

"properties": {

"error": {

"type": "object",

"properties": {

"errorInfo": {

"type": "string"

}

}

}

}

},

"properties": {

"uri": {

"$ref": "#/components/schemas/uri-Type"

},

"notificationId": {

"$ref": "#/components/schemas/notificationId-Type"

},

"notificationType": {

"$ref": "#/components/schemas/notificationType-Type"

},

"eventTime": {

"$ref": "#/components/schemas/dateTime-Type"

},

"filter-QueryType": {

"type": "string"

},

"files-ResponseType": {

"type": "object",

"properties": {

"data": {

"type": "array",

"items": {

"$ref": "#/components/schemas/files-ResourceType"

}

}

}

},

"subscriptionId-PathType": {

"type": "string"

},

"consumerReferenceId-QueryType": {

"$ref": "#/components/schemas/uri-Type"

},

"error-ResponseType": {

"type": "object",

"properties": {

"error": {

"type": "object",

"properties": {

"errorInfo": {

"type": "string"

}

}

}

}

},

"subscription-RequestType": {

"type": "object",

"properties": {

"data": {

"$ref": "#/components/schemas/subscription-ResourceType"

}

}

},

"subscription-ResponseType": {

"type": "object",

"properties": {

"data": {

"$ref": "#/components/schemas/subscription-ResourceType"

}

}

},

"notifyFileReady-NotifType": {

"type": "object",

"properties": {

"header": {

"$ref": "#/components/schemas/header-Type"

},

"body": {

"type": "object",

"properties": {

"fileInfoList": {

"$ref": "#/components/schemas/fileInfoList-Type"

},

"additionalText": {

"$ref": "#/components/schemas/additionalText-Type"

},

}

}

}

},

"notifyFilePreparationError-NotifType": {

"type": "object",

"properties": {

"header": {

"$ref": "#/components/schemas/header-Type"

},

"body": {

"type": "object",

"properties": {

"fileInfoList": {

"$ref": "#/components/schemas/fileInfoList-Type"

},

"additionalText": {

"$ref": "#/components/schemas/additionalText-Type"

},

"reason": {

"$ref": "#/components/schemas/reason-Type"

},

}

}

}

},

"notificationId-Type": {

"$ref": "#/components/schemas/long-Type"

},

"notificationType-Type": {

"type": "string",

"enum": [

"notifyFileReady",

"notifyFilePreparationError",

]

},

"reason-Type": {

"type": "string"

},

"additionalText-Type": {

"type": "string"

},

"fileInfoList-Type": {

"$ref": "#/components/schemas/fileInfoList-Type”

},

}

}

}

}

End of Change