3GPP TSG-SA5 Meeting #125Adhoc 	S5-194153

# Sapporo, Japan 25-28 June 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0111 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on nSIIdList stage 2 and stage 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the SA2 LS reply on the slicing terminology and the role of S-NSSAI parameter (S5-193254), SA2 clarified that - For the "S-NSSAI can be associated with one or more Network Slice instances", that can be used when there are more than one Network Slice instance supporting the same S-NSSAI that share the same AMF Set and resources in 5G-AN. In such case, the NSI ID can be used for identifying the other Core Network parts (e.g. SMF) of a Network Slice instance, if there is a need to differentiate between them in the 5GC. Note that the NSI ID is solely used within the 5GC (i.e. neither 5G-AN nor the UE gets the NSI ID). - SA2 agreed a CR (S2-1902846) clarifying some of the aspects SA5 asked to be clarified and sent that CR in the LS also. It is therefore proposed to correct nSIIdList which contains NSI ID to align with SA2 latest definition of NSI ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct nSIIdList which contains NSI ID to align with SA2 latest definition of NSI ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | nSIIdList which contains NSI ID does not align with definition of NSI ID in SA2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.10.3, 5.3.11.3, 5.4.1, F.4.3, G.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

5.3.10.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: network slicing feature is supported. |
| nfProfileList Support Qualifier | Condition: NF profile is registered and deregistered by management system. |
| nSIIdList Support Qualifier | Condition: Network slicing feature is supported and when multiple Network Slice instances of the same Network Slice are deployed, and there is a need to differentiate between them in the 5GC. |

Next change

5.3.11.3	Attribute constraints

| Name | Definition |
| --- | --- |
| nSIIdList Support Qualifier | Condition: Network slicing feature is supported and when multiple Network Slice instances of the same Network Slice are deployed, and there is a need to differentiate between them in the 5GC. |

Next change

5.4	Attribute definitions

5.4.1	Attribute properties

The following table defines the attributes that are present in several Information Object Classes (IOCs) of the present document.

| Attribute Name |  | Documentation and Allowed Values |  | Properties |
| --- | --- | --- | --- | --- |
| aMFIdentifier |  | The AMFI is constructed from an AMF Region ID, an AMF Set ID and an AMF Pointer. The AMF Region ID identifies the region, the AMF Set ID uniquely identifies the AMF Set within the AMF Region, and the AMF Pointer uniquely identifies the AMF within the AMF Set. (Ref. 3GPP TS 23.003 [13]) |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| aMFSetId |  | It represents the AMF Set ID, which is uniquely identifies the AMF Set within the AMF Region. allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 [13]. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| aMFSetMemberList |  | It is the list of DNs of AMFFunction instances of the AMFSet. allowedValues: N/A |  | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| aMFRegionId |  | It represents the AMF Region ID, which identifies the region. allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 [13]. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| localAddress | This parameter specifies the localAddress including IP address and VLAN ID used for initialization of the underlying transport. First string is IP address, IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). Second string is VLAN Id (See IEEE 802.1Q [39]). |  | type: String multiplicity: 2 isOrdered: True isUnique: N/A defaultValue: None isNullable: False |  |  |
| remoteAddress | Remote address including IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |
| nfProfileList |  | It is a set of NFProfile(s) to be registered in the NRF instance. NFProfile is defined in 3GPP TS 29.510 [23]. |  | type: <<dataType>> multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| nSIIdList |  | It is a set of NSI IDs. Definition of NSI ID see 3GPP TS 23.501 [2]. |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sNSSAIList |  | See subclause 4.4.1. |  |  |
| sBIFQDN |  | It is used to indicate the FQDN of the registered NF instance in service-based interface, for example, NF instance FQDN structure is: nftype<nfnum>.slicetype<sliceid>.mnc<MNC>.mcc<MCC>.3gppnetwork.org |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sBIServiceList |  | It is used to indicate the all supported NF services registered on service-based interface. |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| nRTACList |  | It is the list of Tracking Area Code (either legacy TAC or extended TAC) where the represented management function serving. allowedValues: Legacy TAC and Extended TAC are defined in clause 9.3.3.10 of TS 38.413 [5]. |  | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| weightFactor |  | The weight factor is typically set according to the capacity of local node (AMF) relative to other nodes in the same type. And it is used to achieve load balancing among a set of same type of network functions. (Ref. 3GPP TS 23.501 [2]) |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |

Next change

F.4.3	XML schema "ngcNrm.xsd"

<?xml version="1.0" encoding="UTF-8"?>

<!--

3GPP TS 28.541 5GC Network Resource Model

XML schema definition

ngcNrm.xsd

-->

<schema

targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

elementFormDefault="qualified"

attributeFormDefault="unqualified"

xmlns="http://www.w3.org/2001/XMLSchema"

xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"

xmlns:nn="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"     xmlns:en="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"

xmlns:ngc="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"/>

<!--NGC NRM IM class associated XML elements -->

<complexType name="aMFIdentifier">

<sequence>

<element name="amfRegionId" type="ngc:AmfRegionId"/>

<element name="amfSetId" type="ngc:AmfSetId"/>

<element name="amfPointer" type="ngc:AmfPointer"/>

</sequence>

</complexType>

<simpleType name="AmfRegionId">

<restriction base="integer">

<maxInclusive value="255"/>

<!-- The AMF Region ID is 8-bitslength, defined in 23.003 -->

</restriction>

</simpleType>

<simpleType name="AmfSetId">

<restriction base="integer">

<maxInclusive value="1023"/>

<!-- The AMF Region ID is 10-bits length, defined in 23.003 -->

</restriction>

</simpleType>

<simpleType name="AmfPointer">

<restriction base="integer">

<maxInclusive value="63"/>

<!-- The AMF Pointer is 6-bits length, defined in 23.003 -->

</restriction>

</simpleType>

<complexType name="NrTACList">

<sequence>

<element name="tac" type="nn:NrTac" minOccurs="0" maxOccurs="unbounded"/>

</sequence>

</complexType>

<complexType name="NFProfileList">

<sequence>

<element name="nfProfile" type="ngc:NfProfile"/>

</sequence>

</complexType>

<complexType name="NfProfile">

<sequence>

<element name="nfInstanceId" type="string"/>

<!-- nfInstanceId is uuid of NF instance -->

<element name="nfType" type="ngc:NfType"/>

<element name="nfStatus" type="ngc:NfStatus"/>

<element name="plmn" type="en:PLMNId"/>

<element name="nfType" type="ngc:NfType"/>

<element name="sNssais" type="ngc: SnssaiList"/>

<element name="fqdn" type="string"/>

<element name="interPlmnFqdn" type="string"/>

<element name="fqdn" type="string"/>

<element name="ipv4Addresses" type="string"/>

<element name="ipv6Addresses" type="string"/>

<element name="ipv6Prefixes" type="string"/>

<element name="capacity" type="string"/>

<element name="udrInfo" type="ngc:UdrInfo"/>

<element name="amfInfo" type="ngc:AmfInfo"/>

<element name="smfInfo" type="ngc:SmfInfo"/>

<element name="upfInfo" type="ngc:UpfInfo"/>

<element name="nfServices" type="ngc:NfServices"/>

</sequence>

</complexType>

<complexType name="NfServices">

<sequence>

<element name="serviceInstanceId" type="string"/>

<element name="serviceName" type="string"/>

<element name="version" type="string"/>

<element name="schema" type="string"/>

<element name="fqdn" type="string"/>

<element name="interPlmnFqdn" type="string"/>

<element name="ipEndPoints" type="ngc:IpEndpoints"/>

<element name="apiPrefix" type="string"/>

<element name="defaultNotificationSubscriptions" type="ngc:DefaultNotificationSubscriptions"/>

<element name="allowedPlmns" type="en:PLMNIdList"/>

<element name="allowedNfTypes" type="ngc:NFTypeList"/>

<element name="allowedNssais" type="ngc:Nssai"/>

<element name="capacity" type="string"/>

<element name="supportedFeatures" type="string"/>

</sequence>

</complexType>

<simpleType name="NfType">

<restriction base="string">

<!-- NF name is defined in TS 23.501 -->

<enumeration value="NRF"/>

<enumeration value="UDM"/>

<enumeration value="AMF"/>

<enumeration value="SMF"/>

<enumeration value="AUSF"/>

<enumeration value="NEF"/>

<enumeration value="PCF"/>

<enumeration value="SMSF"/>

<enumeration value="NSSF"/>

<enumeration value="UDR"/>

<enumeration value="LMF"/>

<enumeration value="GMLC"/>

<enumeration value="5GEIR"/>

<enumeration value="SEPP"/>

<enumeration value="UPF"/>

<enumeration value="N3IWF"/>

<enumeration value="AF"/>

<enumeration value="UDSF"/>

<enumeration value="DN"/>

</restriction>

</simpleType>

<complexType name="NFTypeList">

<sequence>

<element name="NFType" type="ngc:NfType"/>

</sequence>

</complexType>

<complexType name="IpEndpoints">

<sequence>

<element name="ipEndPoint" type="nn:IpEndPoint"/>

</sequence>

</complexType>

<complexType name="UdrInfo">

<sequence>

<element name="supiRange" type="ngc:SupiRange"/>

</sequence>

</complexType>

<complexType name="SupiRange">

<sequence>

<element name="start" type="string"/>

<element name="end" type="string"/>

<element name="pattern" type="string"/>

</sequence>

</complexType>

<complexType name="AmfInfo">

<sequence>

<element name="amfSetId" type="ngc:AmfSetId"/>

</sequence>

</complexType>

<complexType name="SmfInfo">

<sequence>

<element name="dnn" type="string"/>

</sequence>

</complexType>

<complexType name="UpfInfo">

<sequence>

<element name="snssaiUpfInfo" type="ngc:SnssaiUpfInfo"/>

</sequence>

</complexType>

<complexType name="SnssaiUpfInfo">

<sequence>

<element name="sNssai" type="ngc:SNssai"/>

<element name="dnnUpfInfoList" type="ngc:DnnUpfInfoList"/>

</sequence>

</complexType>

<complexType name="DnnUpfInfoList">

<sequence>

<element name="dnn" type="string"/>

</sequence>

</complexType>

<complexType name="DefaultNotificationSubscription">

<sequence>

<element name="notificationType" type="ngc:NotificationType"/>

<element name="callbackUri" type="string"/>

<element name="n1MessageClass" type="string"/>

<element name="n2InformationClass" type="string"/>

</sequence>

</complexType>

<simpleType name="NotificationType">

<restriction base="string">

<enumeration value="N1_MESSAGES"/>

<enumeration value="N2_INFORMATION"/>

<enumeration value="LOCATION_NOTIFICATION"/>

</restriction>

</simpleType>

<simpleType name="TransportProtocol">

<restriction base="string">

<enumeration value="TCP"/>

</restriction>

</simpleType>

<simpleType name="NfStatus">

<restriction base="string">

<enumeration value="REGISTERED"/>

<enumeration value="SUSPENDED"/>

</restriction>

</simpleType>

<complexType name="NfRegistrationData">

<sequence>

<element name="heartBeatTimer" type="integer"/>

<element name="nfProfile" type="ngc:NfProfile"/>

</sequence>

</complexType>

<complexType name="NSILdList">

<sequence>

<element name="nSIId" type="string"/>

</sequence>

</complexType>

<complexType name="SnssaiList">

<sequence>

<element name="sNssai" type="ngc:SNssai"/>

</sequence>

</complexType>

<complexType name="SNssai">

<sequence>

<element name="sst" type="ngc:Sst" minOccurs="0"/>

<element name="sd" type="ngc:Sd"/>

</sequence>

</complexType>

<simpleType name="Sst">

<restriction base="integer">

<maxInclusive value="255"/>

<!-- SST is 1-octets length and defined in TS 23.003 -->

</restriction>

</simpleType>

<simpleType name="Sd">

<restriction base="integer">

<maxInclusive value="65535"/>

<!-- SST is 2-octets length and defined in TS 23.003 -->

</restriction>

</simpleType>

<complexType name="SBIServiceList">

<sequence>

<element name="nfServices" type="ngc:NfServices"/>

</sequence>

</complexType>

<simpleType name="WeightFactor">

<restriction base="integer">

</restriction>

</simpleType>

<element name="AMFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="aMFIdentifier" type="ngc:aMFIdentifier"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="weightFactor" type="ngc:WeightFactor"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="aMFSet" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N2"/>

<element ref="ngc:EP_N8"/>

<element ref="ngc:EP_N11"/>

<element ref="ngc:EP_N12"/>

<element ref="ngc:EP_N14"/>

<element ref="ngc:EP_N15"/>

<element ref="ngc:EP_N17"/>

<element ref="ngc:EP_N22"/>

<element ref="ngc:EP_N26"/>

<element ref="ngc:EP_N20"/>

<element ref="ngc:EP_NLS"/>

<element ref="ngc:EP_NLG"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="SMFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="nRTACList" type="ngc:NrTACList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N4"/>

<element ref="ngc:EP_N10"/>

<element ref="ngc:EP_N11"/>

<element ref="ngc:EP_N7"/>

<element ref="ngc:EP_N16"/>

<element ref="ngc:EP_S5C"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="UPFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="nRTACList" type="ngc:NrTACList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N4"/>

<element ref="ngc:EP_N3"/>

<element ref="ngc:EP_N9"/>

<element ref="ngc:EP_S5U"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="ngc:EP_N6"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="N3IWFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N2"/>

<element ref="ngc:EP_N3"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="PCFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList" />

<element name="sBIFqdn" type="string" />

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N7"/>

<element ref="ngc:EP_N15"/>

<element ref="ngc:EP_N16"/>

<element ref="ngc:EP_N5"/>

<element ref="ngc:EP_Rx"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="AUSFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N12"/>

<element ref="ngc:EP_N13"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="UDMFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N8"/>

<element ref="ngc:EP_N10"/>

<element ref="ngc:EP_N13"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="UDRFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="UDSFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="nSIIdList" type="ngc:NSIIdList" minOccurs="0"/>

<element name="nFProfileList" type="ngc:NFProfileList" minOccurs="0"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N27"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NSSFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="nSIIdList" type="ngc:NSIIdList" minOccurs="0"/>

<element name="snssaiList" type="ngc: SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N27"/>

<element ref="ngc:EP_N31"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="SMSFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N20"/>

<element ref="ngc:EP_N21"/>

<element ref="ngc:EP_MAP_SMSC"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="LMFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_NLS"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NGEIRFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N17"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="SEPPFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N32"/>

<element ref="ngc:EP_SBI_IPX"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NWDAFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N2">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N3">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N4">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N5">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N6">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N7">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N8">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N9">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N10">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N11">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N12">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N13">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N14">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N15">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N16">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N17">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N20">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N21">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N22">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N26">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N27">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N31">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N32">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_SBI_X">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_SBI_IPX">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_S5C">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_S5U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_Rx">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_MAP_SMSC">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NLS">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NLG">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

</schema>

Next change

G.4.3	JSON schema "ngcNrm.json"

{

"$schema": "http://json-schema.org/draft-05/schema#",

"id": "http://3gpp.org/28541/ngcNrm.json",

"description": "JSON based solution set definitions for 5GC",

"_referenced_schema":"http://3gpp.org/28623/genericNrm.json",

"_referenced_schema":"http://3gpp.org/28626/stateManagementNrm.json",

"_referenced_schema":"http://3gpp.org/28541/nrNrm.json",

"definitions": {

"amfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"amfIdentifier": {

"$ref": "#/definitions/AmfIdentifier"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"weightFactor": {

"$ref": "#/definitions/WeightFactor"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

},

"amfSet": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

}

},

"required": [

"userLabel",

"plmnIdList",

"amfIdentifier",

"sBIFqdn",

"sBIServiceList",

"weightFactor"

]

},

"smfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"nRTACList": {

"$ref": "#/definitions/TACList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"nRTACList",

"sBIFqdn",

"sBIServiceList"

]

},

"upfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"nRTACList": {

"$ref": "#/definitions/TACList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"nRTACList"

]

},

"n3iwfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

}

},

"required": [

"userLabel",

"plmnIdList"

]

},

"pcfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"ausfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"udmFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"udrFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"udsfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"nrfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"nSIIdList": {

"$ref": "#/definitions/NSIIdList"

},

"nFProfileList": {

"$ref": "#/definitions/NFProfileList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn"

]

},

"nssfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"nSIIdList": {

"$ref": "#/definitions/NSIIdList"

},

"nFProfileList": {

"$ref": "#/definitions/NFProfileList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn"

]

},

"smsfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"lmfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

}

},

"required": [

"userLabel",

"plmnIdList"

]

},

"ngeirFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"seppFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

}

},

"required": [

"userLabel",

"plmnIdList",

]

},

"nwdafFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"eP_N2": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N3": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N4": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N5": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N6": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N7": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N8": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N9": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N10": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N11": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N12": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N13": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N14": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N15": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N16": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N17": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N20": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N21": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N22": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N26": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_SBI_X": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_SBI_IPX": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_S5C": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_S5U": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_Rx": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_MAP_SMSC": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_NLS": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_NLG": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"amfSet": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"nRTACList": {

"$ref": "#/definitions/TACList"

},

"amfSetId": {

"$ref": "#/definitions/AmfSetId"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"plmnIdList",

"nRTACList",

"amfSetId"

]

},

"amfRegion": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"nRTACList": {

"$ref": "#/definitions/TACList"

},

"amfRegionId": {

"$ref": "#/definitions/AmfRegionId"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"plmnIdList",

"nRTACList",

"amfRegionId"

]

},

"AmfIdentifier": {

"type": "object",

"description": "AmfIdentifier comprise of amfRegionId, amfSetId and amfPointer",

"properties": {

"amfRegionId": {

"$ref": "#/definitions/AmfRegionId"

},

"amfSetId": {

"$ref": "#/definitions/AmfSetId"

},

"amfPointer": {

"$ref": "#/definitions/AmfPointer"

}

}

},

"AmfRegionId": {

"type": "integer",

"description": "AmfRegionId is defined in TS 23.003",

"maximum": 255

},

"AmfSetId": {

"type": "string",

"description": "AmfSetId is defined in TS 23.003",

"maximum": 1023

},

"AmfPointer": {

"type": "integer",

"description": "AmfPointer is defined in TS 23.003",

"maximum": 63

},

"nFProfileList": {

"type": "array",

"description": "List of NF profile",

"items": {

"$ref": "#/definitions/NFProfile"

}

},

"NFProfile": {

"type": "object",

"description": "NF profile stored in NRF, defined in TS 29.510",

"properties": {

"nFInstanceId": {

"type": "string",

"description": "uuid of NF instance"

},

"nFType": {

"$ref": "#/definitions/NFType"

},

"nFStatus": {

"$ref": "#/definitions/NFStatus"

},

"plmn": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnId"

},

"sNssais": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/Snssai"

},

"fqdn": {

"$ref": "#/definitions/Fqdn"

},

"interPlmnFqdn": {

"$ref": "#/definitions/Fqdn"

},

"ipv4Addresses": {

"type": "array",

"item": {

"$ref": "#/definitions/Ipv4Addr"

}

},

"ipv6Addresses": {

"type": "array",

"item": {

"$ref": "#/definitions/Ipv6Addr"

}

},

"ipv6Prefixes": {

"type": "array",

"item": {

"$ref": "#/definitions/Ipv6Prefix"

}

},

"capacity": {

"$ref": "#/definitions/Capacity"

},

"udrInfo": {

"$ref": "#/definitions/UdrInfo"

},

"amfInfo": {

"$ref": "#/definitions/amfInfo"

},

"smfInfo": {

"$ref": "#/definitions/SmfInfo"

},

"upfInfo": {

"$ref": "#/definitions/UpfInfo"

},

"nfServices": {

"type": "array",

"item": {

"$ref": "#/definitions/NFService"

}

}

}

},

"NFService": {

"type": "object",

"description": "NF Service is defined in TS 29.510",

"properties": {

"serviceInstanceId": {

"type": "string",

},

"serviceName": {

"type": "string",

},

"version": {

"type": "string",

},

"schema": {

"type": "string",

},

"fqdn": {

"$ref": "#/definitions/Fqdn"

},

"interPlmnFqdn": {

"$ref": "#/definitions/Fqdn"

},

"ipEndPoints": {

"type": "array",

"item": {

"$ref": "#/definitions/IpEndPoint"

}

},

"apiPrfix": {

"type": "string",

},

"defaultNotificationSubscriptions": {

"type": "array",

"item": {

"$ref": "#/definitions/DefaultNotificationSubscriptions"

}

},

"allowedPlmns": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnId"

},

"allowedNfTypes": {

"type": "array",

"item": {

"$ref": "#/definitions/NFType"

}

},

"allowedNssais": {

"type": "array",

"item": {

"$ref": "#/definitions/Snssai"

}

},

"capacity": {

"$ref": "#/definitions/Capacity"

},

"supportedFeatures": {

"$ref": "#/definitions/SupportedFeatures"

}

}

},

"NFType": {

"type": "string",

"description": " NF name defined in TS 23.501",

"enum": [

"NRF",

"UDM",

"AMF",

"SMF",

"AUSF",

"NEF",

"PCF",

"SMSF",

"NSSF",

"UDR",

"LMF",

"GMLC",

"5G_EIR",

"SEPP",

"UPF",

"N3IWF",

"AF",

"UDSF",

"DN"

]

},

"Fqdn": {

"type": "string"

},

"IpEndPoint": {

"type": "object",

"properties": {

"ipv4Address": {

"$ref": "#/definitions/Ipv4Addr"

},

"ipv6Address": {

"$ref": "#/definitions/Ipv6Addr"

},

"ipv6Prefix": {

"$ref": "#/definitions/Ipv6Prefix"

},

"transport": {

"$ref": "#/definitions/TransportProtocol"

},

"port": {

"type": "integer"

}

}

},

"UdrInfo": {

"type": "object",

"properties": {

"supiRanges": {

"type": "array",

"items": {

"$ref": "#/definitions/SupiRange"

}

}

}

},

"SupiRange": {

"type": "object",

"properties": {

"start": {

"type": "string"

},

"end": {

"type": "string"

},

"pattern": {

"type": "string"

}

}

},

"AmfInfo": {

"type": "object",

"properties": {

"amfSetId": {

"$ref": "#/definitions/AmfSetId"

}

}

},

"SmfInfo": {

"type": "object",

"properties": {

"dnnList": {

"type": "array",

"items": {

"$ref": "#/definitions/Dnn"

}

}

}

},

"UpfInfo": {

"type": "object",

"properties": {

"sNssaiUpfInfoList": {

"type": "array",

"items": {

"$ref": "#/definitions/SnssaiUpfInfoItem"

}

}

}

},

"SnssaiUpfInfoItem": {

"type": "object",

"properties": {

"sNssai": {

"$ref": "#/definitions/Snssai"

},

"dnnUpfInfoList": {

"type": "array",

"items": {

"$ref": "#/definitions/DnnUpfInfoListInfo"

}

}

}

},

"DnnUpfInfoItem": {

"type": "object",

"properties": {

"dnn": {

"$ref": "#/definitions/Dnn"

}

}

},

"DefaultNotificationSubscription": {

"type": "object",

"properties": {

"notificationType": {

"$ref": "#/definitions/NotificationType"

},

"callbackUri": {

"$ref": "#/definitions/Uri"

},

"n1MessageClass": {

"$ref": "#/definitions/N1MessageClass"

},

"n2InformationClass": {

"$ref": "#/definitions/n2InformationClass"

}

}

},

"NotificationType": {

"type": "string",

"description": "any of enumrated value",

"enum": [

"N1_MESSAGES",

"N2_INFORMATION",

"LOCATION_NOTIFICATION"

]

},

"TranportProtocol": {

"type": "string",

"description": "any of enumrated value",

"enum": [

"TCP"

]

},

"NFStatus": {

"type": "string",

"description": "any of enumrated value",

"enum": [

"REGISTERED",

"SUSPENDED"

]

},

"NFRegistrationData": {

"type": "object",

"properties": {

"heartBeatTimer": {

"type": "integer"

},

"nfProfile": {

"$ref": "#/definitions/NFProfile"

}

}

},

"NSIIdList": {

"type": "array",

"items": {

"nSIId": {

"$ref": "#/definitions/NSIId"

}

}

},

"NSIId": {

"type": "string",

"description": "NSI Id is defined in TS 23.501"

},

"NrCellId": {

"type": "string",

},

"SBIFqdn": {

"$ref": "#/definitions/Fqdn"

},

"SBIServiceList": {

"type": "array",

"item": {

"$ref": "#/definitions/NFService"

}

},

"TACList": {

"type": "array",

"item": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/NrTac"

}

},

"weightFactor": {

"type": "integer"

}

}

}

End of changes