3GPP TSG-SA5 Meeting #125Adhoc 	S5-194181

# Sapporo, Japan 25-28 June 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.533 | CR | 0019 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add table of management capabilities offered by various management service instances |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GDMS |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | One of the objectives of 5GDMS work item is discovery of MnS enchancement. According to TS 28.533 a management service offers management capabilities that are accessed by management service consumers via standardized service interface composed of individually specified management service components. As part of discovery of MnS and exposure governance, MnS capability query is expected to be specified. Since it is unclear what information represents the MnS capability, this contribution provides a table with offered network slice management capabilities by various management service instance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding table of management capabilities offered by various management service instances |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of modified section in TS 28.533

4.3	Combination of management service components

A management service may contain the combination of the following:

1)	management service component type A and management service component type B

2)	management service component type A, management service component type B and management service component type C.

The instances of management services carry information about specified management service components in the metadata attributes. Figure 4.3.1.illustrates examples of management service instances with various management service components of type A, type B and type C:

Figure 4.3.1: Example of Management Service and component type A, B and C

For the same specified management service component type B, a management service instance can offer one or more management capability. The management service capability differentiator is the specified management service component type A or the combination of specified management service component type A and the specified management service component type C.  Figure 4.3.X.illustrates various MnS instance offering different MnS capabilities for a network slice.

| MnS instance | MnS component A | MnS component B | MnS component C | MnS capabilities |
| --- | --- | --- | --- | --- |
| Network slice provisioning | Operations defined in TS 28.532 | Network slice information model defined in TS 28.541 |  | createMOI for a network slice |
|  |  |  |  | deleteMOI for a network slice |
|  |  |  |  | getMOIAttributes for a network slice |
|  |  |  |  | modifyMOI for a network slice |
|  |  |  |  | allocate a network slice |
|  |  |  |  | deallocate a network slice |
| Network slice performance assurance | Operations defined in TS 28.550 | Network slice information model defined in TS 28.541 | Performance measurements and assurance data for network slice defined in TS 28.552 | createMeasurementJob for a network slice performance measurements and assurance data |
|  |  |  |  | stopMeasurementJob for a network slice performance measurements and assurance data |
|  |  |  |  | listMeasurementJobs for a network slice performance measurements and assurance data |
| Network slice performance data streaming | Operations defined in TS 28.550 | Network slice information model defined in TS 28.541 | Performance measurements and assurance data for network slice defined in TS 28.552 | establishStreamingConnection for a network slice performance measurements |
|  |  |  |  | terminateStreamingConnection for a network slice performance measurements |
|  |  |  |  | reportStreamData for a network slice performance measurements |
| Network slice performance data file reporting | Operations defined in TS 28.532 | Network slice information model defined in TS 28.541 | Performance measurements and assurance data for network slice defined in TS 28.552 | notifyFileReady for a network slice performance measurements and assurance data |
|  |  |  |  | notifyFilePreparationError for a network slice performance measurements and assurance data |
|  |  |  |  | listAvailableFiles for a network slice performance measurements and assurance data |
| Network slice fault supervision data report | Operations defined in TS 28.532 | Network slice information model defined in TS 28.541 | Alarm information defined in TS 28.532 | getAlarmList for a network slice alarm information |
|  |  |  |  | subscribe for a network slice alarm information |
|  |  |  |  | unsubscribe for a network slice alarm information |
|  |  |  |  | notifyNewAlarm for a network slice alarm information |
|  |  |  |  | notifyAlarmListRebuilt for a network slice alarm information |
|  |  |  |  | notifyChangedAlarm for a network slice alarm information |
| Network slice fault supervision control | Operations defined in TS 28.532 | Network slice information model defined in TS 28.541 | Alarm information defined in TS 28.532 | acknowledgeAlarms for a network slice alarm information |
|  |  |  |  | clearAlarms for a network slice alarm information |
|  |  |  |  | notifyAckStateChanged for a network slice alarm information |
|  |  |  |  | notifyClearedAlarm for a network slice alarm information |
|  |  |  |  | unacknowledgeAlarms for a network slice alarm information |

Figure 4.3.1: Different management capability offerings for a network slice

End of modification in TS 28.533