3GPP TSG-SA5 Meeting #0-Ad Hoc	S5-194204

# Sapporo, Japan, 25th Jun 2019 - 28th Jun 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.275 | CR | 0072 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of converged charging CDR handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSIMSCH |  |  |  |  |  | Date: |  |  | 2019-06-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding of convergent charging to MMTEL CDR handling |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clauses for convergent charging CDR handling |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MMTel cannot support convergent charging |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x3, 5.x.4, 5.x.5 (new after 5.3) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.x.3	CDR generation

#### 5.x.3.1	Introduction

The CHF CDRs for MMTel charging are generated by the CHF to collect charging information that they subsequently transfer to the Charging Gateway Function (CGF).

The following clauses describe in details the conditions for generating, opening, updating and closing the CHF CDR, which shall be supported by the CHF.

#### 5.x.3.2	Triggers for CHF CDR

##### 5.x.3.2.1	General

A MMTel charging CHF CDR is used to collect charging information related to MMTel chargeable events for IEC and SCUR.

##### 5.x.3.2.2	Triggers for CHF CDR generation

A CHF CDR is generated by the CHF for each received Charging Data Request [Event].

##### 5.x.3.2.4	Triggers for CHF CDR opening

A CHF CDR shall be opened when the CHF receives Charging Data Request [Initial].

##### 5.x.3.2.5	Triggers for CHF CDR update

A CHF CDR shall be opened when the CHF receives Charging Data Request [Update].

##### 5.x.3.2.6	Triggers for CHF CDR closure

The CHF CDR shall be closed when the CHF receives Charging Data Request [Termination].

### 5.x.4	Ga record transfer flows

Details of the Ga protocol application are specified in TS 32.295 [6].

### 5.x.5	Bi CDR file transfer

Details of the Bi protocol application are specified in TS 32.297 [5].

End of change