3GPP TSG-SA5 Meeting #0-Ad Hoc	S5-194207

# Sapporo, Japan, 25th Jun 2019 - 28th Jun 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.281 | CR | 0005 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of converged charging flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSIMSCH |  |  |  |  |  | Date: |  |  | 2019-06-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding of convergent charging principles for announcement service |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new text for convergent charging principles |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Announcement services cannot be support in convergent charging |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.x (new after 5.2.2) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.2.x	Message flows and types for convered charging

The following figures show the Charging Data transactions that are required between the IMS-GWF/AS and the CHF for session-related IMS procedures when the Announcement service is invoked.

The SIP messages and Charging Data transactions associated with these charging scenarios are shown primarily for general information and to illustrate the charging triggers. They are not intended to be exhaustive and they depend on the Charging Data Request triggers configured by the operator.

All of the scenarios depict originating sessions only.

# Scenario 1: Pre-resource usage announcement with successful session establishment

Figure 5.2.2.1 shows the Charging Data transactions that are required in the IMS-GWF/AS during a SIP session establishment for which a pre-resouce usage announcement is requested with successful session establishment.

Figure 5.2.2.1: Message sequence chart for pre-resource usage announcement with successful session establishment

1)	An initial SIP Invite Request is received in the S-CSCF. This request is forwarded to the IMS-GWF/AS.

2)	The IMS-GWF/AS sends a Charging Data Request [Initial] to the CHF requesting service units.

3)	The CHF sends a Charging Data Response [Initial] granting authorization and requests an announcement to be played to the calling party immediately. Announcement information may indicate whether the granted quota for the IMS session shall be used for playing the announcement.

3a)	The IMS-GWF/AS establishes a SIP session between the calling party and an MRFC/MRFP for the requested announcement as per TS 24.628 [244] and TS 24.229 [204].

3b)	The MRFP plays the announcement to the calling party.

3c)	When the announcement is complete, the IMS-GWF/AS disconnects the SIP session with the MRFC/MRFP as per TS 24.628 [244] and TS 24.229 [204].

4)	The IMS-GWF/AS and S-CSCF forward the initial SIP INVITE.

5)	A final response is received in the IMS-GWF/AS.

6)	If the trigger is active, the SIP 200 OK answer triggers a Charging Data Request [Update] in the IMS-GWF/AS with request for more unit for the service to continue and reporting the used units.

7)	The CHF sends the Charging Data Response [Update] to grant quota for the service to continue.

8)	The final answer is forwarded.
Scenario 2: Mid-resource usage announcement during IMS session

Figure 5.2.2.3 shows the Charging Data transactions that are required in the IMS-GWF/AS for a mid-resource usage announcement during an established IMS session, followed by session continuation.

Figure 5.2.2.3: Message sequence chart for mid-quota announcement during IMS session

1)	During an ongoing SIP session, a charging trigger encountered for an update as per table 5.3.1.1 in TS 32.260 [20]. The IMS-GWF/AS sends a Charging Data Request [Update] to the CHF requesting service units.

2)	The CHF sends a Charging Data Response [Update] granting quota and requests an announcement to be played to the calling party prior to quota exhaustion. Announcement information may indicate whether the granted quota for the IMS session shall be used for playing the announcement. The announcement is indicated as to be played to a specified duration before the end of the quota is reached.

2a)	When the timing conditions are met for the requeted announcement, the IMS-GWF/AS establishes a SIP session between the calling party and an MRFC/MRFP for the requested announcement as per TS 24.628 [244] and TS 24.229 [204].

2b)	The MRFP plays the announcement to the calling party.

2c)	When the announcement is complete, the IMS-GWF/AS re-connects the called party, disconnecting the SIP session with the MRFC/MRFP as per TS 24.628 [244] and TS 24.229 [204].

# Scenario 3: Post-resource usage announcement with IMS session termination

Figure 5.2.2.4 shows the Charging Data transactions that are required in the IMS-GWF/AS for a post-resource usage announcement which is initiated at final quota exhaust.

Figure 5.2.2.4: Message sequence chart for post-quota announcement with IMS session termination

1)	During an ongoing SIP session, a charging trigger encountered for an update as per table 5.3.1.1 in TS 32.260 [20]. The IMS-GWF/AS sends a Charging Data Request [Update] to the CHF requesting service units.

2)	The CHF sends a Charging Data Response [Update] granting quota and a final unit indication, with a request for an announcement to be played when the quota is exhausted. Granted quota can not be used for an announcement playback at final quota exhaust.

3)	When end of quota is reached, the IMS-GWF/AS initiates session termination to the called party by sending a SIP BYE message which is forwarded by the S-CSCF.

4)	The final answer to the SIP BYE message is forwarded by the S-CSCF to the IMS-GWF/AS ending the SIP dialog with the called party. The IMS-GWF/AS does not wait for this response before continuing the message sequence.

5a)	The IMS-GWF/AS establishes a SIP session between the calling party and an MRFC/MRFP for the requested announcement as per TS 24.628 [244] and TS 24.229 [204].

5b)	The MRFP plays the announcement to the calling party.

5c)	When the announcement is complete, the IMS-GWF/AS disconnects the SIP session with the MRFC/MRFP as per TS 24.628 [244] and TS 24.229 [204].

6)	The IMS-GWF/AS initiates session termination to the calling party by sending a SIP BYE message which is forwarded by the S-CSCF.

7)	The IMS-GWF/AS sends a Charging Data Request [Termination] reporting the used service units.

8)	The CHF responds with a Charging Data Response [Termintion] with the result of the request.

9)	The final answer to the SIP BYE message is forwarded by the S-CSCF to the IMS-GWF/AS ending the SIP dialog.

End of change