3GPP TSG-SA5 Meeting #125 AH 	S5-194310

# Sapporo, Japan,  25  – 28 Junel 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0124 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add datatype definition for NfService |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNRM |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the methodology (32.160) the datatypes have the definitions documented in separate clauses. There is no datatype for the NfService which is used as attribute in the NfProfile. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New sub-clause for datatype NfService is added, attribute definition table is updated accordingly |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no definition for datatype NfService. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.3.x(New), 5.3.x.1 (New), 5.3.x.2 (New), 5.3.x.3 (New), 5.3.x.4 (New), 5.3.y(New), 5.3.y.1 (New), 5.3.y.2 (New), 5.3.y.3 (New), 5.3.y.4 (New), 5.3.z(New), 5.3.z.1 (New), 5.3.z.2 (New), 5.3.z.3 (New), 5.3.z.4 (New), 5.3.k(New), 5.3.k.1 (New), 5.3.k.2 (New), 5.3.k.3 (New), 5.3.k.4 (New) 5.3.l(New), 5.3.l.1 (New), 5.3.l.2 (New), 5.3.l.3 (New), 5.3.l.4 (New), 5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First modified section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[6]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[7]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[8]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[9]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[10]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM);Stage 1".

[11]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[14]	3GPP TS 36.410: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 general aspects and principles".

[15]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol".

[16]	3GPP TS 36.425: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 interface user plane protocol".

[17]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS)".

[18]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

[19]	3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[20]	3GPP TS 28.702: "Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[21]	3GPP TS 28.708: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[22]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[23]	3GPP TS 29.510: "5G system; Network Function Repository Services; Stage 3".

[24]	3GPP TS 29.531: "5G System; Network Slice Selection Services Stage 3".

[25]	3GPP TS 32.152: "Telecommunication management; Integration Reference Point (IRP) Information Service (IS) Unified Modelling Language (UML) repertoire".

[26]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[27]	3GPP TS 28.554: "Management and orchestration; 5G End to end Key Performance Indicators (KPI)".

[28]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[29]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualisation (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[30]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[31]	Void.

[32]	3GPP TS 38.211: "NR; Physical channels and modulation".

[33]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[34]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[35]	3GPP TS 28.532: "Management and orchestration; Management services".

[36]	Void.

[37]	IETF RFC 791: "Internet Protocol".

[38]	IETF RFC 2373: "IP Version 6 Addressing Architecture".

[39]	IEEE 802.1Q: "Media Access Control Bridges and Virtual Bridged Local Area Networks".

[40]	ETSI GS NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV); Management and Orchestration; Report on NFV Information Model".

[X1]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[X2]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[X3]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax"

[X4]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

First modified section

# 5		Information Model definitions for 5GC NRM

## 5.1 	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.622 [30], IOC, ManagedElement | ManagedElement |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [30], IOC, EP_RP | EP_RP |
| TS 28.708 [21], IOC, ServingGWFunction | ServingGWFunction |
| TS 28.702 [20], IOC, SmsIwmscFunction | SmsIwmscFunction |
| TS 28.702 [20], IOC, SmsGmscFunction | SmsGmscFunction |
| TS 28.702 [20], IOC, GmlcFunction | GmlcFunction |
| TS 28.658 [19], attribute, pLmnIdList | pLMNIDList |

Second modified section

### 5.3.x	NfService <<dataType>>

#### 5.3.x.1	Definition

This datatype represents the properties of network function service related requirement that should be supported by the network function instance in a 5G network. NfService is defined in 3GPP TS 29.510 [23].

#### 5.3.x.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceInstanceID | M | T | T | T | T |
| serviceName | M | T | T | T | T |
| versions | M | T | F | T | T |
| scheme | M | T | T | F | T |
| nfServiceStatus | M | T | T | F | T |
| fqdn | O | T | T | F | T |
| interPlmnFqdn | O | T | T | F | T |
| ipEndPoints | O | T | T | F | T |
| apiPrefix | O | T | F | T | T |
| defaultNotificationSubscriptions | O | T | T | F | T |
| allowedPlmns | O | T | T | F | T |
| allowedNfTypes | O | T | T | F | T |
| allowedNfDomains | O | T | T | F | T |
| allowedNssais | O | T | T | F | T |
| priority | O | T | T | F | T |
| capacity | O | T | T | F | T |
| load | O | T | F | F | F |
| recoveryTime | O | T | T | F | T |
| chfServiceInfo | O | T | T | F | T |
| supportedFeatures | O | T | F | T | T |

#### 5.3.x.3	Attribute constraints

| Name | Definition |
| --- | --- |
| allowedPlmns Support Qualifier O | If this attribute is present in the NFService and in the NFProfile, the attribute from the NFService shall prevail. The absence of this attribute in the NFService and in the NFProfile indicates that there is no corresponding restriction to access the service instance. |

#### 5.3.x.4               Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.y	NfServiceVersion <<dataType>>

#### 5.3.y.1	Definition

This datatype represents the properties of Network Function Service Version and is defined in 3GPP TS 29.510 [23].

#### 5.3.y.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| apiVersionInUri | M | T | F | T | T |
| apiFullVersion | M | T | F | T | T |
| expiry | O | T | F | T | T |

#### 5.3.y.3	Attribute constraints

None

#### 5.3.y.4               Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.z	IpEndPoint <<dataType>>

#### 5.3.z.1	Definition

This datatype represents the properties of IpEndPoint and is defined in 3GPP TS 29.510 [23].

#### 5.3.z.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ipv4Address | CM | T | T | F | T |
| ipv6Address | CM | T | T | F | T |
| transport | O | T | T | F | T |
| port | O | T | T | F | T |

#### 5.3.z.3	Attribute constraints

| Name | Definition |
| --- | --- |
| ipv4Address Support Qualifier CM | At most one occurrence of either ipv4Address or ipv6Address shall be included in this data structure. |
| ipv6Address Support Qualifier CM | At most one occurrence of either ipv4Address or ipv6Address shall be included in this data structure. |
| port Support Qualifier O | If the port number is absent from the ipEndPoints attribute, the NF service consumer shall use the default HTTP port number, i.e. TCP port 80 for "http" URIs or TCP port 443 for "https" URIs |

#### 5.3.z.4               Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.k	DefaultNotificationSubscription <<dataType>>

#### 5.3.k.1	Definition

This datatype represents the properties of a notification end point and is defined in 3GPP TS 29.510 [23].

#### 5.3.k.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| notificationType | M | T | T | F | T |
| callbackUri | M | T | T | F | T |
| n1MessageClass | CM | T | T | F | T |
| n2InformationClass | CM | T | T | F | T |

#### 5.3.k.3	Attribute constraints

| Name | Definition |
| --- | --- |
| n1MessageClass Support Qualifier CM | If the notification type is N1_MESSAGES, this attribute shall be present and shall identify the class of N1 messages to be notified |
| n2InformationClass Support Qualifier CM | If the notification type is N2_INFORMATION, this attribute shall be present and shall identify the class of N2 information to be notified. |

#### 5.3.k.4               Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.l	Uri <<dataType>>

#### 5.3.l.1	Definition

This datatype represents a string providing an URI formatted according to IETF RFC 3986 [X3].

#### 5.3.l.2	Attributes

None

#### 5.3.l.3	Attribute constraints

None

#### 5.3.l.4               Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

Second modified section

## 5.4	Attribute definitions

### 5.4.1	Attribute properties

The following table defines the attributes that are present in several Information Object Classes (IOCs) of the present document.

| Attribute Name | Documentation and Allowed Values | Properties |  |
| --- | --- | --- | --- |
| aMFIdentifier | The AMFI is constructed from an AMF Region ID, an AMF Set ID and an AMF Pointer. The AMF Region ID identifies the region, the AMF Set ID uniquely identifies the AMF Set within the AMF Region, and the AMF Pointer uniquely identifies the AMF within the AMF Set. (Ref. 3GPP TS 23.003 [13]) | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| aMFSetId | It represents the AMF Set ID, which is uniquely identifies the AMF Set within the AMF Region. allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 [13]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| aMFSetMemberList | It is the list of DNs of AMFFunction instances of the AMFSet. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |  |
| aMFRegionId | It represents the AMF Region ID, which identifies the region. allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 [13]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| localAddress | This parameter specifies the localAddress including IP address and VLAN ID used for initialization of the underlying transport. First string is IP address, IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). Second string is VLAN Id (See IEEE 802.1Q [39]). | type: String multiplicity: 2 isOrdered: True isUnique: N/A defaultValue: None isNullable: False |
| remoteAddress | Remote address including IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nfProfileList | It is a set of NFProfile(s) to be registered in the NRF instance. NFProfile is defined in 3GPP TS 29.510 [23]. | type: <<dataType>> multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| nSIIdList | It is a set of NSI Id. The NSI ID is defined in subclause 6.1.6.2.8 of 3GPP TS 29.531 [24]. | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| sNSSAIList | See subclause 4.4.1. |  |  |
| sBIFQDN | It is used to indicate the FQDN of the registered NF instance in service-based interface, for example, NF instance FQDN structure is: nftype<nfnum>.slicetype<sliceid>.mnc<MNC>.mcc<MCC>.3gppnetwork.org | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| sBIServiceList | It is used to indicate the all supported NF services registered on service-based interface. | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| nRTACList | It is the list of Tracking Area Code (either legacy TAC or extended TAC) where the represented management function serving. allowedValues: Legacy TAC and Extended TAC are defined in clause 9.3.3.10 of TS 38.413 [5]. | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| weightFactor | The weight factor is typically set according to the capacity of local node (AMF) relative to other nodes in the same type. And it is used to achieve load balancing among a set of same type of network functions. (Ref. 3GPP TS 23.501 [2]) | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| serviceInstanceID | Unique ID of the service instance within a given NF Instance See attribute serviceInstanceID in TS 29.510 [23] subsection 6.1.6.2.3 | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| serviceName | Name of the service instance (e.g. "udm-sdm") See attribute serviceName in TS 29.510 [23] subsection 6.1.6.2.3. The allowed values for serviceName are specified in table 6.1.63.11-1 in subsection 6.1.6.3.11 . | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| versions | The API versions supported by the NF Service and if available, the corresponding retirement date of the NF Service. The different array elements shall have distinct unique values for "apiVersionInUri", and consequently, the values of "apiFullVersion" shall have a unique first digit version number. See attribute versions in TS 29.510 [23] subsection 6.1.6.2.31 | type: NFServiceVersion multiplicity: 1.*. isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| scheme | URI scheme (e.g. "http", "https") The allowed values for Urischeme are specified in TS 29.571 [X1], table 6.1.6.3.11-1 in clause 6.1.6.3.11 | type: UriScheme multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| nfServiceStatus | Status of the NF Service Instance See attribute nfServiceStatus in TS 29.510 [23] subsection 6.1.6.2.3 The allowed values for nfServiceStatus are specified in TS 29.510 [23], table 6.1.6.3.12-1 in clause 6.1.6.3.12 | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| ipEndPoints | IP address(es) and port information of the Network Function (including IPv4 and/or IPv6 address) where the service is listening for incoming service requests. See attribute ipEndPoints in TS 29.510 [23] subsection 6.1.6.2.3 | type: IpEndPoint multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| apiPrefix | Optional path segment(s) used to construct the {apiRoot} variable of the different API URIs, as described in 3GPP TS 29.501 [X2], subclause 4.4.1 (optional deployment-specific string that starts with a "/" character) See attribute apiPrefix in TS 29.510 [23] subsection 6.1.6.2.3 | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| defaultNotificationSubscriptions | Notification endpoints for different notification types. See attribute defaultNotificationSubscriptions in TS 29.510 [23] subsection 6.1.6.2.4 | type: DefaultNotificationSubscription multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| chfServiceInfo | Specific data for a CHF service instance See attribute chfServiceInfo in TS 29.510 [23] subsection 6.1.6.2.4 The ChfServiceInfo datatype is specified in TS 29.510 [23], table 6.1.6.2.33-1 in clause 6.1.6.2.33 | Type; String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| supportedFeatures | Supported Features of the NF Service instance See attribute supportedFeatures in TS 29.510 [23] subsection 6.1.6.2.4 The use of this attribute of type string is described in [X2] section 5.2.2 table 5.2.2-1 | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| ipv4Address | It represents the IP address(es) of the Network Function as aprt of the data type IpEndPoint. The ipV4Address is specified in TS 29.510 [23], table 6.1.6.2.5-1 in clause 6.1.6.2.5 | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |
| ipv6Address | It represents the IP address(es) of the Network Function as aprt of the data type IpEndPoint. The ipV6Address is specified in TS 29.510 [23], table 6.1.6.2.5-1 in clause 6.1.6.2.5 | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |

## 5.5	Common notifications

This subclause presents a list of notifications, defined in [35], that provisioning management service consumer can receive. The notification parameter objectClass/objectInstance, defined in [26], would capture the DN of an instance of an IOC defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOIAttributeValueChanges | O |  |
| notifyMOICreation | O |  |
| notifyMOIDeletion | O |  |

End of change