3GPP TSG-SA5 Meeting #125Adhoc 	S5-194411

# Sapporo, Japan 25-28 June 2019												Revision of S5-194159

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0094 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on kbits abbreviation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are some undefined kbits used in some measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Unify the use of kbit in the corresponding measurements and add a clear abbreviation of kbit. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Use of the undefined kbits may lead to interoperation issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.1.1.3.1, 5.1.1.3.2, 5.1.1.3.3, 5.1.1.3.4, 5.1.1.3.5, 5.1.1.3.6, 5.4.1.5, 5.4.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], 3GPP TS 23.501 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] and 3GPP TS 23.501 [4].

PI	Performance Indicator.

kbit	kilobit (1000 bits)

MN	Master Node.

NG-RAN	Next Generation Radio Access Network

NSI	Network Slice Instance

SN	Secondary Node.

Next change

5.1.1.3	UE throughput

5.1.1.3.1	Average DL UE throughput in gNB

a)	This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

b)	DER(N=1)

c)	This measurement is obtained according to the following formula based on the "ThpVolDl" and "ThpTimeDl" defined below. It is optionally split into subcounters for each QoS level.

If UEsThpTimeDl>0, UEsThpVolDlUEsThpTimeDl×1000 [kbit/s]

If UEsThpTimeDl=0, 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission,  , otherwise

| ThpTimeDl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeDl" for each time the DL buffer for one DataRadioBearer (DRB) is emptied. |
| --- | --- |
|  | The point in time after T2 when data up until the second last piece of data in the transmitted data burst which emptied the RLC SDU available for transmission for the particular DRB was successfully transmitted, as acknowledged by the UE. |
|  | The point in time when the first transmission begins after a RLC SDU becomes available for transmission, where previously no RLC SDUs were available for transmission for the particular DRB. |
|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbit successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer). |

d)	Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level measurement is perfomed, the number of measurements is equal to the number of mapped 5QIs.

e)	The measurement name has the form 
DRB.UEThpDl, or optionally DRB.UEThpDl.QOS, where QOS identifies the target quality of service class.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

5.1.1.3.2	Distribution of DL UE throughput in gNB

a)	This measurement provides the distribution of the UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers.

b)	CC

c)	Considering there are n samples during measurement time T and each sample has the same time period tn, the measurement of one sample is obtained by the following formula for a measurement period tn:

If UEsThpTimeDl>0, UEsThpVolDlUEsThpTimeDl×1000 [kbit/s]

If UEsThpTimeDl=0, 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission, , otherwise

| ThpTimeDl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeDl" for each time the DL buffer for one DataRadioBearer (DRB) is emptied. |
| --- | --- |
|  | The point in time after T2 when data up until the second last piece of data in the transmitted data burst which emptied the RLC SDU available for transmission for the particular DRB was successfully transmitted, as acknowledged by the UE. |
|  | The point in time when the first transmission begins after a RLC SDU becomes available for transmission, where previously no RLC SDUs were available for transmission for the particular DRB. |
|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbit successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer). |

Alternatively, for small data bursts, that are successfully transmitted in any given slot (i.e. the requirement that data bursts need to span across several slots excluding transmission of the last piece of the data in a data burst does not apply). where all buffered data is included in one initial HARQ transmission, fraction of the slot time (ThpTimeDL)ThpTimeUL) may be counted and obtained by the formula:

| slot | Duration of the slot |
| --- | --- |
| TBVol | Volume of the TB related to one slot burst |
| PaddingVol | Volume of padding bits added into Transport Block related to one slot burst. |

For each measurement sample, the bin corresponding to the DL throughput experienced by the UE is incremented by one.

d)	A set of integers, each representing the (integer) number of samples with a DL UE throughput in the range represented by that bin.

e)	The measurement name has the form 
DRB.UEThpDlDist.BinX where BinX represents the bin.

NOTE: Number of bins and the range for each bin is left to implementation

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

5.1.1.3.3	Average UL UE throughput in gNB

a)	This measurement provides the average UE throughput in uplink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

B)	DER(N=1)

c)	This measurement is obtained according to the following formula based on the "ThpVolUl" and "ThpTimeUl" defined below. It is optionally split into subcounters for each QoS level.

If UEsThpTimeUl>0, UEsThpVolUlUEsThpTimeUl×1000 [kbit/s]

If UEsThpTimeUl=0, 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission otherwise:

| ThpTimeUl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeUl" for each time the UL buffer for one DataRadioBearer (DRB) is emptied. |
| --- | --- |
|  | The point in time when the data up until the second last piece of data in data burst has been successfully received for a particular DRB |
|  | The point in time when transmission is started for the first data in data burst for a particular DRB. |
|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolUl is the data volume counted on RLC SDU level in kbit received in UL for one DRB during a sample of ThpTimeUl, (It shall exclude the volume of the last piece of data emptying the buffer). |

d)	Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level measurement is perfomed, the number of measurements is equal to the number of mapped 5QIs.

e)	The measurement name has the form 
DRB.UEThpUl, or optionally DRB.UEThpUl.QOS, where QOS identifies the target quality of service class.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

5.1.1.3.4	Distribution of UL UE throughput in gNB

a)	This measurement provides the distribution of the UE throughput in uplink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers.

b)	CC

c)	Considering there are n samples during measurement time T and each sample has the same time period tn, the measurement of one sample is obtained by the following formula for a measurement period tn:

If UEsThpTimeUl>0, UEsThpVolUlUEsThpTimeUl×1000 [kbit/s]

If UEsThpTimeUl=0, 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission otherwise:

| ThpTimeUl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeUl" for each time the UL buffer for one DataRadioBearer (DRB) is emptied. |
| --- | --- |
| T1 | The point in time when the data up until the second last piece of data in data burst has been successfully received for a particular DRB |
| T2 | The point in time when transmission is started for the first data in data burst for a particular DRB. |
| ThpVolUL | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolUl is the data volume counted on RLC SDU level in kbit received in UL for one DRB during a sample of ThpTimeUl, (It shall exclude the volume of the last piece of data emptying the buffer). |

Alternatively, for small data bursts, that are successfully transmitted in any given slot (i.e. the requirement that data bursts need to span across several slots excluding transmission of the last piece of the data in a data burst does not apply). where all buffered data is included in one initial HARQ transmission, fraction of the slot time (ThpTimeUL) may be counted and obtained by the formula:

| slot | Duration of the slot |
| --- | --- |
| TBVol | Volume of the TB related to one slot burst |
| PaddingVol | Volume of padding bits added into Transport Block related to one slot burst. |

For each measurement sample, the bin corresponding to the UL throughput experienced by the UE is incremented by one.

d)	A set of integers, each representing the (integer) number of samples with a UL UE throughput in the range represented by that bin.

e)	The measurement name has the form 
DRB.UEThpUlDist.BinX where BinX represents the bin.

NOTE: Number of bins and the range for each bin is left to implementation

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

5.1.1.3.5	Percentage of unrestricted DL UE data volume in gNB

a)	This measurement provides the percentage of DL data volume for UEs in the cell that is classified as unrestricted, i.e., when the volume is so low that all data can be transferred in one slot and no UE throughput sample could be calculated. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

b)	SI.

c)	For periods when no data is transferred at all Percentage Unrestricted Volume DL = 0, otherwise:

| ThpUnresVolDl | The volume of a data burst that is transmitted in the slot when the buffer is emptied (which could be the only slot needed to transmit the data burst) and not included in the UE throughput measurement. A sample for ThpUnresVolDl is the data volume counted on RLC SDU level in kbit sent in DL for one DRB. |
| --- | --- |
| ThpVolDl | The volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume counted on RLC SDU level in kbit sent in DL for one DRB. |

d)	Each measurement is a single integer value from 0 to 100. The number of measurements is equal to one. If the optional QoS level measurement is perfomed, the number of measurements is equal to the number of mapped 5QIs.

e)	The measurement name has the form 
DRB.UEUnresVolDl or optionally DRB.UEUnresVolDl.QOS, where QOS identifies the target quality of service class.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

5.1.1.3.6	Percentage of unrestricted UL UE data volume in gNB

a)	This measurement provides the percentage of UL data volume for UEs in the cell that is classified as unrestricted, i.e., when the volume is so low that all data can be transferred in one slot and no UE throughput sample could be calculated. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

b)	SI

c)	For periods when no data is transferred at all Percentage Unrestricted Volume UL = 0, otherwise:

| ThpUnresVolUl | The volume of a data burst that is transmitted in the slot when the buffer is emptied (which could be the only slot needed to transmit the data burst) and not included in the UE throughput measurement. A sample for ThpUnresVolUl is the data volume counted on RLC SDU level in kbit received in UL for one DRB. |
| --- | --- |
| ThpVolUl | The volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolUl is the data volume counted on RLC SDU level in kbit received in UL for one DRB. |

d)	Each measurement is a single integer value from 0 to 100. The number of measurements is equal to one. If the optional QoS level measurement is performed, the number of measurements is equal to the number of mapped 5QIs.

e)	The measurement name has the form 
DRB.UEUnresVolUl or optionally DRB.UEUnresVolUl.QOS, where QOS identifies the target quality of service class.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

Next change

5.4.1.5	Data volume of incoming GTP data packets per QoS level on the N3 interface, from (R)AN to UPF

a)	This measurement provides the data volume of the incoming GTP data packets per QoS level which have been accepted and processed by the GTP-U protocol entity on the N3 interface.  The measurement is calculated and split into subcounters per QoS level (5QI).

b)	CC.

c)	This measurement is obtained by counting the number of GTP PDU bits sent from GNB to UPF on the N3 interface. The measurement is performed per configured QoS level (5QI).

d)	Each measurement is an integer value representing the number of bits measured in kbit. The number of measurements is equal to the number of QoS levels.

e)	GTP.InDataVolumeQoSLevelN3UPF.

f)	EP_N3.

g)	Valid for packet switching.

h)	5GS.

5.4.1.6	Data volume of outgoing GTP data packets per QoS level on the N3 interface, from UPF to (R)AN

a)	This measurement provides the data volume of the outgoing GTP data packets per QoS level which have been generated by the GTP-U protocol entity on the N3 interface.  The measurement is calculated and split into subcounters per QoS level (5QI).

b)	CC.

c)	This measurement is obtained by counting the number of GTP PDU bits sent from UPF to GNB on the N3 interface. The measurement is performed per configured QoS level (5QI).

d)	Each measurement is an integer value representing the number of bits measured in kbit. The number of measurements is equal to the number of QoS levels.

e)	GTP.OutDataVolumeQoSLevelN3UPF

f)	EP_N3.

g)	Valid for packet switching.

h)	5GS.

End of changes