3GPP TSG-SA5 Meeting #125Adhoc 	S5-194536

# Sapporo, Japan 25-28 June 2019												Revision of S5-194422

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0095 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurement of SMF for N4 interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-06-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add measurement for N4 session modification and deletion based on current measurement KPIs for N4 interface in TS 28.552. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following measure KPIs for N4 are added: - N4 Session modifications; - N4 Session deletion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | N4 session modification and deletion are not covered by measurement in TS 28.552. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.x (new), 5.3.x.1, 5.3.x.2, 5.3.x.3, 5.3.x.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start change—measurement from SMF

5.3.x	Performance measurement for N4 interface

5.3.x.1			Number of N4 session modifications

a)    This measurement provides the number of attempted N4 session modifications.

b)    CC

c)    Transmission of "N4 Session Modification Request" message from SMF, this counter is cumulated by different N4 Session Modification Request messages sent by SMF as specified in TS 23.502 [7] and TS 29.244 [16].

d)    A single integer value.

e)    SM.N4SessionModify.

f)     SMFFunction

g)    Valid for packet switching

h)   5GS.

5.3.x.2			Number of failed N4 session modifications

a)    This measurement provides the number of failed N4 session modifications.

b)    CC

c)    Receipt of "N4 Session Modification Response" message with appropriate error cause value from UPF, SMF identifies a failed N4 session modification as defined in TS 23.502 [7] and TS 29.244 [16]. Each N4 Session Modification Request rejected increments the relevant counter by 1.

d)    A single integer value.

e)    SM.N4SessionModifyFail.Cause.

Where Cause identifies the reject cause of N4 session modification request, per the encoding of the cause specified in TS 29.244 [16].

f)     SMFFunction

g)    Valid for packet switching

h)   5GS.

5.3.x.3			Number of N4 session deletions

a)    This measurement provides the number of attempted N4 session deletions.

b)    CC

c)    Transmission of "N4 Session Deletion Request" message from SMF, this counter is cumulated by different N4 Session Deletion Request messages sent by SMF as specified in TS 23.502 [7] and TS 29.244 [16].

d)    A single integer value.

e)    SM.N4SessionDelete.

f)     SMF

g)    Valid for packet switching

h)   5GS.

5.3.x.4			Number of failed N4 session deletions

a)    This measurement provides the number of failed N4 session deletions.

b)    CC

c)    Receipt of "N4 Session Deletion Response" message with appropriate error cause value from UPF, SMF identifies a failed N4 session deletion as defined in TS 23.502 [7] and TS 29.244 [16]. Each N4 Session Deletion Request rejected increments the relevant counter by 1.

d)    A single integer value.

e)    SM.N4SessionDeleteFail.Cause.

Where Cause identifies the reject cause of N4 session deletion request, per the encoding of the cause specified in TS 29.244 [16].

f)     SMFFunction

g)    Valid for packet switching

h)   5GS.

End of change