3GPP TSG-SA5 Meeting #125 AH 	S5-194279

# Sapporo, Japan, 25 – 28 June 2019												Revision of S5-192xxx

# Source:	Ericsson

# Title:	Discussion paper around NR beam representation

# Document for:	Approval

Agenda Item:	4.3.10

# 1	Decision/action requested

The group is asked to discuss and endorse the proposal

# 2	References

[1]	3GPP TS 28.541 “5G Network Resource Model (NRM)”

[2]	3GPP TS 28.552 “Management and orchestration; 5G performance measurements”

[3]	3GPP TS 28.554 “Management and orchestration; 5G end to end Key Performance Indicators (KPI)”

[4]	3GPP TR 37.816 “Study on RAN-centric data collection and utilization”

[5]	3GPP TR 28.861 “Study on the SON for 5G networks”

[6]			S5-192312  “Rel-15 CR TS 28.541 Add NRM Info Model definitions for beam managed object class”

[7]			S5-192379  “R16 CR TS28.552 Add RSRP measurements”

[8]			S5-192380  “R16 CR TS 28.552 Add UE Rx-Tx time difference related measurements”

[9]			S5-193422 (revision of S5-193210) Discussion paper around beam types

[10]		S5-193401 (revision of S5-193210) Rel-15 CR TS 28.541 Add NRM Info Model definitions for

beam managed object class

[11]	28.627:	“Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration Reference Point (IRP): Requirements”

[12]	28.628:	“Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration ReferencePoint (IRP): Information Service (IS)”

[13]	28.629:	“Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration Reference Point (IRP): Solution Set (SS) definitions”

[14]	RP-182863 Revised WID on NR MIMO Enh

# 3	Rationale

The representation of “NR beam” has previous been discussed in two contributions:

- The “NR beam” is a representation of NRCellDU [10]

- The “NR beam” is a representation of NRSectorCarrier [9]

This discussion paper proposes the latter option to be selected as a baseline for the 3GPP NR NRM.

From 3GPP TS 28.540, the following requirements apply to NG-RAN:

REQ-NGRAN_NRM-CON-007: The NRM definitions shall have a read-only representation of NR beam properties in NG-RAN.

In last SA5 #125 meeting proposal 1 was agreed [9]: the ‘NR beam’ should not be a logical reference to reference signals that are dynamic and/or UE specifically configured.

Observation 1: Beams carring the SSB common reference signals shall have a read-only representation of NR beam properties in NG-RAN.

The WID Enhancements on MIMO for NR [14], NR_eMIMO for Rel-16, enhances the limited support for multi-TRP/panel operation in Rel-15. Rel-15 NR MIMO provisionally accommodates multi-TRP/panel operation, but the supported features are limited to standard-transparent transmission operations and small number of TRPs/panels. The agreement in the ongoing WI is that both inter- and intra-cell operation shall be covered.”

Thus, the NG-RAN support multi-antenna transmission to provide good performance and coverage on different carrier layers. A transmission point (TRP) may serve one or more cells, where also a cell may be served by one or more transmission points. The cells need not to have the same coverage. A transmission point may be deployed using an advanced antenna sytem (AAS).

Observation 2: A cell may be served by multiple antennas at different transmission points, where an antenna may serve multiple cells.

Figure 1	Example a DU with two cells on different carriers being served by three TRPs each, where each TRP is deployed using an antenna.

The NRSectorCarrier in the NRM below models the resource utilization of the SectorEquipmentFunction, where the SectorEquipmentFunction represents the antenna equipment at the TRP. The NRsectorCarrier is associated with at most one NRCellDU. Thus, the cell related transmission and reception on a TRP is represented by the NRSectorCarrier.

Figure 2	Current NRM (Figure 4.2.1.1-3 in [1])

Observation 3: The NRSectorCarrier models the antenna resoures used for cell specific transmission and reception.

Reference [10] proposes a Beam IOC with properties representing

- The index of a beam

- The beam type

- The azimuth of a beam transmission

- The tilt of a beam transmission

- The horizontal beam width of a beam transmission

- The vertical beam width of a beam transmission

Observation 4: The beam type is not needed to characterize beams limited to beams carrying SSB common reference signals

The SON function Coverage & Capacity Optimization (CCO) is envisioned to use configuration capabilities of the antenna system. “Study on the Self-Organizing Networks (SON) for 5G networks” in TR 28.861 is still ongoing and has not concluded on requirements on configuration capabilitites on the antenna system.

Observation 5: No normative requirements exist on capabilitites for configuration of an antenna systems.

Proposal 1:

A CommonBeamforming IOC representing the beams carrying the SSB common reference signal is proposed contained under the NRSectorCarrier. The IOC provides a read-only list of information on the properties of the associated SSB beams. Related capabilitites for configuration will be addressed when normative requirements exist.

Figure 3	Proposed CommonBeamforming IOC

This <<IOC>>CommonBeamforming represents the information required by the NRSectorCarrier that is responsible for the management of SSB beam resources. Measurements on static, common Synchronization Signal Block (SSB) beams may be correlated with associated spatial beam information to assist use cases like troubleshooting performance problems, or SON functions like Coverage & Capactity Optimization. 3GPP TS 38.104 relates to SSB transmission, TS 38.215 to SSB beam measurements, and TS TS 38.331 to reporting of those measurements.

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ssbBeamList | O | T | F | F | T |

The attribute ssbBeamList can be defined as a <<datatype>> ssbBeam that is a set of attributes defining the spatial information for each SSB beam. Through these attributes, the coverage area and radiation pattern of the associated SSB transmissions are represented.  There may be more than one SSB beam per NRSectorCarrier in an NRCellDU.

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ssbBeamIndex | M | T | F | F | T |
| ssbBeamPan | M | T | F | F | T |
| ssbBeamTilt | M | T | F | F | T |
| ssbBeamWidthPan | CM | T | F | F | T |
| ssbBeamWidthTilt | CM | T | F | F | T |

CM means, if provided by the equipment.

# 4	Detailed proposal

Based on the observations above, we ask for Endorsement for Proposal 1.

Proposal 1: A CommonBeamforming IOC representing the beams carrying the SSB common reference signal is proposed contained under the NRSectorCarrier. The IOC provides a read-only list of information on the properties of the associated SSB beams. Related capabilitites for configuration will be addressed when normative requirements exist.