3GPP TSG-SA5 Meeting #125Adhoc 	S5-194419

# Sapporo, Japan 25-28 June 2019									   Revision of S5-194396, S5-194086

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0017 | rev | 2 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add a new KPI definition of PDU session Establishment Success Rate of one NSI identifier (S-NSSAI) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-06-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is one network performance measurement that useful for the 5G network management but not defined in TS 28.554. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new definition of PDU session Establishment Success Rate of one NSI identifier (S-NSSAI). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | None |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of Change

### 6.2.X PDU session Establishment Success Rate of one NSI identifier (S-NSSAI)

- PDU session establishment success rate of one NSI identifier (S-NSSAI).

- This KPI describes the ratio of the number of successful PDU session establishment request to the number of PDU session establishment request attempts for 5G network for the SMF which related to one NSI identifier (S-NSSAI) and is used to evaluate accessibility provided by the end-to-end network slice and network performance.

- This KPI is obtained by the number of successful PDU session requests divided by the number of attempted PDU session requests.

- SM.PduSessionCreationSuccNSI.SNSSAI 
SM.PduSessionCreationReqNSI.SNSSAI

- 5G System

- Accessibility.

- Percentage.

- RATIO.

# A.X	Use case for PDU session Establishment success rate of one NSI identifier (S-NSSAI) related KPI

It is necessary to evaluate accessibility performance provided by 5GS. PDU session Establishment for a UE is important when it has registered to the network slice. If users or subscribers cannot establish PDU sessions in slice instance, they cannot access any network services in the network slice. This KPI is focusing on network view.

# .

End of Change