3GPP TSG-SA5 Meeting #126	S5-195660

# Bruges,Belgium, 19-23 August 2019 												        revision of S5-195180

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0111 | rev | 2 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Power, Energy and Environmental (PEE) measurements and related use case description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, ORANGE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EE_5G |  |  |  |  |  | Date: |  |  | 2019-08-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In order to be able to calculate the energy efficiency of 5G networks, it is required to collect Power, Energy and Environmental (PEE) parameters of 5G Physical Network Functions (PNFs). |  |  |  |  |  |  |  |  |
| Summary of change: |  | New performance measurements are introduced, related to Power, Energy and Environmental (PEE) parameters of 5G physical network functions, at ManagedElement level . A corresponding use case description is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It would not possible to collect Power, Energy and Environmental (PEE) parameters of 5G physical network functions and, hence, to calculate their energy efficiency. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.4, 5.1.1.x (new), A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[3]	3GPP TS 32.404: "Performance Management (PM); Performance measurements - Definitions and template".

[4]	3GPP TS 23.501: "System Architecture for the 5G System".

[5]	IETF RFC 5136: "Defining Network Capacity".

[6]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

[7]	3GPP TS 23.502: "Procedures for the 5G System".

[8]	3GPP TS 28.554: “Management and orchestration; 5G end to end Key Performance Indicators (KPI)”.

[9]	3GPP TS 32.425: “Performance Management (PM); Performance measurements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN)”

[10]	3GPP TS 32.451: "Key Performance Indicators (KPI) for Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Requirements”

[x]	ETSI ES 202 336-12 V1.2.1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[y]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

Next change

## 3.4	Measurement family

The measurement names defined in the present document are all beginning with a prefix containing the measurement family name. This family name identifies all measurements which relate to a given functionality and it may be used for measurement administration.

The list of families currently used in the present document is as follows:

-	DRB (measurements related to Data Radio Bearer).

-	RRC (measurements related to Radio Resource Control).

-	UECNTX (measurements related to UE Context).

-	RRU (measurements related to Radio Resource Utilization).

-	RM (measurements related to Registration Management).

-	SM (measurements related to Session Management).

-	GTP (measurements related to GTP Management).

-	IP (measurements related to IP Management).

-	PA (measurements related to Policy Association).

-	MM (measurements related to Mobility Management).

-	VR (measurements related to Virtualized Resource).

-	CARR (measurements related to Carrier).

-	QF (measurements related to QoS Flow).

-	PEE (measurements related to Power, Energy and Environment).

Next change

## 5.1	Performance measurements for gNB

### 5.1.1	Performance measurements valid for all gNB deployment scenarios

#### 5.1.1.x	Power, Energy and Environmental (PEE) measurements

##### 5.1.1.x.1	Applicability of measurements

The PEE related measurements defined here are valid for a 5G Physical Network Function (PNF). The NR NRM is defined in TS 28.541 [y].

##### 5.1.1.x.2	PNF Power Consumption

###### 5.1.1.x.2.1	Average Power

a)	This measurement provides the average power consumed over the measurement period.

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [x] – clauses 4.4.3.1, 4.4.3.4, Annex A.

d)	A real value in watts (W).

e)	The measurement name has the form PEE.AvgPower

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

###### 5.1.1.x.2.2	Minimum Power

a)	This measurement provides the minimum power consumed during the measurement period

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [x] – clauses 4.4.3.1, 4.4.3.4, Annex A.

d)	A real value in watts (W).

e)	The measurement name has the form PEE.MinPower

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

###### 5.1.1.x.2.3	Maximum Power

a)	This measurement provides the maximum power consumed during the measurement period.

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [x] – clauses 4.4.3.1, 4.4.3.4, Annex A.

d)	A real value in watts (W).

e)	The measurement name has the form PEE.MaxPower

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.x.3	PNF Energy consumption

a)	This measurement provides the energy consumed.

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [x] – clauses 4.4.3.1, 4.4.3.4, Annex A.

d)	A real value in kilowatt-hours (kWh).

e)	The measurement name has the form PEE.Energy

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.x.4	PNF Temperature

###### 5.1.1.x.4.1	Average Temperature

a)	This measurement provides the average temperature over the measurement period.

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [x] – clause 4.4.3.4, Annex A.

d)	A real value in degrees Celsius (°C).

e)	The measurement name has the form PEE.AvgTemperature

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

###### 5.1.1.x.4.2	Minimum Temperature

a)	This measurement provides the minimum temperature during the measurement period.

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [x] – clause 4.4.3.4, Annex A.

d)	A real value in degrees Celsius (°C).

e)	The measurement name has the form PEE.MinTemperature

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

###### 5.1.1.x.4.3	Maximum Temperature

a)	This measurement provides the maximum temperature during the measurement period.

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [x] – clause 4.4.3.4, Annex A.

d)	A real value in degrees Celsius (°C).

e)	The measurement name has the form PEE.MaxTemperature

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.x.5	PNF Voltage

a)	This measurement provides the voltage.

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [x] – Clauses 4.4.3.3, 4.4.3.4, Annex B.

d)	A real value in volts (V).

e)	The measurement name has the form PEE.Voltage.

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.x.6	 PNF Current

a)	This measurement provides the current.

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [x] – Clauses 4.4.3.3, 4.4.3.4, Annex B.

d)	A real value in amperes (A).

e)	The measurement name has the form PEE.Current.

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.x.7	PNF Humidity

a)	This measurement provides the percentage of humidity during the measurement period

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [x] – clause 4.4.3.3, Annex B.

d)	An integer value from 0 to 100.

e)	The measurement name has the form PEE.Humidity.

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

Next change

Annex A (informative):
Use cases for performance measurements

# A.x	Monitoring of Power, Energy and Environmental (PEE) parameters

Power, Energy and Environmental (PEE) parameters, combined with data volume measurements, are valuable information for operators to measure the energy efficiency of their 5G network. Hence it is necessary to define PEE parameters related to 5G network such as power, energy, temperature, voltage, current and humidity.

End of changes