3GPP TSG-SA5 Meeting #126	S5-195133

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.297 | CR | 0036 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of reference architecture in 5G |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SBI_CH |  |  |  |  |  | Date: |  |  | 2019-08-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The scope and architecture description for 5G are missing. TS 32.254 is missing in TS number field |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend the scope to include 5G and introduce 5G charging architecture Add TS 32.254 in TS number field |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to transfer 5G CDRs to BD |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 4, 5.1.1, 6.1.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 1	Scope

The present document is part of a series of Technical Specifications (TSs) that specify charging functionality and charging management in 3GPP networks. The 3GPP core network charging architecture and principles are specified in TS 32.240 [1], which provides an umbrella for other charging management TSs that specify:

- 	the content of the CDRs per domain / subsystem / service (offline and converged charging);

- 	the content of real-time charging messages per domain / subsystem / service (converged charging);

- 	the functionality of offline and converged charging for those domains / subsystems / services;

- 	the interfaces that are used in the charging framework to transfer the charging information (i.e. CDRs or charging events)

The complete document structure for these TSs is defined in TS 32.240 [1].

The present document specifies the 
file based mechanism used to transfer the CDRs from the network to the operator's billing domain (e.g. the post-processing system or a mediation device).

The present document is related to other 3GPP charging TSs as follows:

-	The common 3GPP charging architecture is specified in TS 32.240 [1];

-	The parameters, abstract syntax and encoding rules for the CDRs are specified in TS 32.298 [51];

-	A transaction based mechanism for the transfer of CDRs within the network is specified in TS 32.295 [54].

-	The 3GPP Diameter application that is used for offline and online charging is specified in TS 32.299 [50].

-	The services, operations and procedures of charging, using Service Based Interface are specified in TS 32.290 [57].

-	The charging service of 5G system is specified in TS 32.291 [58].

All terms, definitions and abbreviations used in the present document, that are common across 3GPP TSs, are defined in the 3GPP Vocabulary, TR 21.905 [100]. Those that are common across charging management in 3GPP domains services, or subsystems are provided in the umbrella document TS 32.240 [1] and are copied into clause 3 of the present document for ease of reading. Finally, those items that are specific to the present document are defined exclusively in the present document.

Furthermore, requirements that govern the charging work are specified in TS 22.115 [101].

Next change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[2] - [9]	Void.

[10]	3GPP TS 32.250: "Telecommunication management; Charging management; Circuit Switched (CS) domain charging".

[11]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

[12]	Void.

[13]	3GPP TS 32.253: "Telecommunication management; Charging management; Control Plane (CP) data transfer domain charging".

[14]	3GPP TS 32.254: "Telecommunication management; Charging management; Exposure function Northbound Application Program Interfaces (APIs) charging".

[15]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G Data connectivity domain charging; stage 2".

[16] - [19]	Void

[20]	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

[21] - [29]	Void.

[30]	3GPP TS 32.270: "Telecommunication management; Charging management; Multimedia Messaging Service (MMS) charging".

[31]	3GPP TS 32.271: "Telecommunication management; Charging management; Location Services (LCS) charging".

[32]	3GPP TS 32.272: "Telecommunication management; Charging management; Push-to-talk over Cellular (PoC) charging".

[33]	3GPP TS 32.273: "Telecommunication management; Charging management; Multimedia Broadcast and Multicast Service (MBMS) charging".

[34]	3GPP TS 32.274: "Telecommunication management; Charging management; Short Message Service (SMS) charging".

[35]	3GPP TS 32.275: "Telecommunication management; Charging management; Multimedia Telephony (MMTel) charging".

[36]	Void.

[37]	3GPP TS 32.277: "Telecommunication management; Charging management; Proximity-based Services (ProSe) Charging".

[38]	3GPP TS 32.278: "Telecommunication management; Charging management; Monitoring Event charging".

[39] - [49] 	Void.

[50]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application".

[51]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) encoding rules description".

[52]	Void.

[53]	3GPP TS 32.296: "Telecommunication management; Charging management; Online Charging System (OCS) applications and interfaces".

[54]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[55] - [56]	Void.

[57]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[58]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[59] - [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101]	3GPP TS 22.115: "Service aspects; Charging and billing".

[102] - [199]	Void.

[200]	3GPP TS 23.078: "Customised Applications for Mobile network Enhanced Logic (CAMEL); Stage 2".

[201] 	3GPP TS 32.341: "Telecommunication management; File Transfer (FT) Integration Reference Point (IRP); Requirements".

[202]	3GPP TS 32.342: "Telecommunication management; File Transfer (FT) Integration Reference Point (IRP); Information Service (IS)".

[203]	3GPP TS 32.343: "Telecommunication management; File Transfer (FT) Integration Reference Point (IRP); Common Object Request Broker Architecture (CORBA) Solution Set (SS)".

[204]	3GPP TS 32.344: "Telecommunication management; File Transfer (FT) Integration Reference Point (IRP); Common Management Information Protocol (CMIP) Solution Set (SS)".

[205] - [399]	Void

[400]	IETF RFC 959 (1985): "File Transfer Protocol".

[401]	ATIS-PP-0300075.1.200X "Usage Data Management for Packet-Based Services - Service Neutral Protocol Specification for Billing Applications"

[402]	IPDR "IPDR/File Transfer Protocol".

[403]	IPDR "IPDR/SP Protocol Specification".

Next change

# 4	Architecture considerations

"Bx" is a common designator for the reference points from the network to the Billing Domain (BD) that are intended for the transport of CDR files. The letter "x" indicates the different 3GPP network domain, subsystem or service, where

c	represents Circuit Switched (CS) in TS 32.250 [10],

cp	represents Control Plane data transfer (CP) in TS 32.253 [13],

d	represents 5G Data Connectivity in TS 32.255 [15],

ea	represents Exposure function API in TS 32.254 [14],

i	represents IP Multimedia Subsystem (IMS) in TS 32.260 [20],

l	represents Location Service (LCS) in TS 32.271 [31],

m	represents Multimedia Messaging Service (MMS) in TS 32.270 [30],

mb	represents Multimedia Broadcast/Multicast Service (MBMS) in TS 32.273 [33],

o	represents the Online Charging System (OCS) in TS 32.296 [53],

p	represents Packet Switched (PS) in TS 32.251 [11],

pr	represents Proximity-based Services (ProSe) in TS 32.277 [37],

s	represents the CAMEL SCF in TS 23.078 [200],

sm	represents Short MessageService (SMS) in TS 32.274 [34],

t	represents the PoC service (PoC) in TS 32.272 [32],

w	represents interworking Wireless LAN (discontinued in Release 12), and

mn	represents Monitoring Event in TS 32.278 [38].

In the 3GPP charging architecture, the Bx reference point connects the CGF in each network domain, service or subsystem to the Billing Domain. 
Further details of the Billing Domain, i.e. beyond terminating Bx, are outside the scope of 3GPP standardization. Refer to TS 32.240 [1] for the complete description of the 3GPP charging architecture.

Note that the OCS can also generate CDRs and transfer them to the BD across the Bo reference point as described in TS 32.296 [53].

Furthermore the GSM SCF, defined in CAMEL, can also generate CDRs and transfer them to the BD across the Bs reference point as described in TS 23.078 [200].

Next changes

### 5.1.1	CDR processing

The CGF collects CDRs from the CDF. If the CDF and the CGF are separate entities, then the standard Ga interface as specified in TS 32.295 [54] is used to transfer the CDRs from the CDF to the CGF. 
If CDF and CGF are integrated, then a proprietary, internal mechanism is used. The possibilities of separate or integrated CDF and CGF are specified per domain, subsystem and service in the respective "middle tier" charging TSs, i.e. those in the [10] through [49] reference number range, and in TS 32.296 [53] for the OCS.

In any case, CDRs are transferred, in near real-time, from the CDF to the CGF as soon as they have been closed by the CDF. Refer to TS 32.295 [54] for further details. Once received by the CGF, the CDRs may undergo semantical and/or syntactical sanity checks, however, these checks are not specified further within the present document.

If the CGF determines that a CDR is not well formatted, or otherwise incorrect, then the defective CDR parameter(s) shall be filled with an appropriate "replacement" indicator within the limits of the syntax allowed for the parameter. 
If the error renders the complete CDR unusable (i.e. the above replacement of erroneous parameters is not possible), then no further action of the CGF can be performed regarding this CDR. An example of a case where the erroneous parameter cannot be replaced is when the "CDR type" attribute of a CDR received by the CGF is corrupted. 
Details of this function are implementation specific.

CDRs that have been processed by the CGF without error, or where errors have been corrected by the CGF as described above, are considered "acceptable" by the CGF. CDRs that have non-recoverable errors are not considered "acceptable" by the CGF. The "acceptable" CDRs are immediately placed into a CDR file by the CGF. 
The CDRs that are not "acceptable" should be properly reflected in an error log and appropriate alarms should be generated, after which they may be destroyed. Furthermore, to the extent possible, the number of lost CDRs and the fact that CDRs were lost, shall be indicated in the CDR file.

It is acknowledged that the processing of a CDR between being received until it is placed on the CDR file, takes a small amount of time. Hence, where the present document mandates the "immediate" treatment of CDRs received by the CGF, it is to be interpreted such that it is not allowed to postpone the processing of any received CDRs for any reason. In technical terms, "immediate" shall be interpreted as:

-	The system shall be capable of complying with near real-time requirements as specified in clause 3.1.

-	The system should be capable of complying, as closely as possible, with real-time requirements as specified in clause 3.1.

Once a CDR has been stored on the appropriate file, the CGF may destroy any other reference to that CDR.

Next change

#### 6.1.2.5	TS number

This five bit field contains a binary value that identifies the number of the TS associated to the domain CDRs encoding according to table 6.1.2.5.1:

Table 6.1.2.5.1: "TS number" identifier

| "TS number" identifier (5 bits) | TS number |
| --- | --- |
| 0 | 32.005 |
| 1 | 32.015 |
| 2 | 32.205 |
| 3 | 32.215 |
| 4 | 32.225 |
| 5 | 32.235 |
| 6 | 32.250 |
| 7 | 32.251 |
| 9 | 32.260 |
| 10 | 32.270 |
| 11 | 32.271 |
| 12 | 32.272 |
| 13 | 32.273 |
| 14 | 32.275 |
| 15 | 32.274 |
| 16 | 32.277 |
| 17 | 32.296 |
| 18 | 32.278 |
| 19 | 32.253 |
| 20 | 32.255 |
| 21 | 32.254 |
| NOTE 1: 21-31 are for future use. NOTE 2: 8 is discontinued in Release 12. |  |

End of changes