3GPP TSG-SA5 Meeting #127	S5-196219

# Sophia Antipolis, 14th Oct 2019 – 18th Oct 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0033 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clean up for incosistence |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-PRO_NS |  |  |  |  |  | Date: |  |  | 2019-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. Currently, Slice Profile concept is described in TS 28.530, however, Service Profile concept is described in TS 28.531. 2. NSI and NSSI state handling is already defined in Annex B in TS 28.541, the NSSI state described in clause 4.1 in TS 28.531 is not align with TS 28.541 3. The term “NetworkSliceSubnetActivation service provider” and “NetworkSliceActivation service provider” are used in clause 5.1.7, 5.1.8,5.1.10 and 5.1.11, which is not align with other use cases using generic term network slice management service consumer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Move Service concept from TS 28.531 to TS 28.530. - Remove the NSSI state in TS 28.531 - Change the term “NetworkSliceSubnetActivation service provider” to “Network slice subnet management service provider ”, Change the term “NetworkSliceActivation service provider” to “Network slice management service provider ” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The Slice Prfoile and Service profile defined in different TS will lead some misunderstanding - The NSSI state descibed in TS 28.531 is redundant and inconsist with TS 28.541. - The term “NetworkSliceSubnetActivation service provider” and “NetworkSliceActivation service provider” make confusing and is not align with other use cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.5, 5.1.7,5.1.8,5.1.10,5.1.11, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

1st Change

4.1	Overview

A 5G system consists of a 5G Access Network (AN), and a 5G Core Network (5GC). Network slicing is one of the key 5G features.

The management aspects of a Network Slice Instance (NSI) are described by the four phases shown in Figure 4.3.1.1 of TS 28.530 [4].

The provisioning of network slicing includes the four phases which are preparation, commissioning, operation and decommissioning:

-	In the preparation phase the NSI does not exist. The preparation phase includes network slice design, on-boarding, evaluation of the network slice requirements, preparing the network environment and other necessary preparations required to be done before the creation of an NSI.

-	During the NSI lifecycle stage which include commissioning phase, operation phase and decommissioning phase, the NSI provisioning operations include:

-	Create an NSI;

-	Activate an NSI;

-	De-active an NSI;

-	Modify an NSI;

-	Terminate an NSI.

The operations of the provisioning of an NSI occurs during different phases of a NSI:

a)	During the commissioning phase:

-	Create an NSI.

During NSI creation all resources to the NSI have been created and configured to satisfy the network slice requirements. NSI creation may trigger NSSI(s) creation or using existing NSSI(s) and setting up the corresponding associations.

b)	During the operation phase:

-	Activate an NSI;

-	Modify an NSI;

-	De-active an NSI.

NSI activation includes any actions that make the NSI active to provide communication services. NSI activation may trigger NSSI activation.

NSI modification in operation phase could map to several workflows, e.g. changes of NSI capacity, changes of NSI topology, NSI reconfiguration. NSI modification can be triggered by receiving new network slice related requirements, new communication service requirements, or the result of NSI supervision automatically. NSI modification may trigger NSSI modification.

The NSI deactivation operation may be needed before NSI modification operation and the NSI activation operation may be needed after the NSI modification operation. NSI deactivation includes any actions that make the NSI inactive and not providing any communication services. NSI deactivation trigger NSSI deactivation to deactivate constituent NSSI(s) which is not used by other NSI(s). Operator may decide to keep the NSI without termination after deactivation and reactivate it when receives new communication service request.

c)	During the decommissioning phase:

-	Terminate an NSI.

NSI termination step includes any action that make the NSI does not exist anymore and release resources that are not used by other NSI(s). NSI termination may trigger NSSI termination to terminate constituent NSSI(s) which is not used by other NSI(s).

Similarly, provisioning for network slice subnet instance includes the following operations:

-	Create an NSSI;

-	Activate an NSSI and associate it with certain NSI to be used by the NSI;

-	Disassociate the NSSI with certain NSI and de-active the NSSI if it’s not associated with any NSI;

-	Modify an NSSI;

-	Terminate an NSSI.

2nd Change

4.5	Void

3rd Change

5.1.7	Network slice instance activation

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To activate an existing network slice instance which is in inactive state |  |
| Actors and Roles | Network slice management service consumer. For example, CSMF or CSP providing NSaaS plays the role of Network slice management service consumer. |  |
| Telecom resources | Network slice instance Network slice management service provider. For example, NSMF plays the role of Network slice management service provider. |  |
| Assumptions | N/A |  |
| Pre-conditions | An NSI has already been created and it is inactive. |  |
| Begins when | The Network slice management service provider decides to activate an NSI based on the received network slice related request from its authorized consumer. |  |
| Step 1 (M) | Network slice management service provider checks whether NSSIs associated with the NSI are all in active state, if there is an inactive NSSI, Network slice management service provider requests Network slice management service provider to activate the corresponding NSSI. | Network slice subnet instance activation use case |
| Step 2 (M) | Network slice management service provider receives response from Network slice management service provider indicating that the NSSI is active. |  |
| Step 3 (M) | Network slice management service provider sets the state of the NSI as active and sends response to the requesting consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | An NSI has been activated. |  |
| Traceability | REQ-PRO_NSI–FUN-4 |  |

5.1.8	Network slice instance deactivation

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To deactivate an existing network slice instance which is in active state. |  |
| Actors and Roles | Network slice management service consumer. For example, CSMF or CSP providing NSaaS plays the role of Network slice management service consumer. |  |
| Telecom resources | Network slice instance Network slice management service provider. For example, NSMF plays the role of Network slice management service provider. |  |
| Assumptions | N/A |  |
| Pre-conditions | NSI has already been created and it is active. |  |
| Begins when | The Network slice management service provider decides to deactivate an NSI based on the received network slice related request from its authorized consumer. |  |
| Step 1 (M) | The Network slice management service provider stops the NSI serving its subscribers |  |
| Step 2 (M) | Network slice management service provider checks whether NSSIs associated with the NSI are all in inactive state. If there is an active NSSI, Network slice management service provider requests Network slice management service provider to deactivate the corresponding NSSI. Network slice management service provider receives the request and decides if the NSSI will be disassociated and deactivated. | Network slice subnet instance deactivation use case |
| Step 3 (M) | The Network slice management service provider receives response from N Network slice management service provider that the NSSI deactivation request has been processed. |  |
| Step 4 (M) | Network slice management service provider sets the NSI state as inactive and sends response to its authorized consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | An NSI has been deactivated. |  |
| Traceability | REQ-PRO_NSI–FUN-5 |  |

4th Change

5.1.10	Network slice subnet instance activation

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To activate an existing network slice subnet instance which is in inactive state. |  |
| Actors and Roles | Network slice subnet management service consumer. For example, NSMF or NSSMF plays the role of network slice subnet management service consumer. |  |
| Telecom resources | Network slice subnet instance Network slice subnet management service provider. For example, NSSMF plays the role of Network slice subnet management service provider. |  |
| Assumptions | N/A |  |
| Pre-conditions | An NSSI has already been created and it is inactive. |  |
| Begins when | The Network slice subnet management service provider decides to activate an NSSI based on the received network slice subnet related request from its authorized consumer. |  |
| Step 1 (M) | The Network slice subnet management service provider identifies inactive constituents (e.g. NSSI, NF) of the NSSI and decides to activate those constituents. |  |
| Step 2 (M) | If the constituent of NSSI is managed directly by the Network slice subnet management service provider, Network slice subnet management service provider activates the NSSI constituent directly. |  |
| Step 3 (M) | If an NSSI constituent is managed by other Network slice subnet management service provider, Network slice subnet management service provider requests other Network slice subnet management service provider to activate the constituent NSSI. |  |
| Step 4 (M) | If an NSSI constituent is an NF managed by NF related management service provider, the Network slice subnet management service provider request NF related management service provider to activate the NF (e.g., activate the NF in sleep mode, turn on the ports). |  |
| Step 5 (M) | Network slice subnet management service provider receives response indicating that NSSI constituents are all activated. |  |
| Step 6 (M) | Network slice subnet management service provider sets the state of the network slice subnet instance as active and sends response to its authorized consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | An NSSI has been activated. |  |
| Traceability | REQ-PRO_NSSI–FUN-9 |  |

5.1.11	Network slice subnet instance deactivation

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To deactivate an existing network slice subnet instance which is in active state. |  |
| Actors and Roles | Network slice subnet management service consumer. For example, NSMF or NSSMF plays the role of Network slice subnet management service consumer. |  |
| Telecom resources | Network slice subnet instance Network slice subnet management service provider. For example, NSSMF plays the role of network slice subnet service provider. |  |
| Assumptions | N/A |  |
| Pre-conditions | An NSSI has already been created and is in active state. |  |
| Begins when | The Network slice subnet management service provider decides to deactivate an NSSI based on the received network slice subnet related request from its authorized customer. |  |
| Step 1 (M) | The Network slice subnet management service provider identifies the NSSI constituents that need to be deactivated. |  |
| Step 2 (M) | If the constituent of NSSI is managed directly by the Network slice subnet management service provider, Network slice subnet management service provider deactivates the NSSI constituent directly. |  |
| Step 3 (M) | If an NSSI constituent is managed by other Network slice subnet management service provider, the Network slice subnet management service provider request other Network slice subnet management service provider to deactivate the constituent NSSI. |  |
| Step 4 (M) | If an NSSI constituent is managed by NF related management service provider, Network slice subnet management service provider requests NF related management service provider to deactivate the NF. |  |
| Step 5 (M) | Network slice subnet management service provider receives response indicating that corresponding NSSI constituents are deactivated or not deactivated (e.g., shared constituents cannot be deactivated). |  |
| Step 6 (M) | Network slice subnet management service provider sets the state of the network slice subnet instance as inactive and send response to its authorized consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | A network slice subnet instance has been deactivated. |  |
| Traceability | REQ-PRO_NSSI–FUN-10 |  |

End of Change