3GPP TSG-SA5 Meeting #127	S5-196394

# Sophia-Antipolis, France, 14th Oct 2019 - 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.550 | CR | 0036 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add usecase and requirements for NSI KPI job control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecommunications, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA-OAM_KPIRPT |  |  |  |  |  | Date: |  |  | 2019-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the objective of the new work item KPI reporting, it mentioned that “enhance (with maximum reuse) the performance assurance MnS to support KPI collection and reporting”. In the current specfication TS 28.550, it is lack of the use cases and requirements for KPI job control. So it is proposed to add new KPI job control related use cases and requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add usecase and requirements for NSI KPI job control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no requirements and corresponding use cases for NSI KPI job control. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3.X (new), 5.1.3.X.1 (new), 5.1.3.X.2 (new), 5.1.3.X.3 (new), 5.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.1.3	NSI PM services

5.1.3.X	NSI KPI job control service

5.1.3.X.1	Creation of KPI job for NSI(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to create a KPI job for collecting the KPIs of NSI(s). |  |
| Actors and Roles | An authorized consumer of NSI KPI job control service. |  |
| Telecom resources | NSI(s); NSI KPI job control service producer; The set of NSSI measurement job control service producer, NSSI performance data file reporting service producer and/or NSSI performance data streaming service producer; and/or The set of NF measurement job control service producer, NF performance data file reporting service producer and/or NF performance data streaming service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | - The NSI(s) have been deployed. - The NSI KPI job control service producer is in operation. |  |
| Begins when | The authorized consumer needs to create KPI job for collecting the KPIs of NSI(s). |  |
| Step 1 (M) | The authorized consumer requests the NSI KPI job control service producer to create a NSI KPI job to calculate and collect the KPIs of NSI(s). The request needs to indicate that the KPIs needs to be reported by performance data file or by performance data streaming. |  |
| Step 2 (M) | The NSI KPI job control service producer identifies the performance measurement types used for the NSI KPI(s) and checks whether the corresponding performance measurement types can be collected by the existing measurement job(s) for the NSI(s) and/or the corresponding constituent NSSI(s) and/or NF(s). If new measurement job(s) are required, the NSI KPI job control service producer consumes the NSI measurement job control service and/or the NSSI measurement job control service and/or the NF measurement job control service to create the new measurement job(s) for the related NSI(s) and/or NSSI(s) and/or NF(s) respectively (according to the use case "Creation of measurement job for NSI(s)" as described in clause 5.1.3.1.1 or the use case "Creation of measurement job for NSSI(s)" as described in clause 5.1.2.1.1 or the use case "Creation of measurement job for NF(s)" as described in clause 5.1.1.1.1). | Creation of measurement job for NSI(s); and/or Creation of measurement job for NSSI(s); and/or Creation of measurement job for NF(s) |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The KPI job for NSI has been created, and the NSI KPI job control service producer consumes the NSI performance data file reporting service and/or NSI performance data streaming service, the NSSI performance data file reporting service and/or NSSI performance data streaming service, the NF performance data file reporting service and/or NF performance data streaming service to get the performance data of the NSI(s) and/or constituent NSSI(s) and/or NF(s), and generates the KPIs for the NSI KPI job. | NSI performance data file reporting; and/or NSI performance data streaming NSSI performance data file reporting; and/or NSSI performance data streaming NF performance data file reporting; and/or NF performance data streaming |
| Traceability | REQ- KJCS _NSI-FUN-1, REQ- KJCS _NSI-FUN-2, REQ- KJCS _NSI-FUN-3, REQ- KJCS _NSI-FUN-4 and REQ- KJCS _NSI-FUN-7. |  |

5.1.3.X.2	Termination of KPI job for NSI(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request the NSI KPI job control service producer to terminate a NSI KPI job. |  |
| Actors and Roles | An authorized consumer of NSI KPI job control service. |  |
| Telecom resources | NSI(s); NSI KPI job control service producer; NSSI measurement job control service producer; NF measurement job control service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The NSI KPI job has been created. |  |
| Begins when | The authorized consumer does not need the NSI KPI job. |  |
| Step 1 (M) | The authorized consumer requests the NSI KPI job control service producer to terminate a NSI KPI job that is calculating or collecting the KPIs of NSI(s). |  |
| Step 2 (M) | The NSI KPI job control service producer terminates the NSI KPI job |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The NSI KPI job is terminated. |  |
| Traceability | REQ- KJCS _NSI-FUN-5 |  |

5.1.2.X.3	Query of KPI jobs for NSI(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to query the ongoing NSI KPI jobs (i.e. the NSI KPI jobs that have been created by the subject consumer and not terminated). |  |
| Actors and Roles | An authorized consumer of NSI KPI job control service. |  |
| Telecom resources | NSI(s) NSI KPI job control service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The NSI KPI job control service producer is in operation. |  |
| Begins when | The authorized consumer needs to query the ongoing NSI KPI jobs. |  |
| Step 1 (M) | The authorized consumer queries the information about the ongoing NSI KPI jobs from the NSI KPI job control service producer. |  |
| Step 2 (M) | The NSI KPI job control service producer provides the information about the ongoing NSI KPI jobs to the consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The information about the ongoing NSI KPI jobs are available to the consumer. |  |
| Traceability | REQ- KJCS _NSI-FUN-6 |  |

Next change

5.2	Requirements

5.2.X	Requirements for NF KPI job control service

REQ-KJCS_ NSI -FUN-1	The management service producer responsible for NSI KPI job control shall have the capability allowing its authorized consumer to request creation of a KPI job to collect the KPIs of NSI (s).

REQ- KJCS _ NSI -FUN-2	The management service producer responsible for NSI KPI job control shall have the capability allowing its authorized consumer to indicate the reporting method (i.e. by performance data file or by performance data streaming) for the KPIs when requesting to create a KPI job for NSI (s).

REQ- KJCS _ NSI -FUN-3	The management service producer responsible for NSI KPI job control shall have the capability to fulfill the consumer's request to create a KPI job for NSI (s).

REQ- KJCS _ NSI -FUN-4	The management service producer responsible for NSI KPI job control shall have the capability to generate the KPIs of NSI (s).

REQ- KJCS _ NSI -FUN-5	The management service producer responsible for NSSI KPI job control shall have the capability to fulfill the request from its authorized consumer to terminate a NSI KPI job.

REQ- KJCS _ NSI -FUN-6	The management service producer responsible for NSI KPI job control shall have the capability allowing its authorized consumer to query the information about the ongoing NSI KPI jobs.

REQ- KJCS _ NSI -FUN-7	The management service producer responsible for NSI KPI job control may reject a NSI KPI job creation request.

End of changes