3GPP TSG-<SA5> Meeting # 127	S5-196399

# Sophia Antipolis, France,  14 - 18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.805 | CR | 0003 | rev | <Rev#> | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | √ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add description of communication service catelogue |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GSAT_MO |  |  |  |  |  | Date: |  |  | 2019-09-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Statues of current network capability should affect the list of service catelogue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description of communication service catelogue that it will keep updating according to the current network capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.2.1	Communication service instances realized in a single network slice instance

5.2.1.1	Description

A CSP offers a portfolio of communication services to the market (CSC). CSC selects a suitable product and orders communication service by inputting some service requirements e.g. SLA requirements information.

The communication service catalogue provided by the CSP should be updated according to the existing capability of network. Communication service which netwok cannot provide or the existing usage has reached the maximum capability will not be listed in the service catalogue.

The CSP receives a request from a customer to realize a communication service instance:

- 	The CSP checks the communication service catalogue to find communication services with the communication service description which matches the request. The CSP performs feasibility check, i.e., checking the attainable service quality from both resource and service aspects.

- 	The CSP translates the service SLS into parameterized E2E network slice template information. In order to achieve this purpose, interactions between the CSP and NOP or some manually network design and planning process are needed. The CSP provides the service profile to the NOP for network resources allocation. The NOP checks the network slice inventory to find a network slice instances which matches the request.

- 	The network slice instance to be used will be checked whether there is sufficient capacity available to support the new communication service instance. If there is enough capacity available, the communication service instance will be assigned to the network slice instance. The mapping between the communication service instance and the network slice instance will be maintained by both the CSP and NOP.

The communication service instance has been realized.

5.2.1.2	Potential requirements

REQ-CSM_LCM-CON-01 The 3GPP management system shall provide a management service, which offers the capability to receive and process a request to realize a communication service instance.

REQ-CSM_LCM-CON-02 The 3GPP management system shall provide management service, which offers the capability to ensure that the list of realized communication service instances are up to date.

REQ-CSM_LCM-CON-03 The 3GPP management system shall provide a management service, which offers the capability to find a communication service description based on a request.

REQ-CSM_LCM-CON-04 The 3GPP management system shall provide a management service, which offers the capability to assign a communication service instance to an existing network slice instance.

REQ-CSM_LCM-CON-05 The 3GPP management system shall provide a management service, which offers the capability to keep the communication service catalogue up to date according to the existing network capability.