3GPP TSG-SA5 Meeting #127	S5-196695

# Sophia-Antipolis, France, 14th Oct 2019 - 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0123 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of QoS Based Charging description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-DCH, TEI16 |  |  |  |  |  | Date: |  |  | 2019-10-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The charging methods supported by QBC isn’t described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding description of how the charging methods can be used together with QBC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of what charging method may be used is left for interpretation and may lead to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-196373: Initial CR |  |  |  |  |  |  |  |  |

First change

#### 5.2.1.6	QoS flow Based Charging

QoS flow Based Charging allows the SMF to collect charging information related to data volumes per PDU session, categorized per QoS Flow.

The user can be identified by SUPI.

For a given PDU session, QBC shall be performed by the SMF within the same charging session used for Flow Based Charging. For the case where QBC is performed from SMF in VPLMN, Flow Based Charging is not applicable and there is no possibility to have quota management for the PDU Session.

The SMF categorizes the volume within PDU session by QoS Flow identified by QoS Flow Identifier (QFI).

The amount of data counted for the QoS Flow shall be the user plane payload at the UPF.

Table 5.2.1.6.1 summarizes the set of default trigger conditions and their category which shall be supported by the SMF in QBC. For "immediate report" category, the table also provides the corresponding Charging Data Request [Initial, Update, Termination] message sent from SMF towards the CHF.

Table 5.2.1.6.1: Default Chargeable events in SMF for QBC

| Chargeable event | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Start of PDU session | PDU session | Immediate | Not Applicable | Not Applicable | Charging Data Request [Initial] |
| Start of a QoS Flow | QoS Flow | Deferred | Not Applicable | Not Applicable | Charging Data Request [Update] |
| Change of Charging conditions |  |  |  |  |  |
| QoS change | QoS Flow | Deferred | Yes | Yes |  |
| User Location change | PDU session | Deferred | Yes | Yes |  |
| Serving Node change | PDU session | Deferred | Yes | Yes |  |
| Change of UE presence in Presence Reporting Area(s) | PDU session | Deferred | Yes | Yes |  |
| Change of 3GPP PS Data off Status | PDU session | Deferred | Yes | Yes |  |
| Tariff time change | PDU session | Deferred | No | No |  |
| UE time zone change | PDU session | Immediate | Yes | Yes |  |
| PLMN change | PDU session | Immediate | Yes | Yes |  |
| RAT type change | PDU session | Immediate | Yes | Yes |  |
| Session-AMBR change | PDU session | Immediate | Yes | Yes |  |
| Addition of UPF | PDU session | Immediate | Yes | Yes |  |
| Removal of UPF | PDU session | Immediate | Yes | Yes |  |
| Handover cancel | PDU session | Immediate | Yes | Yes |  |
| Handover start | PDU session | Immediate | Yes | Yes |  |
| Handover complete | PDU session | Immediate | Yes | Yes |  |
| Limit per PDU session |  |  |  |  |  |
| Expiry of data time limit per PDU session | PDU session | Immediate | Yes | Yes |  |
| Expiry of data volume limit per PDU session | PDU session | Immediate | Yes | Yes |  |
| Expiry of data event limit per PDU session | PDU session | Immediate | Yes | Yes |  |
| Expiry of limit of number of charging condition changes | PDU session | Immediate | Yes | Yes |  |
| Limit per QoS Flow |  |  |  |  |  |
| Expiry of data time limit per QoS Flow | QoS Flow | Deferred | Yes | Yes |  |
| Expiry of data volume limit per QoS Flow | QoS Flow | Deferred | Yes | Yes |  |
| Others |  |  |  |  |  |
| End of QoS Flow | QoS Flow | Deferred | Yes | Yes |  |
| Management intervention | PDU session | Immediate | No | No |  |
| End of PDU session | PDU session | Immediate | No | No | Charging Data Request [Termination] |
| Abort request is received from the CHF | PDU session | Immediate | No | No |  |

The default "Limit" trigger conditions, are trigger thresholds configured in the Charging Characteristics applied to the PDU session for QBC. It shall be possible for the CHF to override these default triggers when providing Charging Data Response [Initial], either to disable the triggers, or to enable triggers new thresholds value.

For QBC the following details of chargeable events and corresponding actions in the SMF are defined in Table 5.2.1.6.2:

Table 5.2.1.6.2: Chargeable events and their related actions in SMF for QBC

| Chargeable event | Conditions | SMF action |
| --- | --- | --- |
| Start of PDU session |  | Charging Data Request [Initial] |
| Start of a QoS Flow | Start of the QoS Flow associated with the default QoS rule | Charging Data Request [Update]. |
|  | Start of a QoS Flow | Start new counts with time stamps. |
| End of a QoS Flow |  | Close the counts with time stamps for the QoS flows |
| End of PDU session |  | Charging Data Request [Termination] Close the counts with time stamps |
| Change of charging condition in the SMF (e.g. QoS change, Session-AMBR change, user location change, Radio access type change, PLMN change, Serving Node change, UE Time Zone change, change of UE presence in Presence Reporting Area(s), change of 3GPP PS Data Off status, handover cancel) | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps for all active QoS flows. |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update] |
| Handover start | If the corresponding trigger is enabled | Start new counts with time stamps. |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update] |
| Handover complete | If the corresponding trigger is enabled | Close the counts with time stamps for all active QoS flows. |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update] |
| Addition of UPF |  | Charging Data Request [Update]. |
| Removal of UPF |  | Close the counts with time stamps for the removed UPF |
| Expiry of time limit per QoS Flow | If the corresponding trigger is enabled | Close the counts with time stamps. |
|  | If the category is set to "immediate reporting" | Charging Data Request [Update] |
|  | If the QoS Flow is still active | Start new counts with time stamps |
| Expiry of data volume limit per QoS Flow | If the corresponding trigger is enabled | Close the counts with time stamps |
|  | If the category is set to "immediate reporting" | Charging Data Request [Update] |
|  | If the QoS Flow is still active | Start new counts with time stamps |
| Expiry of time limit per PDU session | If the corresponding trigger is enabled | Close the counts with time stamps for all QoS flows. |
|  | If the category is set to "immediate reporting" | Charging Data Request [Update] |
|  | If the PDU session is still active | Start new counts with time stamps for all active QoS flows |
| Expiry of data volume limit per PDU session | If the corresponding trigger is enabled | Close the counts with time stamps for all QoS flows. |
|  | If the category is set to "immediate reporting" | Charging Data Request [Update] |
|  | If the PDU session is still active | Start new counts with time stamps for all active QoS flows |
| Expiry of a limit of number of charging condition changes per PDU session | If the corresponding trigger is enabled | Close the counts with time stamps for all QoS flows. |
|  | If the category is set to "immediate reporting" | Charging Data Request [Update] |
|  | If the PDU session is still active | Start new counts with time stamps for all active QoS flows |
| Management intervention |  | Charging Data Request [Update] Close the counts with time stamps for all QoS Flows |
|  | If the PDU session is still active | Start new counts with time stamps |
| Abort |  | Charging Data Request [Termination] Close the counts with time stamps |

The CDR generation mechanism processed by the CHF upon receiving Charging Data Request [Initial, Update, Termination] issued by the SMF for these chargeable events in QBC, is specified in clause 5.2.3.

End of changes