3GPP TSG-SA5 Meeting #127	S5-196824

# Sophia Antipolis, 14th Oct 2019 – 18th Oct 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0061 | rev | 2 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct fault supervision management service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE |  |  |  |  |  | Date: |  |  | 2019-09-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Stage 2 information model definition of fault supervision management service is not align with stage 3 solution set. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the stage2 related description of fault supervision management service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will be published with incorrect IS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.1.1.6, 11.2.1.1.x(new), 11.2.1.2.4, 11.2.1.2.5, 11.2.1.2.x(new), 11.2.1.2.y(new), 11.2.1.2.z(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

Start of 1st Change

11.2.1.1.6	notifyAlarmListRebuilt

11.2.1.1.6.1	Definition

This interface supports notifying the alarm list rebuilding information if part or all of AlarmList has been rebuilt.

11.2.1.1.6.2	Input Parameters

| Parameter Name | Qualifier | Legal type | Comment |
| --- | --- | --- | --- |
| objectClass | M | -- | It identifies the object class that changed state. |
| objectInstance | M | -- | It identifies the object instance that changed state. |
| notificationId | M | -- | It identifies the notification that carries the AlarmInformation. |
| eventTime | M | -- | It identifies the last time when the event occurred. |
| systemDN | C | -- | It identifies the DN of service providers. |
| notificationType | M | "notifyAlarmListRebuilt". |  |
| reason | M | "System-NE communication error", "System restarts", "indeterminate". Other values can be added. | It carries the reason why the system has rebuilt the AlarmList. This may carry different reasons than that carried by the immediate previous notifyPotentialFaultyAlarmList. |
| alarmListAlignmentRequirement | O | "alignmentRequired", "alignmentNotRequired". | It carries an enumeration of "alignmentRequired" and "alignmentNotRequired". |

11.2.1.1.6.3	Triggering event

11.2.1.1.6.3.1	From-state

alarmListRebuilt_0 OR alarmListRebuilt_1.

| Assertion Name | Definition |
| --- | --- |
| alarmListRebuilt_0 | Provider has cold-started, initialized, re-initialized or rebooted and it has initiated procedure to rebuild its AlarmList. |
| alarmListRebuilt_1 | Provider loses confidence in part or whole of its AlarmList. Provider has initiated procedure to repair its AlarmList. |

11.2.1.1.6.3.2	To-state

alarmListRebuilt_2.

| Assertion Name | Definition |
| --- | --- |
| alarmListRebuilt_2 | Provider rebuilds the whole or part of AlarmList. |

End of 1st Change

Start of 2nd Change

11.2.1.1.x	setComment

11.2.1.1.x.1	Definition

The authorized consumer invokes this operation to record a comment in one or more AlarmInformation instances in AlarmList.

11.2.1.1.x.2	Input Parameters

| Name | Qualifier | Information Type | Comment |
| --- | --- | --- | --- |
| alarmInformation ReferenceList | M | List of AlarmInformation.alarmId | It carries one or more identifiers identifying AlarmInformation instances in the AlarmList. |
| commentUserId | M | The Comment.commentUserId where Comment is involved in relation-AlarmInformation-Comment with an AlarmInformation. |  |
| commentSystemId | O | The Comment.commentSystemId where Comment is involved in relation-AlarmInformation-Comment with an AlarmInformation. |  |
| commentText | M | The comment.commentText where Comment is involved in relation-AlarmInformation-Comment with an AlarmInformation. |  |

11.2.1.1.x.3	Output Parameters

| Name | Qualifier |  | Matching Information | Comment |
| --- | --- | --- | --- | --- |
| badAlarm Information ReferenceList |  | M | List of pair of AlarmInformation.alarmId and the failure reason. | If allUpdated is true, it contains no information. If someUpdated is true, then it contains identifications of AlarmInformation that are not present in AlarmList or that they are present, but AlarmInformation.comments has not changed, in contrast to authorized consumer's request. |
| status |  | M | ENUM( Operation succeeded, Operation failed, Operation partially failed) | If allUpdated is true, then status = OperationSucceeded. If someUpdated is true, then status = OperationPartiallyFailed. If exception operationFailed is raised, then status = OperationFailed. |

End of 2nd Change

Start of 3rd Change

11.2.1.2.4	notifyClearedAlarm

11.2.1.2.4.1	Definition

This interface notifies the alarm clearing information if it satisfies filter constraint in AlarmInformation. The notification shall satisfy all filter constraint and notify in the notifyNewAlarmNotification.

11.2.1.2.4.2	Input Parameters

| Parameter Name | Qualifier | Legal type | Comment |
| --- | --- | --- | --- |
| objectClass | M | -- | It identifies the object class whose perceived severity level is cleared. |
| objectInstance | M | -- | It identifies the object instance whose perceived severity level is cleared. |
| notificationId | M | -- | It identifies the notification that carries the AlarmInformation. |
| eventTime | M | -- | It identifies the last time when the event occurred. |
| systemDN | C | -- | It identifies the DN of service providers. |
| notificationType | M | "notifyClearedAlarm" |  |
| probableCause | M | -- |  |
| perceivedSeverity | M | -- | Its value shall indicate Cleared. |
| alarmType | M | -- |  |
| correlated Notifications | O | The set of CorrelatedNotification related to this AlarmInformation. | It contains references to other AlarmInformation instances whose perceivedSeverity levels are Cleared as well. In this way, perceivedSeverity level of multiple AlarmInformation instances can be Cleared by one notification. |
| clearUserId | O | -- | It carries the identity of the user who invokes the clearAlarms operation. |
| clearSystemId | O | -- | It carries the identity of the authorized consumer. |
| alarmId | M | -- | It identifies one AlarmInformation in the AlarmList. |

11.2.1.2.4.3	Triggering event

11.2.1.2.4.3.1	From-state

alarmMatchedAndCleared OR clearedByProvider.

| Assertion Name | Definition |
| --- | --- |
| alarmMatchedAndCleared | The matching-criteria-attributes of the newly generated network alarm have values that are identical (matched) with ones in one AlarmInformation in AlarmList and the perceivedSeverity of the matched AlarmInformation is not Cleared AND The perceivedSeverity of the newly generated network alarm is cleared. |
| clearedByProvider | Reception of a valid clearAlarms operation that identifies the subject AlarmInformation instances. This triggering event shall occur regardless of the perceivedSeverity state of the identified AlarmInformation instances. |

11.2.1.2.4.3.2	To-state

alarmInformationCleared_1 OR alarmInformationCleared_2.

| Assertion Name | Definition |
| --- | --- |
| alarmInformationCleared_1 | Case if From-state is alarmMatchedAndCleared: The following attributes of the subject AlarmInformation are updated: notificationId, perceivedSeverity (updated to Cleared), alarmClearedTime. |
| alarmInformationCleared_2 | Case if From-state is clearedByProvider: The following attributes of the subject AlarmInformation are updated: notificationId, perceivedSeverity (updated to Cleared), alarmClearedTime, alarmClearedUserId, alarmClearedSystemId. |

11.2.1.2.5	notifyAckStateChanged

11.2.1.2.5.1	Definition

This interface indicates two types of AckStateChanged alarm, which are acknowledged alarm and unacknowledged alarm respectively. The capability of acknowledging alarms is vendor defined.

The relative state change information of these two types of alarm has been referred to stateChangeDefinition as specific attributes of AlarmInformation. The notification shall satisfy all filter constraint and notify in the notifyNewAlarmNotification.

11.2.1.2.5.2	Input Parameters

These parameters are filters for the interfaces.

| Parameter Name | Qualifier | Legal type | Comment |
| --- | --- | --- | --- |
| objectClass | M | -- | It identifies the object class that changed state. |
| objectInstance | M | -- | It identifies the object instance that changed state. |
| notificationId | M | -- | It identifies the notification that carries the AlarmInformation. |
| eventTime | M | -- | It identifies the last time when the event occurred. |
| systemDN | C | -- | It identifies the DN of service providers. |
| notificationType | M | "notifyAckStateChanged". |  |
| probableCause | M | -- | It qualifies alarm and provides further information than eventType. |
| perceivedSeverity | M | -- | It indicates the relative level of urgency for operator attention. |
| alarmType | M | -- | The notification structure defined by this table is applicable if this parameter indicates "Communications Alarm", "Processing Error Alarm", "Environmental Alarm". "Quality Of Service Alarm" or "Equipment Alarm". |
| alarmId | M | -- | It identifies one AlarmInformation in the AlarmList. |
| ackState | M | -- | It identifies the Acknowledgement State of the alarm. |
| ackUserId | M | -- | It identifies the last user who has changed the Acknowledgement State. |
| ackSystemId | O | -- | It identifies the system (the authorized consumer) that last changed the ackState of an alarm, i.e. acknowledged or unacknowledged the alarm. |

11.2.1.2.5.3	Triggering event

11.2.1.2.5.3.1	From-state

ackedByConsumer OR ackedByProvider AND alarmInformationExists.

| Assertion Name | Definition |
| --- | --- |
| ackedByConsumer | Reception of an acknowledgeAlarms operation and a subsequent operation success return. |
| ackedByProvider | Reception of a local (non-standard) acknowlegeAlarms equivalent operation and a subsequent operation success return. |
| alarmInformationExists | The AlarmInformation exists in AlarmList. |

11.2.1.2.5.3.2	To-state

alarmAckStateHasChanged.

| Assertion Name | Definition |
| --- | --- |
| alarmAckStateHasChanged | The AlarmInformation.ackState of the AlarmInformation identified by from-state assertion alarmInformationExists have been updated. Specifically, the following attributes of the subject AlarmInformation are updated: -- notificationId, ackTime, ackUserId, ackState, ackSystemId. |

11.2.1.2.x	notifyComments

11.2.1.2.x.1	Definition

The subscribed authorized consumer instances are notified regarding the addition of a Comment instance to an AlarmInformation instance in the AlarmList. The AlarmInformation carried in the notification shall satisfy the current filter constraint of the subscription.

The notification shall contain all parameters that are filterable and are present in the original (related) notifyNewAlarm notification.

Service provider shall support this notification if it supports the operation setComment.

11.2.1.2.x.2	Input Parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| objectClass | M,Y | MonitoredEntity.objectClass | The MonitoredEntity is identified by the relation-AlarmedObject-AlarmInformation of the new AlarmInformation. |
| objectInstance | M,Y | MonitoredEntity.objectInstance | The MonitoredEntity is identified by the relation-AlarmedObject-AlarmInformation of the new AlarmInformation. |
| notificationId | M,N | -- |  |
| eventTime | M,Y | Comment.commentTime |  |
| systemDN | C,Y | -- |  |
| notificationType | M,Y | "notifyComments" |  |
| alarmType | M,Y | AlarmInformation.eventType |  |
| probableCause | M,Y | AlarmInformation.probableCause |  |
| perceived Severity | M,Y | AlarmInformation.perceivedSeverity |  |
| comments | M,N | The set of Comment instances involved in a relationship with this AlarmInformation. |  |
| alarmId | M,N | AlarmInformation.alarmId |  |

11.2.1.2.x.3	Trigger event

11.2.1.2.x.3.1	From-state

commentedByServiceprovider OR commentedByServiceprovider AND alarmInformationExists.

| Assertion Name | Definition |
| --- | --- |
| commentedByServiceprovider | Reception of a setComment operation and a subsequent operation success return. |
| commentedByServiceprovider | Reception of a local (non-standard) setComment equivalent operation and a subsequent operation success return. |
| alarmInformationExists | The AlarmInformation is in AlarmList. |

11.2.1.2.x.3.2	To-state

commentInserted.

| Assertion Name | Definition |
| --- | --- |
| commentInserted | One Comment has been created and it is involved in a relationship with the AlarmInformation identified by from-state assertion alarmInformationExists. The following attributes of the newly created Comment instance shall be populated: commentTime, commentText, commentUserId and commentSystemId. |

11.2.1.2.y	notifyPotentialFaultyAlarmList

11.2.1.2.y.1	Definition

The service provider or its related AlarmIRP maintains an AlarmList. They can lose confidence in the integrity of its AlarmList. Under this condition, service provider or its related AlarmIRP or the related AlarmList shall invoke notifyPotentialFaultyAlarmList. They then can begin to rebuild the faulty AlarmList, if found necessary. After the successful rebuilt or the discovery that rebuilt is not necessary, they shall invoke notifyAlarmListRebuilt notification.

This notification can identify a set of AlarmInformation that is potentially faulty or unreliable. This identification is done in the following way. If the MOI of an AlarmInformation is the same or is a subordinate to the MOI carried in the notification, then the AlarmInformation may be faulty or unreliable.

This notification can identify all the AlarmInformation instances of the AlarmList that are potentially faulty or unreliable. In this case, the notification shall carry a MOI identifying the service provider.

The authorized consumer behaviour, on reception of this notifyPotentialFaultyAlarmList notification, is not specified. The authorized consumer behaviour is considered not essential for the specification of the interface itself. However, the following are recommended actions the uthorized consumer should take, in case it receives this notification.

1)	The uthorized consumer should not perform any task requiring the integrity of the AlarmInformation identified as faulty or unreliable by the subject notification.

2)	The uthorized consumer should not invoke operations that require integrity of the AlarmList such as getAlarmList., acknolwedgeAlarms operations.

11.2.1.2.y.2	Input Parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| objectClass | M,Y | It identifies - the class of the instance identified by systemDN or - the class of MonitoredEntity. | If it identifies the class of the instance identified in systemDN, then all AlarmInformation instances in the AlarmList may not be reliable. If it identifies the class of MonitoredEntity, then some or all AlarmInformation instances in the AlarmList may not be reliable. See next parameter for the identification of the set of AlarmInformation that may not be reliable. |
| objectInstance | M,Y | It identifies - the instance identified by systemDN or - an instance of MonitoredEntity. | If it identifies the instance identified by systemDN, then all AlarmInformation instances in the AlarmList may not be reliable. If it identifies an instance of MonitoredENtity, then AlarmInformation of this instance and AlarmInformation of its subordinate instances may not be reliable. |
| notificationId | M,N | -- |  |
| eventTime | M,Y | -- |  |
| systemDN | C,Y | -- |  |
| notificationType | M,Y | "notifyPotentialFaultyAlarmList". |  |
| reason | M,N | "serviceprovider-NE communication error", " serviceprovider restarts", "indeterminate". Other values can be added. | It carries the reason why the service provider has to rebuild its AlarmList. |

11.2.1.2.y.3	Trigger event

11.2.1.2.y.3.1	From-state

faultyAlarmListDetected.

| Assertion Name | Definition |
| --- | --- |
| faultyAlarmListDetected | Service provider detects faults in part or whole of its AlarmList. |

11.2.1.2.y.3.2	To-state

faultyAlarmList

| Assertion Name | Definition |
| --- | --- |
| faultyAlarmList | Service provider initiates the AlarmList rebuild process. |

11.2.1.2.z	notifyChangedAlarmGeneral

11.2.1.2.z.1	Definition

The subscribed authorized consumer instances are notified regarding changes in backedUpStatus, backUpObject, trendIndication, thresholdInfo, stateChangeDefinition, monitoredAttributes, proposedRepairActions, additionalText, additionalInformation, serviceUser, serviceProvider or securityAlarmDetector of an AlarmInformation instance in the AlarmList. This notification is triggered by value change in one or some of these attributes. The AlarmInformation carried in the notification shall satisfy the current filter constraint of the subscription.

The notification shall contain all parameters holding a value.

11.2.1.2.z.2	Input Parameters

There are two tables for Input Parameters. If alarmType parameter indicates "Communications Alarm", "Processing Error Alarm", "Environmental Alarm". "Quality Of Service Alarm" or "Equipment Alarm", the first table (see clause 11.2.1.2.z.2) shall be applicable. If alarmType parameter indicates "Integrity Violation", "Operational Violation", "Physical Violation", "Security Service or Mechanism Violation" or "Time Domain Violation", the second table (see clause 11.2.1.2.z.3) shall be applicable.

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| objectClass | M,Y | MonitoredEntity.objectClass | It shall carry the MonitoredEntity class name. The MonitoredEntity is identified by the relation-AlarmedObject-AlarmInformation of the new AlarmInformation. |
| objectInstance | M,Y | MonitoredEntity.objectInstance | It shall carry the DN of the MonitoredEntity. The MonitoredEntity is identified by the relation-AlarmedObject-AlarmInformation of the new AlarmInformation. |
| notificationId | M,N | -- |  |
| eventTime | M,Y | AlarmInformation.alarmChangedTime |  |
| systemDN | C,Y | -- |  |
| notificationType | M,Y | "notifyChangedAlarmGeneral". |  |
| probableCause | M,Y | AlarmInformation.probableCause |  |
| perceivedSeverity | M,Y | AlarmInformation.perceivedSeverity |  |
| rootCauseIndicator | O,N | It indicates that this AlarmInformation is the root cause of the events captured by the notifications whose identifiers are in the related CorrelatedNotification instances. | “Yes”, “No” |
| alarmType | M,Y | AlarmInformation.eventType | The notification structure defined by this table is applicable if this parameter indicates "Communications Alarm", "Processing Error Alarm", "Environmental Alarm". "Quality Of Service Alarm" or "Equipment Alarm". |
| specificProblem | O,N | AlarmInformation.specificProblem |  |
| correlatedNotifications | O,N | The set of CorrelatedNotification related to this AlarmInformation. |  |
| backedUpStatus | O,N | AlarmInformation.backedUpStatus |  |
| backUpObject | O,N | MonitoredEntity.objectInstance | It carries the DN of the back up object. The object is identified by relation-BackUpObject-AlarmInformation of the new AlarmInformation. |
| trendIndication | O,N | AlarmInformation.trendIndication |  |
| thresholdInfo | O,N | AlarmInformation.thresholdInfo |  |
| stateChangeDefinition | O,N | AlarmInformation.stateChange |  |
| monitoredAttributes | O,N | AlarmInformation.monitoredAttributes |  |
| proposedRepairActions | O,N | AlarmInformaton.proposedRepairActions |  |
| additionalText | O,N | AlarmInformation.additionalText |  |
| additionalInformation | O,N | AlarmInformation.additionalInformation |  |
| alarmId | M,N | AlarmInformation.alarmId |  |
| changedAlarmAttributes | M,N | LIST OF SEQUENCE <AttributeName, OldAttributeValue> | The changed alarm attributes (name/value pairs) (with old values). |

11.2.1.2.z.3	Input Parameters for notification related to security alarm

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| objectClass | M,Y | MonitoredEntity.objectClass | It shall carry the MonitoredEntity class name. The MonitoredEntity is identified by the relation-AlarmedObject-AlarmInformation of the new AlarmInformation. |
| objectInstance | M,Y | MonitoredEntity.objectInstance | It shall carry the DN of the MonitoredEntity. The MonitoredEntity is identified by the relation-AlarmedObject-AlarmInformation of the new AlarmInformation. |
| notificationId | M,N | -- |  |
| eventTime | M,Y | AlarmInformation.alarmChangedTime |  |
| systemDN | C,Y | -- |  |
| notificationType | M,Y | "notifyChangedAlarmGeneral". |  |
| probableCause | M,Y | AlarmInformation.probableCause |  |
| perceivedSeverity | M,Y | AlarmInformation.perceivedSeverity |  |
| rootCauseIndicator | O,N | It indicates that this AlarmInformation is the root cause of the events captured by the notifications whose identifiers are in the related CorrelatedNotification instances. | “Yes”, “No” |
| alarmType | M,Y | AlarmInformation.eventType | The notification structure of this table is applicable if this parameter indicates "Integrity Violation", "Operational Violation", "Physical Violation", "Security Service or Mechanism Violation", "Time Domain Violation". |
| correlatedNotifications | O,N | The set of CorrelatedNotification related to this AlarmInformation. |  |
| additionalText | O,N | AlarmInformation.additionalText |  |
| additionalInformation | O,N | AlarmInformation.additionalInformation |  |
| serviceUser | M,N | AlarmInformation.serviceUser | This may contain no information if the identify of the service-user (requesting the service) is not known. |
| serviceProvider | M,N | AlarmInformation.serviceProvider | This shall always identify the service-provider receiving a service request, from serviceUser, that provokes the security alarm. |
| securityAlarmDetector | M,N | AlarmInformation.securityAlarmDetector | This may contain no information if the detector of the security alarm is the serviceProvider. |
| alarmId | M,N | AlarmInformation.alarmId |  |
| changedAlarmAttributes | M,N | LIST OF SEQUENCE <AttributeName, OldAttributeValue> | The changed alarm attributes (name/value pairs) (with old values). |

11.2.1.2.z.4	Trigger event

11.2.1.2.z.4.1	From-state

alarmMatched AND alarmNotCleared AND alarmChanged.

| Assertion Name | Definition |
| --- | --- |
| alarmMatched | The matching-criteria-attributes of the newly generated network alarm has values that are identical (matches) with ones in one AlarmInformation in AlarmList. |
| alarmChanged | The backedUpStatus, backUpObject, trendIndication, thresholdInfo, stateChangeDefinition, monitoredAttributes, proposedRepairActions, additionalText, additionalInformation, serviceUser, serviceProvider or securityAlarmDetector of the newly generated network alarm and of the matched AlarmInformation are different. |

11.2.1.2.y.4.2	To-state

informationUpdate.

| Assertion Name | Definition |
| --- | --- |
| informationUpdate | The AlarmInformation identified in alarmMatched in from-state has been updated according to the following rules: backedUpStatus, backUpObject, trendIndication, thresholdInfo, stateChangeDefinition, monitoredAttributes, proposedRepairActions, additionalText, additionalInformation, serviceUser, serviceProvider or securityAlarmDetector is updated; notificationId is updated; alarmChangedTime is updated; ackTime, ackUserId and ackSystemId are updated to contain no information; ackState is updated to "unacknowledged"; |

End of 3rd Change