3GPP TSG-SA5 Meeting #127	S5-196397

# Sophia-Antipolis, France, 14th Oct 2019 - 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.550 | CR | 0037 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add usecase and requirements for network/sub-network KPI job control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecommunications, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA-OAM_KPIRPT |  |  |  |  |  | Date: |  |  | 2019-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the objective of the new work item KPI reporting, it mentioned that “enhance (with maximum reuse) the performance assurance MnS to support KPI collection and reporting”. In the current specfication TS 28.550, it is lack of the use cases and requirements for KPI job control. So it is proposed to add new KPI job control related use cases and requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add usecase and requirements for network/sub-network KPI job control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no requirements and corresponding use cases for network/sub-network KPI job control. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4.X (new), 5.1.4.X.1 (new), 5.1.4.X.2 (new), 5.1.4.X.3 (new), 5.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.1.4	Network/Sub-network PM services

5.1.4.X	Network/Sub-network KPI job control service

5.1.4.X.1	Creation of KPI job for network(s)/sub-network(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to create a KPI job for collecting the network/sub-network KPIs that are not specific to network slicing. |  |
| Actors and Roles | An authorized consumer of network KPI job control service. |  |
| Telecom resources | Network(s)/sub-network(s); Network KPI job control service producer; NF measurement job control service producer; NF performance data file reporting service producer and/or NF performance data streaming service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | - The network(s)/sub-network(s) have been deployed; - The network KPI job control service producer is in operation. |  |
| Begins when | The authorized consumer needs to create a network KPI job for collecting the network/sub-network KPIs that are not specific to network slicing. |  |
| Step 1 (M) | The authorized consumer requests the network KPI job control service producer to create KPI job to calculate and collect the network KPIs that are not specific to network slicing. The request needs to indicate that the KPIs needs to be reported by performance data file or by performance data streaming. |  |
| Step 2 (M) | The network KPI job control service producer identifies the performance measurement types used for the network/sub-network KPI(s) and checks whether the corresponding performance measurement types can be collected by the existing measurement job(s) for the constituent 3GPP NF(s). If new measurement job(s) for the constituent NF(s) are required, the network KPI job control service producer requests the NF measurement job control service producer to create the new measurement job(s) for the constituent NF(s) (according to the use case "Creation of measurement job for NF(s)" as described in clause 5.1.1.1.1). | Creation of measurement job for NF(s) |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The KPI job for network(s)/sub-network(s) has been created, and the network KPI job control service producer consumes the NF performance data file reporting service and/or NF performance data streaming service to get the performance data of the constituent NF(s), and generates the KPIs for the network KPI job. | NF performance data file reporting; NF performance data streaming |
| Traceability | REQ- KJCS _NW-FUN-1, REQ- KJCS _NW-FUN-2, REQ- KJCS _NW-FUN-3, REQ- KJCS _NW-FUN-4 and REQ- KJCS _NW-FUN-7 |  |

5.1.4.X.2	Termination of KPI job for network(s)/sub-network(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request the network KPI job control service producer to terminate a network KPI job. |  |
| Actors and Roles | An authorized consumer of network KPI job control service. |  |
| Telecom resources | Network(s)/sub-network(s); Network KPI job control service producer; NF measurement job control service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The network KPI job has been created. |  |
| Begins when | The authorized consumer does not need the network KPI job. |  |
| Step 1 (M) | The authorized consumer requests the network KPI job control service producer to terminate a network KPI job that is calculating or collecting the KPIs of network(s)/sub-network(s). |  |
| Step 2 (M) | The network KPI job control service producer terminates the network KPI job. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The network KPI job is terminated. |  |
| Traceability | REQ- KJCS _NW-FUN-5 |  |

5.1.4.X.3	Query of KPI jobs for network(s)/sub-network(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to query the ongoing network KPI jobs (i.e. the network KPI jobs that have been created by the subject consumer and not terminated). |  |
| Actors and Roles | An authorized consumer of network KPI job control service. |  |
| Telecom resources | Network(s)/sub-network(s) Network KPI job control service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The network KPI job control service producer is in operation. |  |
| Begins when | The authorized consumer needs to query the ongoing network KPI jobs. |  |
| Step 1 (M) | The authorized consumer queries the information about the ongoing network KPI jobs from the network KPI job control service producer. |  |
| Step 2 (M) | The network KPI job control service producer provides the information about the ongoing network KPI jobs to the consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The information about the ongoing network KPI jobs are available to the consumer. |  |
| Traceability | REQ- KJCS _NW-FUN-6 |  |

Next change

5.2	Requirements

5.2.X	Requirements for network/sub-network KPI job control service

REQ-KJCS_NW-FUN-1	The management service producer responsible for network/sub-network KPI job control shall have the capability allowing its authorized consumer to request creation of a KPI job to collect the network/sub-network KPIs that are not specific to network slicing.

REQ- KJCS _NW-FUN-2	The management service producer responsible for network/sub-network KPI job control shall have the capability allowing its authorized consumer to indicate the reporting method (i.e. by performance data file or by performance data streaming) for the KPIs that are not specific to network slicing when requesting to create a KPI job for network(s)/sub-network(s).

REQ- KJCS _NW-FUN-3	The management service producer responsible for network/sub-network KPI job control shall have the capability to generate the network/sub-network KPIs that are not specific to network slicing.

REQ- KJCS _NW-FUN-4	The management service producer responsible for network/sub-network KPI job control shall have the capability to fulfill the consumer's request to create a KPI job for network(s)/sub-network(s).

REQ- KJCS _NW-FUN-5	The management service producer responsible for network/sub-network KPI job control shall have the capability to fulfill the request from its authorized consumer to terminate a network/sub-network KPI job.

REQ- KJCS _NW-FUN-6	The management service producer responsible for network/sub-network KPI job control shall have the capability to fulfill the request from its authorized consumer to query the information about the ongoing network KPI jobs.

REQ- KJCS _NW-FUN-7	The management service producer responsible for network/sub-network KPI job control may reject a network/sub-network KPI job creation request.

End of changes