3GPP TSG-SA5 Meeting #127	S5-196051

# Sophia-Antipolis (France), 14 - 18 October 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0057 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce Stage 2 and 3 for the heartbeat management service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, Deutsche Telekom, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ONAP3GPP |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Stage 1 for the heartbeat management service has been specified. Stage 2 and Stage 3 are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Stage 2 and Stage 3 for the heartbeat management service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 and Stage 3 for the heartbeat management service would not exist though Stage 1 exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.X (new), 12.X (new), A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

# 10	Management services – Stage 2

## 10.X	Heartbeat management service

### 10.X.1	Operations and notifications

#### 10.X.1.1	getHeartbeatPeriod operation

#### 10.x.1.1.1	Definition

Consumer invokes this operation to obtain the current heartbeat period.

#### 10.x.1.1.2	Input parameters

There is no input parameter.

#### 10.x.1.1.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| heartbeatPeriod | M | heartbeatPeriod |  |
| status | M | ENUM (Operation succeeded, Operation failed) |  |

#### 10.x.1.1.4	Exceptions and constraints

None.

#### 10.X.1.2	setHeartbeatPeriod operation

#### 10.x.1.2.1	Definition

The consumer invokes this operation to set the heartbeatPeriod.

As a consequence and indicative of successful completion of the operation the producer shall emit the notifyHeartbeat specified in clause 10.X.1.4  immediately and continue to emit based on the newly specified heartbeatPeriod, to all established notification subscriptions of all subscribed consumers.

If the heartbeatPeriod specified is the same as the current value in producer, the operation shall fail.

#### 10.x.1.2.2	Input parameters

| Parameter Name | Qualifier | Information Type | Comment |
| --- | --- | --- | --- |
| heartbeatPeriod | M | heartbeatPeriod |  |

#### 10.x.1.2.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (Operation succeeded, Operation failed) |  |

#### 10.x.1.2.4	Exceptions and constraints

None.

#### 10.X.1.3	triggerHeartbeat operation

#### 10.x.1.3.1	Definition

The consumer invokes this operation to solicit a notifyHeartbeat notification. After the successful completion of the operation, the producer shall emit the notifyHeartbeat notification as specified in clause 10.X.1.4 immediately. One notification shall be emitted as follows:

a)	one notification to the soliciting consumer; or

b)	one notification to each of the subscribed consumers.

If the operation fails the notification shall not be emitted.

One of the two options above shall be chosen depending on system performance considerations.

Before invoking this operation, the soliciting consumer should make sure it has subscribed the notifyHeartbeat notification.

#### 10.x.1.3.2	Input parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| managerIdentifier | M | String | The identifier for the triggering consumer. |

#### 10.x.1.3.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (Operation succeeded, Operation failed) |  |

#### 10.x.1.3.4	Exceptions and constraints

None.

#### 10.X.1.4	Notification notifyHeartbeat

##### 10.X.1.4.1	Definition

This notification notifies the subscribed consumer(s) that the HBMnSProvider heartbeat period has expired or that a Heartbeat MnS Consumer requested the emission of an immediate heartbeat notification.

##### 10.X.1.4.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | It shall carry the ManagedEntity class name. | It specifies the class name of the IOC. A network event has occurred in an instance of this class. |
| objectInstance | M | It shall carry the DN of the ManagedEntitiy. | It specifies a new instance of the above IOC in which the network event related to by carrying the Distinguished Name (DN) for the instance. |
| eventTime | M | It indicates the time at which the notification is emitted. | The semantics of Generalised Time specified by ITU-T [17] shall be used here. |
| notificationId | M | This is an identifier for the notification, | The identifier of the notification shall be chosen to be unique across all notifications of a particular managed object instance throughout the time that correlation is significant. It uniquely identifies the notification from other notifications generated by the subject MOI. |
| systemDN | C | It shall carry the DN of management service providers. | - |
| heartbeatPeriod | M | ManagedEntity.heartbeatPeriod | The value of the ManagedEntity.heartbeatPeriod |
| triggerFlag | M | ENUM {HBMnSConsumer, HBMnSProvider} | If this notification is triggered by HBMnS consumer by invoking triggerHeartbeat operation (cf. clause 10.X.1.3), the value of this parameter shall be "HBMnSConsumer". Otherwise, it shall be "HBMnSProvider". |
| hBMnSConsumerRef | M | String | If the value of triggerFlag is "HBMnSConsumer", this field has the same value as the input parameter "hBMnSConsumerRef" of the triggerHeartbeat operation (cf. clause 10.X.1.3). |

##### 10.X.1.4.3	Triggering event

###### 10.X.1.4.3.1	From-state

stateBeforeHeartbeatNotification1 OR stateBeforeHeartbeatNotification2.

| Assertion Name | Definition |
| --- | --- |
| stateBeforeHeartbeatNotification1 | The internal countdown timer of the ManagedEntity instance emitting the notifyHeartbeat notification has reached the value ‘0’ (zero). |
| stateBeforeHeartbeatNotification2 | The value of the attribute actionTriggerHeartbeat of the ManagedEntity instance emitting the notifyHeartbeat notification is True. |

###### 10.X.1.4.3.2	To-state

stateAfterOHeartbeatNotification1 OR stateAfterOHeartbeatNotification2.

| Assertion Name | Definition |
| --- | --- |
| stateAfterHeartbeatNotification1 | If From-state is stateBeforeHeartbeatNotification1 then: the internal countdown timer of the ManagedEntity instance is reset to the value of its heartbeatPeriod attribute. |
| stateAfterHeartbeatNotification2 | If From-state is stateBeforeHeartbeatNotification2 then: the value of the internal countdown timer of the ManagedEntity instance remains unchanged. |

Next change

# 12	Management services – Stage 3

## 12.X	Heartbeat management service

### 12.X.1	RESTful HTTP-based solution set

#### 12.X.1.1	Mapping of operations

For (non-configurable) Heartbeat MnS, all stage 2 operations are not mapped to stage 3 Heartbeat specific operations.

For Configurable Heartbeat MnS, all stage 2 operations are mapped to Generic provisioning MnS operations as bwloe:

| Heartbeat MnS operation | Provisioning MnS operation |
| --- | --- |
| getHeartbeatPeriod | getMOIAttributes |
| setHeartbeatPeriod | modifyMOIAttributes |
| triggerHeartbeat | modifyMOIAttributes |

#### 12.X.1.2	Mapping of notifications

##### 12.X.1.2.1	Introduction

The IS notifications are mapped to SS equivalents according to table 12.X.1.2.1-1.

Table 12.X.1.2.1-1: Mapping of IS notifications to SS equivalents

| IS notifications | HTTP Method | Resource URI | SQ |
| --- | --- | --- | --- |
| notifyHeartbeat | POST | /notificationSink | M |

##### 12.X.1.2.2	Notification "notifyHeartbeat"

The IS notification parameters are mapped to SS equivalents according to table 12.X.1.2.2-1.

Table 12.X.1.2.2-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| objectClass objectInstance | request body | href | uri-Type | M |
| eventTime | request body | eventTime | dateTime-Type | M |
| notificationId | request body | notificationId | notificationId-Type | M |
| systemDN | request body | systemDN | systemDN-Type | O |
| notificationType | request body | notificationType | notificationTyp-Type | M |
| heartbeatPeriod | request body | heartbeatPeriod | heartbeatPeriod-Type | M |
| triggerFlag | request body | triggerFlag | triggerFlag-Type | M |
| hBMnSConsumerRef | request body | href | uri-Type | M |

### 12.X.2	RESTful HTTP-based solution set for integration with ONAP VES API

#### 12.X.2.1	Mapping of operations

N/A.

#### 12.X.2.2	Mapping of notifications

##### 12.X.2.2.1	Introduction

###### 12.X.2.2.1.1	General

The 3GPP IS notifications are mapped to SS equivalents according to table 12.X.2.2.1.1-1.

Table 12.2.2.2.1.1-1: Mapping of 3GPP IS notifications to SS equivalents

| 3GPP IS notifications | HTTP Method | Resource URI | SQ |
| --- | --- | --- | --- |
| notifyHeartbeat | POST | /eventListener | M |

###### 12.X.2.2.1.2	Notification parameter mapping principles

3GPP heartbeat notification parameters are mapped either to:

- ONAP VES API commonEventHeader fields [21] – see Table 12.X.2.2.1.2-1, or to

- ONAP VES API heartbeat3gpp fields – see mapping tables in the following clauses.

Table 12.X.2.2.1.2-1: Mapping of 3GPP IS notification parameters to ONAP VES common event header fields

| 3GPP IS notification parameter name | SQ | Corresponding ONAP VES common event header field name | SQ in ONAP VES common event header | Remark |
| --- | --- | --- | --- | --- |
| no corresponding IS parameter | N/A | sourceId | O |  |
| notificationId | M | eventId | M |  |
| no corresponding IS parameter | N/A | eventName | M | See [23]. |
| eventTime | M | lastEpochMicrosec | M | Type conversion needed [21]. |
| no corresponding IS parameter | N/A | reportingEntityName | M | See [24]. |
| eventTime | M | startEpochMicrosec | M | Same value as lastEpochMicrosec. Type conversion needed [21]. |
| no corresponding IS parameter | N/A | domain | M | “fault3GPP” |
| no corresponding IS parameter | N/A | priority | M | Field not used by this solution set. Any type-compatible value [21] to be provided by the FS data report MnS producer. Actual value out of scope of this document. |
| no corresponding IS parameter | N/A | sequence | M | Field not used by this solution set. Value ‘0’ [21] to be provided by the FS data report MnS producer. Actual value out of scope of this document. |
| no corresponding IS parameter | N/A | sourceName | M | Field not used by this solution set. Any type-compatible value [21] to be provided by the FS data report MnS producer. Actual value out of scope of this document. |
| no corresponding IS parameter | N/A | version | M | Version of the event header [21]. |
| no corresponding IS parameter | N/A | vesEventListenerVersion | M | Version of the VES Event Listener API [21]. |

The ONAP VES API commonEventHeader datatype consists of fields common to all events, as specified in [21].

NOTE 1: Table 12.X.2.2.1.2-1 has the following columns:

- 3GPP IS notification parameter name: name of the input parameter of the notification as specified in clauses of clause 10.2.1 of the present document;

- SQ: Support Qualifier for the notification input parameter, as specified in clauses of clause 10.2.1 of the present document;

- Corresponding VES common event header field name: name of the field of the VES common event header [21];

- SQ in ONAP VES common event header: specified in [21], where ‘M’ corresponds to ‘Required = Yes’ in [21], and ‘O’ corresponds to ‘Required = No’ in [21];

NOTE 2: In Table 12.X.2.2.1.2-1, some VES common event header fields have no corresponding IS parameter. In case those VES common event header field Support Qualifier is ‘M’ (as specified in [21]), the producer of the notification shall provide a value to these parameters. See column ‘Remark’ in Table 12.X.2.2.1.2-1.

NOTE 3: ONAP VES common event header fields which are both optional [21] and have no corresponding IS parameter are not listed in Table 12.X.2.2.1.2-1.

NOTE 4: The mapping tables in following clauses of clause 12.2.2 have the following columns:

- 3GPP IS notification parameter name: name of the input parameter of the notification as specified in clauses of clause 10.2.1 of the present document;

- SQ: Support Qualifier for the notification input parameter, as specified in clauses of clause 10.2.1 of the present document;

- SS parameter name (Corresponding VES ‘heartbeat3GPP’ domain field): name of the field in ‘heartbeat3gppFields’;

- SQ: Support Qualifier in ‘heartbeat3gppFields’. When the IS notification parameter Support Qualifier is ‘M’, this Support Qualifier is set to ‘M’. When the IS notification parameter Support Qualifier is ‘C’, this Support Qualifier is set to ‘O’. When the IS notification parameter Support Qualifier is ‘O’, this Support Qualifier is set to ‘O’.

##### 12.X.2.2.2	Notification notifyHeartbeat

The 3GPP IS notification parameters are mapped to SS equivalents according to table 12.X.2.2.1.2-1 and to table 12.X.2.2.2.1-1.

Table 12.X.2.2.2.1-1: Mapping of IS notification parameters to SS equivalents in ‘heartbeat3gppFields’

| 3GPP IS notification parameter name | SQ | SS parameter name in ‘heartbeat3gppFields’ – clause A.X.2 | SQ |
| --- | --- | --- | --- |
| objectClass objectInstance | M | DN | M |
| systemDN | O | systemDN | O |
| notificationType | M | notificationType (value = ‘notifyHeartbeat’) | M |
| heartbeatPeriod | M | heartbeatPeriod | M |
| triggerFlag | M | triggerFlag | M |
| hBMnSConsumerRef | M | hBMnSConsumerRef | M |

Next change

Annex A (normative): 
OpenAPI specification

## A.0	Introduction

This clause describes the capabilities of the service in the structure of the OpenAPI Specification Version 3.0.1 [A9]. The OpenAPI document is represented in the JSON format option.

## A.X	Heartbeat management service

### A.X.1	Heartbeat management service

Editor’s note: this is FFS.

### A.X.2	Heartbeat management service for integration with ONAP VES

{

"heartbeat3gppFields": {

"description": "fields specific to 3GPP heartbeat notifications for integration with ONAP VES",

"type": "object",

"properties": {

"heartbeat3gppFieldsVersion": {

"description": "version of the heartbeat3gppFields block",

"type": "string",

"enum": ["16.0"]

},

"heartbeatNotifications": {

"oneOf": [

{"$ref": "#/definitions/notifyHeartbeat-NotifType"}

]

}

},

"definitions": {

"notifyHeartbeat-NotifType": {

"description": "fields specific to 3GPP notification NotifyHeartbeat",

"type": "object",

"properties": {

"dN": {

"$ref": "#/definitions/dN-Type"

},

"systemDN": {

"$ref": "#/components/schemas/systemDN-Type"

},

"notificationType": {

"$ref": "#/definitions/notificationType-Type"

},

"heartbeatPeriod": {

"$ref": "#/definitions/heartbeatPeriod-Type"

},

"triggerFlag": {

"$ref": "#/definitions/triggerFlag-Type"

},

"hBMnSConsumerRef": {

"$ref": "#/definitions/hBMnSConsumerRef-Type"

},

"additionalProperties": false,

"required": [ "dN", "notificationType", "heartbeatPeriod", "triggerFlag", "hBMnSConsumerRef" ]

},

"dN-Type": {

"type": "string"

},

"systemDN-Type": {

"type": "string"

},

"notificationType-Type": {

"type": "string",

"enum": [

"notifyHeartbeat"

]

},

"triggerFlag-Type": {

"type": "string",

"enum": [

"HBMnSConsumr",

"HBMnSProvider"

]

},

"hBMnSConsumerRef-Type": {

"type": "string"

}

}

}

}

}

End