3GPP TSG-SA5 Meeting #127 	S5-196578

# Sophia Antipolis, France 14-18 October 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0034 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update allocation and deallocation specification due to new NRM fragment for resource allocation and deallocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNRM |  |  |  |  |  | Date: |  |  | 2019-10-11 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Dedicated operations are currently used for network slice instance (NSI) allocation and network slice subnet instance (NSSI) allocation and deallocation. This is not aligned with the Service Based Management Architecture (SBMA) and REST principles. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new NRM fragment supporting allocation and deallocation is added to TS 28.541, and this TS is updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SBMA and REST principles are not followed leading to inconsistent specifications and increased development effort. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 6.4, 6.5 (voided), 7.2, 7.3, 7.4, 7.5, 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS/TR 28.541 CR 0194 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.525: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Requirements".

[3]	ETSI GS NFV-IFA 013 (V2.4.1) (2018-02): "Network Function Virtualisation (NFV); Release 2; Management and Orchestration; Os-Ma-nfvo reference point - Interface and Information Model Specification".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP TS 22.261 "Service requirements for next generation new services and markets".

[6]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[7]	3GPP TS 28.526: "Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[8]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[9]	GSMA NG.116 Generic Network Slice Template v1.0 (2019-05-23) - https://www.gsma.com/newsroom/wp-content/uploads//NG.116-v1.0-4.pdf.

[10]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects;System Architecture for the 5G System;Stage 2"

[11]	3GPP TS 38.300: "Technical Specification Group Radio Access Network;NR; NR and NG-RAN Overall Description;Stage 2"

[12]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions"

Next modification

## 6.1	Management services for network slice provisioning

The management services for network slice provisioning are listed in table 6.1-1.

Table 6.1-1: Management services for network slice provisioning

| MnS Name | MnS Component Type A (operations and notifications) | MnS Component Type B (information model) | Note |
| --- | --- | --- | --- |
| Provisioning for NSI | Operations defined in clause 5 of TS 28.532 [8]: - createMOI operation - deleteMOI operation - getMOIAttributes operation - modifyMOIAttributes operation | NSI information model defined in clause 6.3 of TS 28.541 [6] | This management service enables its consumer to request allocating, deallocating, or modifying an NSI. The typical scenario is "Network Slices as NOP internals" model where this MnS is consumed by operators. |
| Provisioning data report for NSI | Operations defined in clause 5 of TS 28.532 [8]: - subscribe operation - unSubscribe operation Notifications defined in clause 5 of TS 28.532 [8]: - notifyMOICreation notification - notifyMOIDeletion notification - notifyMOIAttributeValueChanges notification | NSI information model defined in clause 6.3 of TS 28.541 [6] | This management service enables its consumer to obtain notifications about NSI Information model data. The typical scenario is "Network Slices as NOP internals" model where this MnS is consumed by operators |
| Provisioning exposure for NSI | Operations defined in clause 5 of TS 28.532 [8]: - createMOI operation - deleteMOI operation - getMOIAttributes operation - modifyMOIAttributes operation | NSI information model defined in clause 6.3 of TS 28.541 [6] | This management service enables its consumer to request allocating, deallocating or modifying an NSI. The typical scenario is NSaaS model where this MnS is consumed by vertical industry. |
| Provisioning data report exposure for NSI | Operations defined in clause 5 of TS 28.532 [8]: - subscribe operation - unSubscribe operation Notifications defined in clause 5 of TS 28.532 [8]: - notifyMOICreation notification - notifyMOIDeletion notification - notifyMOIAttributeValueChanges notification | NSI information model defined in clause 6.3 of TS 28.541 [6] | This management service enables its consumer to obtain notifications about NSI Information model data. The typical scenario is NSaaS model where this MnS is consumed by vertical industry. |

Next modification

## 6.4	Management services for network and sub-network provisioning

The management services for network and sub-networks provisioning are listed in table 6.4-1.

Table 6.4-1: Management services for network and sub-network provisioning

| MnS name | MnS Component of type A (Operations and notifications) | MnS Component of type B (information model) |
| --- | --- | --- |
| Provisioning for network and sub-networks | Operations defined in clause 5 of TS 28.532 [8]: - createMOI operation - modifyMOIAttributes operation - getMOIAttributes operation - deleteMOI operation | IOC(s) of sub-network, as defined in TS 28.541 [6] |
| Provisioning data report for sub-networks | Operations defined in clause 5 of TS 28.532 [8]: - subscribe operation - unSubscribe operation Notifications defined in clause 5 of TS 28.532 [8]: - notifyMOICreation notification - notifyMOIDeletion notification - notifyMOIAttributeValueChanges notification | IOC(s) of sub-network, as defined in TS 28.541 [6] |

Next modification

## 6.5.	Void

Next modification

## 7.2	Procedure of Network Slice Instance Allocation

The Figure 7.2-1 illustrates the procedure of creating a new NSI or using an existing NSI to satisfy the required network slice related requirements.

Figure 7.2-1: Network Slice Instance Allocation Request procedure

1)	The Network Slice Management Service Consumer (NSMS_Consumer) creates a "AllocationRequest" MOI in the Network Slice Management Service Provider (NSMS_Provider) using a generic create operation.

In the create request the NSMS_Provider specifiers the attributes of the "AllocationRequest" MOI as follows:

- The value of the "reqResourceProfile" attribute is the service profile of the slice to be allocated.

- The value of the "theAllocatedResource" attribute has null semantics.

- The value of the "allocationStatus" attribute is "NOT_STARTED".

The NSMS_Provider may create a "Subscription" MOI attached to the "AllocationRequest" MOI to subscribe to attribute value change notificatios to be notified about the allocation result. Otherwise the NSMS_Provider needs to poll the "theAllocatedResource" attribute of the "AllocationRequest" MOI regularly using a generic read operation.

2)	The NSMS_Provider sets the value of the "allocationStatus" attribute to "IN_PROGRESS". Based on the network slice related requirements, the NSMS_Provider decides whether to use an existing NSI or create a new NSI. If the network slice related requirements allow the requested NSI to be shared and if an existing suitable NSI can be reused, the NSMS_Provider may decide to use the existing NSI.

3a) If using an existing NSI and the existing NSI needs to be modified to satisfy the network slice related requirements, the NSMS_Provider invokes the procedure to modify the existing NSI as described in clause 7.6.

3b-1) If creating a new NSI, the NSMS_Provider derives the network slice subnet related requirements from the received network slice related requirements. Before NSMS_Provider derives the network slice subnet related requirements, NSMS_Provider may invoke corresponding network slice subnet capability information querying procedure as descried in clause 7.8.

3b-2)	The NSMS_Provider invokes the NSSI allocation procedure as described in clause 7.3.

3b-3)	The NSMS_Provider creates the MOI for NSI and configures the MOI with the DN of MOI for the NSSI, other configuration information may be configured for the created MOI.

Note:	The detailed configuration information is described in network slice NRM (see NetworkSlice IOC defined in clause 6.3.1 in TS 28.541 [6]).

4) Upon success, the NSMS_Provider

- updates the "theAllocatedResource" attribute of the "AllocationRequest" MOI with the DN of the allocated "NetworkSlice" MOI

- sets the "allocationStatus" attribute of the "AllocationRequest" MOI to "SUCCESS"

- adds a new DN to the "theAllocationRequests" attribute of the "NetworkSlice" MOI pointing to the "AllocationRequest" MOI

Attribute value change notifications are generated and forwarded to the NSMS_Consumer if the NSMS_Consumer has subscribed to receiving these notifications.

When the NSMS_Consumer has the DN of the allocated "NetworkSlice" MOI he reads the attributes of that MOI using a generic read operation.

In case of failure, the "allocationStatus" attribute is set to "FAILURE". An attribute value change notifications are generated and forwarded to the NSMS_Consumer if the NSMS_Consumer has subscribed to receiving these notification. The value of the "theAllocatedResource" attribute is not modified and continues to have null semantics.

Next modification

## 7.3	Procedure of Network Slice Subnet Instance Allocation

The Figure 7.3-1 illustrates the procedure of creating a new network slice subnet instance or using an existing network slice subnet instance to satisfy the required network slice subnet related requirements.

Figure 7.3-1: Network Slice Subnet Instance Allocation Request procedure

1)	The Network Slice Management Service Consumer (NSMS_Consumer) creates a "AllocationRequest" MOI in the Network Slice Management Service Provider (NSMS_Provider) using a generic create operation.

In the create request the NSMS_Provider specifiers the attributes of the "AllocationRequest" MOI as follows:

- The value of the "reqResourceProfile" attribute is the slice  profile of the slice subnet to be allocated.

- The value of the "theAllocatedResource" attribute has null semantics.

- The value of the "allocationStatus" attribute is "NOT_STARTED".

The NSMS_Provider may create "Subscription" MOI attached to the "AllocationRequest" MOI to subscribe to attribute value change notificatios to be notified about the allocation result. Otherwise the NSMS_Provider needs to poll the "theAllocatedResource" attribute of the "AllocationRequest" MOI regularly using a generic read operation.

2) NSSMS_P check the feasibility of network slice subnet related requirements. If the network slice subnet related requirements can be satisfied, the following step 3) are needed, else go to step 5).

3)	Based on the network slice subnet related requirements, NSSMS_P decides whether to use an existing NSSI or create a new NSSI. If the network slice subnet related requirements allow the requested NSSI to be shared and if an existing suitable NSSI can be reused, the NSSMS_P decides to use the existing NSSI.

4.1a) If using an existing NSSI and the existing NSSI needs to be modified to satisfy the network slice subnet related requirements, the NSSMS_P invokes the procedure to modify the existing NSSI as described in clause 7.7.

4.1b.1) If creating a new NSSI, the NSSMS_P creates the MOI for the NSSI to be created. NSSMS_P derives the corresponding network slice subnet constituent (i.e. NF, constituent NSS) related requirements and transport network related requirements (e.g. 3GPP endpoint information, latency requirements, bandwidth requirements, isolation requirements) from the received network slice subnet related requirements. Before NSSMS_Provider derives the constituent network slice subnet related requirements, NSMS_Provider may invoke corresponding network slice subnet capability information querying procedure as described in clause 7.8.2.

4.1b.2)	If the NSSI to be created contains virtualisation part (i.e. VNF or VL), NSSMS_P derives the NS instance instantiation information (the NS instance instantiation information is described in clause 7.3.2.2 and clause 7.3.3.2 [3]) based on network slice subnet related requirements. NSSMS_P determines VNF instance(s) that need to be deployed according to the necessary network function(s) and then derives the profile of virtual link(s) according to the connection requirements between the network functions. NSSMS_P chooses a proper NSD deployment flavour and creates data concerning the SAPs of the NS instance. NSSMS_P invokes the NS instantiation procedures to create a NS instance. NSSMS_P configures the NSS MOI with the NS instance identifier.

Note:	NS instantiation procedure is described in TS 28.526 [7].

4.1b.3) For each required NSSI constituent, the following step 4.1b.3a) and 4.1b.3b) are needed:

4.1b.3a) If the required NSSI constituent is constituent NSSI, NSSMS_P invokes NSSI Allocation Procedure.

4.1b.3b) If the required NSSI constituent is NF instance, NSSMS_P invokes NF Creation Procedure as described in clause 7.10 or NF Modification Procedure as described in clause 7.11.

4.1b.4)	NSSMS_P configures the MOI for NSSI with the DN of the MOI for NSSI constituent (i.e. NF, constituent NSSI).

4.1b.5)	For each required transport network related requirements, NSSMS_P invokes corresponding procedure of coordination with relevant TN Manager to handle the TN part as described in clause 7.9.

5)	Upon success, the NSMS_Provider

- updates the "theAllocatedResource" attribute of the "AllocationRequest" MOI with the DN of the allocated "NetworkSliceSubnet" MOI.

- sets the "allocationStatus" attribute of the "AllocationRequest" MOI to "SUCCESS"

- adds a new DN to the "theAllocationRequests" attribute of the "NetworkSliceSubnet" MOI pointing to the "AllocationRequest" MOI

Attribute value change notifications are generated and forwarded to the NSMS_Consumer if the NSMS_Consumer has subscribed to receiving these notifications.

When the NSMS_Consumer has the DN of the allocated "NetworkSliceSubnet" MOI he reads the attributes of that MOI using a generic read operation.

In case of failure, the "allocationStatus" attribute is set to "FAILURE". An attribute value change notifications is generated and forwarded to the NSMS_Consumer if the NSMS_Consumer has subscribed to receiving these notification. The value of the "theAllocatedResource" attribute is not modified and continues to have null semantics.

Next modification

## 7.4	Procedure of Network Slice Instance Deallocation

Figure 7.4-1 depicts the procedure of deallocating a network slice instance by the network slice management service provider to satisfy the NSI deallocation request received from an authorized consumer.

Figure 7.4-1: Network slice instance deallocation procedure

1) The NSMS_C sets the "theAllocatedResource" attribute of the "allocationRequest" MOI to null semantics and removes the DN of the "AllocationRequest" MOI from the "theAllocationRequest" attribute of the "NetworkSlice" MOI. The removal of this bidirectional relationship indicates to the NSMS_P that the "NetworkSlice" MOI is deallocated.

2) The NSMS_C may decide to delete the "allocationRequest" MOI. Alternatively, also the NSMS_P may decide to delete the "allocationRequest" MOI.

3) The NSMS_P may decide to terminate the NSI, then it invokes the NSSI deallocation procedure as described in clause 7.5.

4) The NSMS_P may decide not to terminate the NSI but to modify the NSI, then it invokes the NSI modification procedure as described in clause 7.6.

Next modification

## 7.5	Procedure of network slice subnet instance deallocation

Figure 7.5-1 depicts the procedure of deallocating a network slice subnet instance by the network slice subnet management service provider to satisfy the NSSI deallocation request received from an authorized consumer.

Figure 7.5-1: Network slice subnet instance deallocation procedure

1) The network slice subnet management service provider (NSSMS_P) receives NSSI deallocation request (see DeallocateNssi operation defined in clause 6.5.4) from network slice subnet management service consumer (NSSMS_C). The NSMS_C sets the "theAllocatedResource" attribute of the "allocationRequest" MOI to null semantics and removes the DN of the "AllocationRequest" MOI from the "theAllocationRequest" attribute of the "NetworkSliceSubnet" MOI. The removal of this bidirectional relationship indicates that "NetworkSliceSubnet" MOI is deallocated.

2)  The NSMS_C may decide to delete the "allocationRequest" MOI. Alternatively, also the NSMS_P may decide to delete the "allocationRequest" MOI.

3-a) NSSMS_P may decide to terminate the NSSI, it invokes (constituent) NSSI deallocation procedure as described in clause 7.5 if the NSSI consists of constituent NSSI.

3-b) NSSMS_P invokes NF deletion procedure as described in clause 7.12 or NF modification procedure as described in clause 7.11 if the NSSI consists of NFs.

3-c) NSSMS_P invokes TN related coordination procedure with responsible manager as described in clause 7.9 if the NSSI consists of TN part.

3-d) NSSMS_P invokes NS termination procedure if the NSSI contains virtualized part.

Note:	NS termination procedure is described in TS 28.526 [7].

4) NSSMS_P may decide not to terminate the NSSI, it invokes NSSI modification procedure as described in clause 7.7.

Next modification

# 8	RESTful HTTP-based solution set of provisioning

# Provisioning is implemented with the create, read, update and delete operations defined in 28.532 [8] and the NRMs in TS.28.623 [12] and TS 28.541 [6].

Last modification